3GPP TSG-RAN4 Meeting #116	R4-2509074

# Bengaluru, IN, 25th – 29th Aug 2025

3GPP TSG RAN Meeting #109	RP-25xxxx

# Beijing, CN, 15th – 18th Sept 2025

# Source:	China Telecom, China Unicom

# Title:	Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

# Document for:	Approval

Agenda Item:	6.8.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title:   Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

Acronym: HPUE_NR_CADC_SUL_R19

Unique identifier: 1040123

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
|  | Stage 3 |
|  | Other (e.g. testing) |

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore, the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| NR_RF_FR1_enh | 890062 | RF requirements enhancement for NR frequency range 1 (FR1) | This Rel-17 WI specified generic requirements for high power UEs for intra-band CA combinations |
| NR_intra_HPUE_R17 | 900163 | High power UE for NR TDD intra-band carrier aggregation in frequency range FR1 | This Rel-17 basket WI specified high power UEs for certain NR TDD intra-band CA combinations. |
| HPUE_NR_FR1_TDD_intra_CA_R18 | 970087 | High power UE (power class 1.5 or 2) for intra-band Carrier Aggregation combinations of a single NR FR1 TDD band | This Rel-18 basket WI was a continuation of the Rel-17 WI NR_intra_HPUE_R17. The present Rel-19 WI is also covering these cases. |
| HPUE_FR1_TDD_NR_CADC_SUL_R18 | 970089 | Rel-18 High power UE (power class 1,5 and 2) for a single FR1 NR TDD band in UL of NR inter-band CA/DC combinations with/without NR SUL (supplementary uplink) with y bands downlink (y=2,3,4,5,6) and x bands uplink (x=1,2) | This Rel-18 basket WI focused on the requirements of UL TDD PC2 and PC1.5/DL CA for requested band combinations from operators. The present Rel-19 basket WI focuses on uncompleted requests from Rel-18, and new requests made in Rel-19. |
| HPUE_FR1_FDD_NR_CADC_R18 | 970090 | Rel-18 High power UE (power class 2) for a single FR1 NR FDD band in UL of NR intra-band and inter-band CA/DC combinations with y bands downlink (y=1,2,3,4,5,6) and x bands uplink (x=1) | This Rel-18 basket WI focused on the requirements of UL FDD PC2/DL CA for requested band combinations from operators. The present Rel-19 basket WI focuses on uncompleted requests from Rel-18, and new requests made in Rel-19. |
| NR_ENDC_RF_Ph4 | 1030077 | UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 | This Rel-19 WI specifies general requirements for PC1.5 for NR TDD UL intra-band CA configuration. The present Rel-19 WI will specify band combination-specific requirements. |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

Increasing the transmit power of UE has significant benefits on extending uplink coverage area and improving the experience of cell edge users.

In Release 18, several basket WIs for HPUE, including power class 2 for NR FDD band and power class 2/1.5 for NR TDD band, were created to capture the requests and work on the band-combination specific RF requirements for NR inter-band CA/DC. For NR intra-band CA, Rel-18 HPUE_NR_FR1_TDD_intra_CA_R18 WI introduces PC1.5 for single uplink carrier rather than UL intra-band CA configuration.

In release 19, there are new requests from operators for power class 2 and power class 1.5 inter-band CA/DC with/without SUL or intra-band CA combinations with both single uplink carrier and intra-band CA configuration, so to set up the new corresponding basket WI is also necessary to accommodate these requests and to handle these band-combination requirements in a same way.

Hence, this WID is proposed to set up a basket WI to work on the band-combination specific requirements for power class 2 and power class 1.5 UE for NR intra-band CA and inter-band CA/DC with/without SUL band combinations.

The rules and procedures for this basket WI include:

A) Request for additions of band combinations to this WI shall be provided using an agreed template and sent to the 3GPP_TSG_RAN_WG4_NR_BANDS email reflector before a RAN4 Tdoc submission deadline and no new band combinations are allowed to be requested after the deadline except to correct the missing fallback and add more supporting companies for the proposed band combinations.

B) When a proponent requests a new higher power band combination, the corresponding PC3 band combination configuration shall be complete or to planned to be completed at the RAN4 meeting associated with the request. If the fallback configuration is not completed at the RAN4 meeting associated with the request, the new higher power band combination shall be removed from the revised WID for the upcoming RAN Plenary.

C) A band combination configuration can only be considered as completed when the fallback configuration is completed and specified before or at the same meeting. It is the responsibility of the proponent to ensure the status of the fallback mode configuration. Rapporteurs and other companies are encouraged to check the status of the fallback configuration once the higher power band combination is declared as completed.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objectives of the core part are as follows:

Specify the band-combination specific RF requirements for all listed band combinations as defined in attached excel file of this WI. The band combinations list contains following cases.

| Rel-19 High power UE (power class 1,5 and 2) for NR CA/DC combinations with/without NR SUL |  |  |
| --- | --- | --- |
| Obj. | Band combination list | Power class cases for uplink |
| 1 | High power UE (power class 1.5 or 2) for NR Intra-band Carrier Aggregation (CA) with high power on FDD or TDD band | 1UL(FDD): PC2 on FDD band 1UL(TDD): PC1.5 or PC2 on TDD band Note: single UL carrier for FDD, Single UL carrier or intra-band UL CA for TDD |
| 2 | High power UE (power class 1.5 or 2) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with/without Supplementary uplink (SUL) with high power on TDD band(s) Note: Including PC3 FDD/TDD+ PC3 TDD case | PC2 inter-band CA/DC: 1UL(TDD): PC2 on TDD band 2UL (TDD+TDD): PC3 or PC2 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC3 or PC2 on TDD band |
|  |  | PC1.5 inter-band CA/DC: 1UL(TDD): PC 1.5 on TDD band 2UL (TDD+TDD): PC3, PC2 or PC1.5 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC1.5 on TDD band |
|  |  | PC2 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC2 on TDD band PC1.5 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC1.5 on TDD band NUL = Normal Uplink in contrast to SUL. |
| 3 | High power UE (power class 2) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with high power on FDD band(s) Note: Including PC3 FDD+ PC3 FDD | 1UL(FDD): PC2 on FDD band 2UL (FDD+FDD): PC3 on FDD band |
| 4 | High power UE (power class 1.5) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with high power on both FDD and TDD bands | 2UL (FDD+TDD): PC2 on FDD band, PC2 or PC1.5 on TDD band |

Note: Acc. to TS 38.101-1 clause 6.2A.1.3: "For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands." i.e. for 2UL: PC3+PC3 => PC2, PC3+PC2 => PC2, PC3+PC1.5 => PC1.5, PC2+PC1.5 => PC1.5, so they all are high power UE combinations.

The requirements corresponding to the above Band combination list include:

  - For Objective 1:

- For NR intra-band CA with single uplink carrier with PC2 or 1.5 on TDD band or PC2 on FDD band, introduce band combinations to the CA configuration tables and specify maximum output power.

- For NR intra-band CA with uplink intra-band CA with PC2 or 1.5 FR1 on TDD band, introduce band combinations to the CA configuration tables and specify requirements for intra-band UL CA with or without UL MIMO/TxD.

  - For Objective 2, 3 and 4:

- Introduce band combinations to the CA configuration tables and specify maximum output power, Tx power tolerance and A-MPR (Additional Maximum Power Reduction) requirements if needed

- Analyze combinations that have self-desensitization and specify applicable ∆TIB, c (=Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c) and ∆RIB, c (=Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c) and reference sensitivity exceptions including MSD test cases

- For 3Tx band combinations

  - Introduce band combinations to the CA configuration tables, and inter-band CA MOP (Maximum Output Power) tables

  - Specify corresponding MSD (Maximum Sensitivity Degradation) requirements

  - Other additional impact to the requirements due to the high power on UL, if necessary

  - Specify the release independency if necessary

- Unless otherwise stated in this WID, band combinations of this WI are introduced in a REL-independent way starting from REL-15.

- Unless otherwise stated in this WID, band combinations with 3Tx of this WI are introduced in a REL-independent way starting from REL-17.

Note 1: For 3Tx, both high power classes and PC3 for band combinations are included in this WID.

Note 2: The common requirements should be done first before working on the band specific requirements.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TR | 38.794 | High power UE (power class 1.5 or 2) for NR Intra-band Carrier Aggregation (CA) with high power on FDD or TDD band | RAN#109 | RAN#110 | Core part Rapporteur: Aijun Cao, CATT, ajcao@catt.cn |
| TR | 38.746 | High power UE (power class 1.5 or 2) for NR Inter-band Carrier Aggregation (CA)/Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s) | RAN#109 | RAN#110 | Core part Rapporteur: Lei GAO, China Telecom, gaol8@chinatelecom.cn |
| TR | 38.750 | High power UE (power class 2) for NR Inter-band Carrier Aggregation (CA)/Dual Connectivity (DC) with high power on FDD band(s) | RAN#109 | RAN#110 | Core part Rapporteur: Basaier Jialade, China Unicom, basejld@chinaunicom.cn |
| TR | 38.792 | High power UE (power class 1.5) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with high power on both FDD and TDD bands | RAN#109 | RAN#110 | Core part Rapporteur: Jinqiang Xing, OPPO, xingjinqiang@oppo.com |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN#110 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Lei GAO

Company: China Telecom

Email: gaol8@chinatelecom.cn

Basaier Jialade

Company:	China Unicom

Email: basejld@chinaunicom.cn

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN WG4

# 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

China Telecom

T-Mobile USA

Huawei

HiSilicon

AT&T

Verizon

NTT DOCOMO INC.

Ericsson

KDDI

Samsung

SoftBank

Telstra

OPPO

Nokia

China Unicom

LG Uplus

ZTE

Sanechips

TELUS

vivo

CHTTL

CATT

BT

DISH

MediaTek

SK Telecom

CMCC

Spreadtrum

Apple

Xiaomi

Qualcomm

LGE