3GPP TSG-RAN WG4 Meeting #116	R4-2509010

Bengaluru, India, 25th – 29th August 2025

3GPP TSG RAN WG1 Meeting #121	R1-2504869

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	LS on Low NR band carrier aggregation via switching

Response to:

Release:	Release-19

Work Item:	NR_LBCA_Sw

Source:	RAN1

To:	RAN2, RAN4

Cc:

Contact person:

Name:		Haitong Sun

E-mail Address:	haitong_sun@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

# Overall Description

RAN1 would like to inform RAN2 and RAN4 of the following agreements related to Low NR band carrier aggregation via switching

Agreements made in RAN1#120bis

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carriers Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Agreements made in RAN1#121

Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

# Actions

To RAN2:

Action: RAN1 respectfully asks RAN2 to take the above agreements into account.

To RAN4:

Action: RAN1 respectfully asks RAN4 to take the above agreements into account.

# Dates of Next TSG-RAN1 Meeting

RAN1 #122			25 August – 29 August 2025		Bengaluru, India

RAN1 #122bis			13 October – 17 October 2025		Prague, Czech Republic