3GPP TSG-RAN WG4 #116	R4-2509035

Bengaluru, India, Agust 25-29, 2025

Agenda item:	7.23.3.2		Adaptation of Common Signal/Channel Transmission

Source:	Qualcomm Incorporated

Title:	Further core requirements for adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES has been started. Several agreements have been made for SSD adaptation in RAN4 #115 in [1] as listed below.

In this paper we provide our views for the open issues identified in the way forward in [1].

# Discussion

Good progress was made in RAN4 #115 on several aspects of SSB adaptation [1]. In the following subsection swe address the open issues.

One open issue is SCell activation based on SSD adaptation. Chapter 5.9 in [2] states that

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

That means that as long as the SCell is deactivated, the UE does not monitor PDCCH on the SCell nor for the SCell. Therefore, DCI indication for SSB adaptation can only occur on activated SCell for the same cell and/or on other activated SCells. We also do not see any agreement in RAN1 or RAN2 to change this legacy behavior. Hence, in our opinion SCell activation based on SSB adaptation is not supported in Rel-19.

Observation 1:  Legacy behavior in RAN2 is that as long as the SCell is deactivated, the UE does not monitor PDCCH on the SCell nor for the SCell. Therefore, DCI indication for SSB adaptation can only occur on activated SCell for the same cell and/or on other activated SCells. Hence, SCell activation based on SSB adaptation is not supported in Rel-19.

Proposal 1: RAN4 should stop discussing SCell activation based on SSB adaptation as long as RAN1 or RAN2 has not agreed to support this scenario.

So far, the following agreements and proposals have been made in RAN4 for SCell activation based on SSB adaptation.

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related

Agreement:

- For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

- Proposals

  - Option 1: R18 Scell activation with L3 reporting

  - Option 2: multiple SCells activation scenario with SSB adaptation

Based on the legacy behavior in RAN2 it also means that these agreements and proposals for SCell activation based on SSB adaptation do not apply until RAN1 or RAN2 has agreed to change legacy behavior.

Observation 2: Based on the legacy behavior in RAN2 it also means that these agreements and proposals for SCell activation based on SSB adaptation do not apply until RAN1 or RAN2 has agreed to change legacy behavior.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #115. Our observations and proposals are summarized as follows:

Observation 1:  Legacy behavior in RAN2 is that as long as the SCell is deactivated, the UE does not monitor PDCCH on the SCell nor for the SCell. Therefore, DCI indication for SSB adaptation can only occur on activated SCell for the same cell and/or on other activated SCells. Hence, SCell activation based on SSB adaptation is not supported in Rel-19.

Proposal 1: RAN4 should stop discussing SCell activation based on SSB adaptation as long as RAN1 or RAN2 has not agreed to support this scenario.

Observation 2: Based on the legacy behavior in RAN2 it also means that these agreements and proposals for SCell activation based on SSB adaptation do not apply until RAN1 or RAN2 has agreed to change legacy behavior.

# References

- R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, RAN4 #115, May 2025

- TS 38.321, Medium Access Control (MAC) protocol specification, v18.6.0, June 2026