3GPP TSG-RAN WG4 Meeting #116	R4-2509109

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on Less than 5Mhz demodulation for NTN

Agenda item:	7.6.4.6

Document for:	Discussion

Introduction

In this document Nokia presents some perspectives on the ongoing Rel-19 work item on NR IoT NTN for less than 5 MHz.

Some views are presented based on the only open issue from the WF from RAN4#115 [1].

Discussion

PRACH Applicability Rule

During RAN4#115 only 1 remaining issues remains regarding PRACH applicability rule, stated from the WF as:

Options: - Option 1: Introduce applicability rules for PRACH - Proposal 1a: For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply - Option 2: No applicability rule is needed for PRACH

Nokia maintains the view that it would be suitable to define such an applicability rule in accordance with Option 1a and the proposal raised by Samsung at RAN4#115.

RAN4 should consider the introduction of an applicability rule for PRACH using the following wording: “For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply”

Conclusion

The paper discusses the topic of NR IoT NTN for less than 5 MHz.in RAN4 for performance requirements derivation, and presents some of the Nokia views on this topic for discussion during RAN4#165.

The following Observations and Proposals were made:

Proposal 1: RAN4 should consider the introduction of an applicability rule for PRACH using the following wording: “For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply”

- References

- R4-2508725, “WF for NR IoT NTN less than 5MHz demod”, Ericsson, RAN4#115, Matla, May 2025