3GPP TSG RAN WG4 Meeting #116                  	       R4-2509135

Bengaluru, IN, 25th - 29th Aug, 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	7.13.3

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for UE with 8Rx:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

Definition of MMSE-IRC receiver capability for 8Rx UE

Options: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 5: Re-use existing definition in Rel-18.

There was intensive discussion on the 8Rx receiver definition in the last meeting. We agree with the main concern on adding ‘MMSE-IRC receiver’ in the capability definition, which may prevent 8Rx UEs supporting other advanced receivers (e.g., R-ML) from testing the new Rel-19 requirements with interference.

One more information is that, the main motivation of adding ‘MMSE-IRC receiver’ in the capability definition was to differentiate with 8Rx R-ML receiver that may be introduced in the next release. However, based on the latest RAN plenary meeting conclusion, at least for Rel-20, there will be no 8Rx R-ML requirements.

Proposal 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

We would also like to point it out that even if companies cannot reach consensus, this issue can still be solved in the Rel-18 maintenance part. And for this Rel-19 WI, we can firstly follow the existing 8Rx receiver definition and further discuss whether to update the definition in the maintenance part.

Proposal 2: If companies cannot reach consensus, for this Rel-19 WI, companies can firstly follow the existing 8Rx receiver definition and further discuss whether to update the definition in the maintenance part.

Conclusion

Proposal 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 2: If companies cannot reach consensus, for this Rel-19 WI, companies can firstly follow the existing 8Rx receiver definition and further discuss whether to update the definition in the maintenance part.

Reference

R4-2508620 Way forward for [115][323] NR_demod_Ph5_Part2_UE, Nokia, TSG-RAN WG4 Meeting #115