3GPP TSG RAN WG4 Meeting #116                  	       R4-2509135

Bengaluru, IN, 25th - 29th Aug, 2025

Source:	China Telecom

Title:	Simulation results for UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	7.13.3

Document for:	Information

Introduction

In this contribution, simulation results for UE demodulation performance requirements for 8Rx with MMSE-IRC are given.

Simulation results

Table 1. Simulation results for ICI requirements

| TC# | Receiver | Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Ideal Results | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | 8.0 | 10.5 |
| 2 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A | 8.2 | 10.7 |
| 3 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | 10.6 | 13.1 |
| 4 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A | 12.7 | 15.2 |
| 5 | Baseline | TDD 30KHz SCS | 40 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | 8.1 | 10.6 |
| 6 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A | 8.3 | 10.8 |
| 11 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | 10.9 | 13.4 |
| 8 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A | 12.9 | 15.4 |

Table 2. Simulation results for MU-MIMO requirements

| Receiver | Duplex Mode and SCS | CHBW | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Ideal Results | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | FDD 15KHz SCS | 10 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 10.1 | 12.6 |
| Simplified |  |  |  |  |  |  |  |  | 13.3 | 15.8 |
| Baseline | TDD 30KHz SCS | 40 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 10.2 | 12.7 |
| Simplified |  |  |  |  |  |  |  |  | 13.1 | 15.6 |