3GPP TSG RAN WG4 Meeting #116                  	          R4-2509142

Bengaluru, IN, 25th - 29th Aug, 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 6Rx

Agenda Item:	7.1.4

Document for:	discussion

Introduction

The RAN4#114bis is the first meeting we discuss the UE demodulation performance requirements for 6Rx. According to the WID, RAN4 will define 6-layer performance requirements for FWA and 4-layer performance requirements for FWA and handheld UE devices. During RAN4#115, the Way Forward is approved in [2]

In this contribution, our views on the remaining open issues are given.

Discussion

Test applicability rule for 6Rx test cases

- Option 1: Introduce UE declaration and design proper test applicability rule in the specification as per the objective of perf part in RP-250780 if no explicit UE capability signalling to differentiate UE type introduced by RAN2. - Option 2: Introduce test applicability rule only for 6 layers cases - Option 3: Postpone the discussion after RAN2 finishes the UE capability signalling design

Regarding how to specify that ‘6 MIMO layers performance requirements will be only applicable for FWA devices’, we think it can be further discussed pending RAN2 conclusion on the 6Rx MIMO layer capability definition.

If it is finally confirmed that there is no explicit capability signaling to differentiate UE type, RAN4 needs to introduce UE declaration and design proper test applicability in the specification.

Proposal 1: The test applicability should follow Core part agreements:

- 6 MIMO layers performance requirements will be only applicable for FWA devices

- 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Proposal 2: If RAN2 finally confirmed that there is no explicit capability signaling to differentiate UE type, RAN4 needs to introduce UE declaration and design proper test applicability in the specification.

Specification structure

- Proposals - Option 1: Add a new clause for 6Rx - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

There is only one thing we would like to point it out is that the current requirements for 1/2/4/8Rx has been introduced in 38.101-4 without a Void section for 6Rx, which means 6Rx related sections may need to be specified under 8Rx.

We would like to check if it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the Rel-19 specification.

Proposal 3: RAN4 to discuss whether it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

Test cases for 4 MIMO layers for FWA and Handheld UE

- Option 1: Define one single test case - Option 1a: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE - Option 1b: One single test case for 4 MIMO layers corresponding to a Handheld UE - Option 2: Define two sets of 4 layers tests with different MIMO correlation

Firstly, we believe the 4 MIMO layer PDSCH tests should cover both FWA and handheld UEs. We would like to see if there could be large performance difference by using different MIMO correlation for different UE types, if so and if the selected MIMO correlation is feasible, we are fine to consider 2 sets of requirements for different UE types.

Proposal 4: The 4 MIMO layer PDSCH tests should cover both FWA and handheld UEs.

Proposal 5: RAN4 to investigate the performance difference by using different MIMO correlation for different UE types, if the performance difference is large enough and if the selected MIMO correlation is feasible, we are fine to consider 2 sets of requirements for different UE types.

Test applicability rule for FWA

- Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests - Other options

For 8Rx FWA, both rank 2, 4 and 8 PDSCH demodulation requirements are defined without additional applicability rule to skip some of the cases. Therefore, we propose to follow the same concept for 8Rx FWA and not to introduce additional applicability rule to skip corresponding 4 layers test if it passes the FWA 6 layers tests.

Proposal 6: Follow the same concept for 8Rx FWA and not to introduce additional applicability rule to skip corresponding 4 layers test if it passes the FWA 6 layers tests.

DMRS configuration for 6 MIMO layers case

- Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-18 eType1 DMRS 1+1

We propose not to combine 6 MIMO layer test cases with other new features defined in 5G-A.

Proposal 7: Rel-15 Type 1 DMRS 2+2.

CQI test for 6Rx

- Way forward - Option 1: Not define CQI test - Option 2: Define CQI test - Option 2a: Only define CQI requirements with rank 4 under static channel - Option 2b: Define the following CQI test - Periodic CQI reporting requirements under AWGN - Wideband CQI reporting requirements with inter-cell interference under fading conditions

RI test

- Way forward - Option 1: Not define RI test for 6 layers - Option 2: Define RI test for 6 layers

For CQI and RI testing for 6Rx, we support to follow the same test coverage for 2/4Rx, i.e., Periodic CQI reporting requirements under AWGN, Wideband CQI reporting requirements with inter-cell interference under fading conditions and RI test is defined.

Proposal 8: Define periodic CQI reporting requirements under AWGN, Wideband CQI reporting requirements with inter-cell interference under fading conditions and RI test for 6Rx.

Conclusion

Proposal 1: The test applicability should follow Core part agreements:

- 6 MIMO layers performance requirements will be only applicable for FWA devices

- 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

Proposal 2: If RAN2 finally confirmed that there is no explicit capability signaling to differentiate UE type, RAN4 needs to introduce UE declaration and design proper test applicability in the specification.

Proposal 3: RAN4 to discuss whether it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

Proposal 4: The 4 MIMO layer PDSCH tests should cover both FWA and handheld UEs.

Proposal 5: RAN4 to investigate the performance difference by using different MIMO correlation for different UE types, if the performance difference is large enough and if the selected MIMO correlation is feasible, we are fine to consider 2 sets of requirements for different UE types.

Proposal 6: Follow the same concept for 8Rx FWA and not to introduce additional applicability rule to skip corresponding 4 layers test if it passes the FWA 6 layers tests.

Proposal 7: Rel-15 Type 1 DMRS 2+2.

Proposal 8: Define periodic CQI reporting requirements under AWGN, Wideband CQI reporting requirements with inter-cell interference under fading conditions and RI test for 6Rx.

Reference

R4-2508727, Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, 3GPP TSG RAN4 Meeting #115, Malta, MAy, 2025