3GPP TSG RAN WG4 Meeting #116                  	R4-2509145

Bengaluru, IN, 25th - 29th Aug, 2025

Title:	Remaining issues for RRM requirements on UE-initiated/event-driven beam management

Source:	China Telecom

Agenda item:			7.20.4.1

Document for:	Discussion

- Introduction

In the RAN4 #115 meeting, the RRM requirements for NR_MIMO_Ph5_Part1 were discussed and WF [1] has been approved. In this paper, we present our views on some open issues.

- Discussion

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

The following options are captured in the WF:

- Option 1: (Qualcomm, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

In option 2, it specifies requirement for time window based case as well. Although the requirement for time window based case is more like functionality verification, it is still preferred to introduce the requirement given it is a useful function to avoid ping-pong.

Regarding the test case design and test applicability rule for UE supporting both case 1 and case 2, it can be further discussed in the performance part.

Proposal 1: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

The following options are captured in the WF:

- Option 1: (Nokia, Ericsson, MTK)

  - No need to capture this scenario in RAN4 requirements

- Option 2: (Xiaomi, Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Firstly, option 2 is technically correct based on the RAN1 agreement. Meanwhile, it might be not necessary to capture all the related scenarios in RAN4 requirement. So, option 1 is preferred from RAN4 requirement perspective.

Proposal 2: For the scenario when the indicated TCI state is changed during the L1 measurement time, no need to capture this scenario in RAN4 requirements.

Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

The following options are captured in the WF:

<Way forward >

- Option 1: (Samsung, Apple, Huawei, Ericsson)

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2: (Nokia, MTK)

  - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

Given the number of SSB/CSI-RS based UE-initiated/event-driven beams the UE can measure is already been defined by UE capability, option 1 is preferred by us.

Proposal 3: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Summary

In this paper, we provide our views on RRM requirements for UE-initiated/event-driven beam management. From this discussion we have the following proposals:

Proposal 1: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Proposal 2: For the scenario when the indicated TCI state is changed during the L1 measurement time, no need to capture this scenario in RAN4 requirements.

Proposal 3: For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- References

[1] R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung