3GPP TSG-RAN WG4 Meeting	R4-2509153

Bangalore, India, 25-29 Aug. 2025

Agenda item:	7.12.3 spatial channel modelling methodology

Source: 	Orange, Fraunhofer

Title: 	Performance evaluation of low- and high-resolution MIMO codebooks in SU-MIMO under CDL channel models

Document for:	Discussion

- Introduction

This contribution investigates the performance of low- and high-resolution CodeBooks (CBs), namely Type-I and enhanced Type-II (eType-II), for Single-User MIMO (SU-MIMO) systems. The evaluations are conducted over Clustered Delay Line (CDL) channel models from 3GPP TR 38.901 [1] and TR 38.753 (with convergence and alignment speedup framework introduced in TR 38.827) [2]. The simulation parameters comply with the “PMI follow” setting.

The performance of the two precoding strategies are evaluated over the CDL-C 753 channel model under both perfect and imperfect channel estimation (DMRS and CSI-RS based channel estimations). Additionally, the eigen-beamforming per SubBand (SB) bound is given in the case of perfect channel estimation. The eigen-beamforming per SB bound corresponds, for each SB, to a selected precoder from an ideal CB comprising the eigenvectors of the SB equivalent channel assuming a power equipartition between layers where rank l precoder is made of the l eigenvectors associated to the best l eigenvalues. More precisely, if we denote by H(t)∈CNR×2N1 N2  the representative/equivalent channel of SB t , the rank l precoder is made of the l eigenvectors of  HtHH(t) associated to the best l eigenvalues (where .H  is the conjugate transpose). This boundis interesting to estimate the “diversity” of the channel, i.e., its capability to support high rank transmission. Finally, the random PMI Type-I lower bound is also plotted. Note that the PMI delay is always considered for both the precoding strategies and bounds.

The study complies with the simulation parameters agreed in the latest RAN4 meeting Way Forward [3]. The antenna array configurations considered at the base station are:

- (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

- (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

- (M,N,P,Ms,Ns)= (2,8,2,1,1) for 32TX CSI-RS Ports

The UE receive array configuration is fixed as:

- (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4 RX CSI-RS Ports.

Complementing the agreed simulations for alignment, the performance over the CDL-C 753 channel model is compared against the one obtained under CDL-C 901. Additional simulation results are presented for CDL-C 901 incorporating vertical sub-array virtualization (1Y). Finally, performance is evaluated under improved CSI-RS and DMRS practical channel estimation relying on power boosting for CSI-RS and increasing the PRB bundle size for DMRS.

The first objective of this contribution is to present SU-MIMO precoding matrix indicator (PMI) simulation results for alignment and comparison. The second objective is to identify scenarios where the performance gap between eType-II and Type-I CBs is significant enough for discriminating their respective implementation (which is the underlying thread of our contributions).

All simulation results rely on the mutual information effective SINR mapping (MIESM) PHY abstraction model, as described in [4]. On the other hand, the impact of the channel estimation is not evaluated based on a noise model abstraction but relies on a direct implementation of the algorithms described in Annex B.

The MCS13 (Table 1) AWGN curve BLER vs. SNR is based on layer decoding with natural order, min sum algorithm with factor 0.7 and 25 iterations for LDPC decoding. The BLER is interpolated from the following (SNR, BLER) 2-uplets (5.75, 1.00e+00), (5.82, 9.09e-01), (5.87, 7.14e-01), (6.00, 4.76e-01), (6.07, 2.86e-01), (6.12, 1.65e-01), (6.25, 1.36e-02), (6.32, 3.51e-03).

The simulation parameters are provided in Annex A, and the CDL channel profiles are based on Table 7.7.1-3 in TR 38.901 and Table 5.1.4.3-1 in TR 38.753 (with truncation).

- Performance evaluation under CDL-C 753 with perfect channel estimation

The goal is to analyze the performance gap between Type-I and enhanced Type-II CBs under CDL-C 753, using the simulation settings described in the Way Forward [3]. The detailed parameters are provided in Annex A.

Normalized spectral efficiency curves for rank-4 transmissions with 4Tx, 8Tx, and 32Tx (BS) and 4Rx (UE) are shown below, assuming perfect channel estimation, to assess the achievable gains in ideal conditions. Eigen-beamforming per SB is also applied to provide an upper performance bound relative to eType-II.

Figure 1: CDL-C 753 channel model, 10 Hz, 4Tx-4Rx, with 7ms of PMI delay, and with perfect channel estimation

Figure 2: CDL-C 753 channel model, 10 Hz, 8Tx-4Rx, with 7ms of PMI delay, and with perfect channel estimation

Figure 3: CDL-C 753 channel model, 10 Hz, 32Tx-4Rx, with 7ms of PMI delay, and with perfect channel estimation

Observation group 1 (perfect channel estimation, CDL 753)

- The eType-II CB demonstrates 2 dB gain over Type-I CB with ideal CSI-RS and DMRS channel estimations for 32 Tx antenna ports by exploiting the channel spatial diversity. Its performance is very close to the eigen-beamforming.

- In the 8Tx and 4Tx scenarios, Type-I performance closely approaches the eigen-beamforming upper bound, leaving limited room for eType-II gains. Hence, there is no observed performance gap between eType-II and Type-I in these cases.

- Performance evaluation under CDL-C 753 with imperfect channel estimation

The goal is to analyze the performance gap between Type-I and eType-II with CDL-C 753 and simulation settings described in the WF while considering realistic CSI-RS and DMRS channel estimations to evaluate the gains that can be practically achieved in these settings.

Figure 4: CDL-C 753 channel model, 10 Hz, 4Tx-4Rx, with 7ms of PMI delay, and with imperfect channel estimation

Figure 5: CDL-C 753 channel model, 10 Hz, 8Tx-4Rx, with 7ms of PMI delay, and with imperfect channel estimation

Figure 6: CDL-C 753 channel model, 10 Hz, 32Tx-4Rx, with 7ms of PMI delay, and with imperfect channel estimation

Observation group 2 (imperfect channel estimation, CDL 753)

- All schemes experience a performance degradation due to channel estimation noise.

- In the 4Tx case, Type-I follow PMI and eType-II achieve performance comparable to that of the Type-I random PMI lower bound.

- In the 8 Tx case, eType-II yields lower performance than Type-I follow PMI. Performance degradation due to channel estimation is more pronounced for high resolution CBs, since they rely more on accurate channel estimation. Type-I, being coarser in resolution, exhibits better resilience to channel estimation noise.

- In the 32Tx case, eType-II maintains the best overall performance for positive SNR values. However, the gap over Type-I is reduced to 1.5 dB at 70% and less than 0.5 dB gain at 50% efficiency.

- Performance comparison of CDL-C 753 and 901 baseline under perfect channel estimation

This Section presents a performance comparison between the CDL-C 753 and the baseline CDL-C channel model defined in TR 38.901. The goal is to assess the impact of the channel characteristics defined in TR 38.753 such as angular spreads on the performance gap between eType-II and Type-I CBs. The evaluation is conducted under ideal CSI-RS and DMRS channel estimations. All simulations follow the parameter configurations specified in the WF for the 32Tx case.

| CDL-C 753 | CDL-C 901 |
| --- | --- |

Figure 7: CDL-C 753 and 901 channel models, 10 Hz, 32Tx-4Rx, with 7ms of PMI delay, and with perfect channel estimation

The performance of the CBs is assessed below under CDL-C 753 while re-scaling angles to meet the angular spreads of the CDL-C 901 channel model, i.e., ASD=39.0949°, ASA=71.1175°, ZSD=4.0665°, ZSA= 10.4245°.  The angle scaling procedure is detailed in Subclause 5.1.4.1 in TR 38.753.

| CDL-C 753 with angular spreads in TR 38.901 | CDL-C 901 |
| --- | --- |

Figure 8: CDL-C 753 (w. angular spreads of the CDL-C 901) and CDL-C 901 channel models, 10 Hz, 32Tx-4Rx, with 7ms of PMI delay, and with perfect channel estimation

Observation group 3 (perfect channel estimation, CDL 753 vs. 901)

- Type-I shows better performance and is closer to eType-II under CDL-C 753 compared to CDL-C 901 baseline. This can be attributed to the reduced directional diversity in CDL-C 753, due to its lower azimuth spread of departure (ASD). With fewer spatial directions, the advantage of high-resolution compared to low-resolution precoding diminishes.

- This observation is further supported by additional simulations using CDL-C 753 with the angular spread parameters of the CDL-C 901 model. Under these modified conditions, the performance gap between Type-I and eType-II increases again, confirming that spatial diversity (as reflected in angular spread) plays a key role in discriminating the performance of the two schemes.

- Comparison of AAV3 and AAV1Y settings under CDL-C 901 with perfect channel estimation

This Section provides simulation results under CDL-C 901 with two different antenna array virtualizations. The considered antenna array configurations at the base station are the following:

- AAV3 config. with 32 Tx CSI-RS ports - (M,N,P,Ms,Ns)= (2,8,2,1,1)

- AAV1Y config. with 32 Tx CSI-RS ports - (M,N,P,Ms,Ns)= (12,8,2,6,1)

| AAV3 config. | AAV1Y config. |
| --- | --- |

Figure 9: CDL-C 901 channel model, 10 Hz, 32Tx-4Rx, AAV3 (direct mapping) and AAV1Y (vertical sub-arrays) configuration @BS, with 7ms of PMI delay, and with perfect channel estimation

Observation group 4 (perfect channel estimation, CDL 901 AAV3 vs. 1Y)

- Both Type-I and eType-II schemes show slightly better performance with direct AE-to-AP mapping compared to sub-array configuration.

- Performance with the 4 PRB DMRS bundle size appears to benefit slightly more Type-I than eType-II.

- Eigen-beamforming performs better with sub-array configuration which may be due to grating lobes (antenna ports equivalently 3λ apart with 1Y virtualization).

- Type-I with random PMI also benefits from sub-array configurations.

- Performance evaluation under CDL-C 753 with improved imperfect channel estimation

This Section evaluates performance under the CDL-C 753 channel model with improved non-ideal channel estimation. Simulations are conducted with 32Tx using a PRB bundle size of 4 for DMRS-based channel estimation instead of 2, which helps improve estimation accuracy.

| RAN4 settings for CSI-RS and DMRS channel estimation | Enhanced DMRS channel estimation PRB bundle size of 4 instead of 2 PRBs |
| --- | --- |

Figure 10: CDL-C 753 channel model, 10 Hz, 32Tx-4Rx, with 7ms of PMI delay, and with imperfect channel estimation, PRB bundle size of 2 (baseline) and 4 PRBs (enhanced DMRS channel estimation)

In the following, CSI-RS-based channel estimation is considered with power boosting. This is made possible by the structure of the CDM groups in the frequency domain. That is, for each non-zero power CSI-RS RE, there are (#CDM groups per OFDM symbol − 1) REs set to zero. With 32 transmit antennas and 4 CDM groups per OFDM symbol, the CSI-RS EPRE can be increased by a factor of 4 (6 dB) compared to the baseline.

| Enhanced DMRS channel estimation, baseline CSI-RS channel estimation PRB bundle size of 4 instead of 2 PRBs CSI-RS EPRE=1 | Enhanced DMRS and CSI-RS channel estimations PRB bundle size of 4 instead of 2 PRBs CSI-RS EPRE=4 |
| --- | --- |

Figure 11: CDL-C 753 channel model, 10 Hz, 32Tx-4Rx, with 7ms of PMI delay, and with imperfect channel estimation, PRB bundle size of 4 PRBs (enhanced DMRS channel estimation), and CSI-RS EPRE of 4 (enhanced CSI-RS channel estimation)

Hereafter, we consider the CDL-C 753 channel model with angular scaling applied to match the angular spreads of the CDL-C 901 model. Channel estimation is assumed to be imperfect and enhanced, by considering a PRB-bundle size of 4 PRBs for DMRS channel estimation and power boosting for CSI-RS channel estimation.

Figure 12: CDL-C 753 (w. angular spreads of the CDL-C 901) channel model, 10 Hz, 32Tx-4Rx, with 7ms of PMI delay, and with imperfect channel estimation (PRB bundle size of 4 PRBs and CSI-RS EPRE of 4)

Observation group 5 (CDL-C 753 with 901 angular spreads and enhanced channel estimation)

- Performance is slightly improved when using a PRB bundle size of 4 instead of 2 for DMRS-based channel estimation.

- Power boosting applied to CSI-RS based channel estimation further enhances performance, particularly in the low SNR region. The impact is more noticeable for eType-II, as high-resolution CBs are more sensitive to channel estimation accuracy. As a result, eType-II benefits more from improved CSI quality than Type-I, leading to a larger performance gap between the two schemes.

- CDL-C 753 with angular spreads matched to CDL-C 901, under enhanced channel estimation (CSI-RS power boosting, 4 PRB bundle size) conditions, reveals a significant performance gap between Type-I and eType-II CBs: up to 5 dB at 70%–90% of the maximum spectral efficiency.

General observation 1

The 38.753 CDL channel model can demonstrate the benefit of high-resolution compared to low-resolution codebooks for relevant scenarios, in contrast to the legacy TDL channel model, and contributor implementations have shown to be aligned in terms of SU-PMI performance.

Proposal 1

RAN4 to agree to use and recommend usage of the 38.753 CDL channel model for PMI testing.

General Observation 2

To improve performance requirements for high-resolution codebooks under the 38.753 CDL model, RAN4 may define specific CDL parameters as well as CSI-RS channel estimation assumptions to better differentiate the performance of high- and low-resolution codebooks, in the future.

Proposal 2

RAN4 shall consider the following 38.753 CDL model and parameter configurations to improve differentiation of high- and low-resolution codebook performance:

- Increase the number of antenna ports to at least 32.

- Align the angular spread parameters of the 38.753 CDL model with those of 38.901, i.e., ASD=39.0949°, ASA=71.1175°, ZSD=4.0665°, ZSA= 10.4245°.

- Apply power boosting for CSI-RS based channel estimation.

# Conclusion

Observation group 1 (perfect channel estimation, CDL 753)

- The eType-II CB demonstrates 2 dB gain over Type-I CB with ideal CSI-RS and DMRS channel estimations for 32 Tx antenna ports by exploiting the channel spatial diversity. Its performance is very close to the eigen-beamforming.

- In the 8Tx and 4Tx scenarios, Type-I performance closely approaches the eigen-beamforming upper bound, leaving limited room for eType-II gains. Hence, there is no observed performance gap between eType-II and Type-I in these cases.

Observation group 2 (imperfect channel estimation, CDL 753)

- All schemes experience a performance degradation due to channel estimation noise.

- In the 4Tx case, Type-I follow PMI and eType-II achieve performance comparable to that of the Type-I random PMI lower bound.

- In the 8 Tx case, eType-II yields lower performance than Type-I follow PMI. Performance degradation due to channel estimation is more pronounced for high resolution CBs, since they rely more on accurate channel estimation. Type-I, being coarser in resolution, exhibits better resilience to channel estimation noise.

- In the 32Tx case, eType-II maintains the best overall performance for positive SNR values. However, the gap over Type-I is reduced to 1.5 dB at 70% and less than 0.5 dB gain at 50% efficiency.

Observation group 3 (perfect channel estimation, CDL 753 vs. 901)

- Type-I shows better performance and is closer to eType-II under CDL-C 753 compared to CDL-C 901 baseline. This can be attributed to the reduced directional diversity in CDL-C 753, due to its lower azimuth spread of departure (ASD). With fewer spatial directions, the advantage of high-resolution compared to low-resolution precoding diminishes.

- This observation is further supported by additional simulations using CDL-C 753 with the angular spread parameters of the CDL-C 901 model. Under these modified conditions, the performance gap between Type-I and eType-II increases again, confirming that spatial diversity (as reflected in angular spread) plays a key role in discriminating the performance of the two schemes.

Observation group 4 (perfect channel estimation, CDL 901 AAV3 vs. 1Y)

- Both Type-I and eType-II schemes show slightly better performance with direct AE-to-AP mapping compared to sub-array configuration.

- Performance with the 4 PRB DMRS bundle size appears to benefit slightly more Type-I than eType-II.

- Eigen-beamforming performs better with sub-array configuration which may be due to grating lobes (antenna ports equivalently 3λ apart with 1Y virtualization).

- Type-I with random PMI also benefits from sub-array configurations.

Observation group 5 (CDL-C 753 with 901 angular spreads and enhanced channel estimation)

- Performance is slightly improved when using a PRB bundle size of 4 instead of 2 for DMRS-based channel estimation.

- Power boosting applied to CSI-RS based channel estimation further enhances performance, particularly in the low SNR region. The impact is more noticeable for eType-II, as high-resolution CBs are more sensitive to channel estimation accuracy. As a result, eType-II benefits more from improved CSI quality than Type-I, leading to a larger performance gap between the two schemes.

- CDL-C 753 with angular spreads matched to CDL-C 901, under enhanced channel estimation (CSI-RS power boosting, 4 PRB bundle size) conditions, reveals a significant performance gap between Type-I and eType-II CBs: up to 5 dB at 70%–90% of the maximum spectral efficiency.

General observation 1

The 38.753 CDL channel model can demonstrate the benefit of high-resolution compared to low-resolution codebooks for relevant scenarios, in contrast to the legacy TDL channel model, and contributor implementations have shown to be aligned in terms of SU-PMI performance.

Proposal 1

RAN4 to agree to use and recommend usage of the 38.753 CDL channel model for PMI testing.

General Observation 2

To improve performance requirements for high-resolution codebooks under the 38.753 CDL model, RAN4 may define specific CDL parameters as well as CSI-RS channel estimation assumptions to better differentiate the performance of high- and low-resolution codebooks, in the future.

Proposal 2

RAN4 shall consider the following 38.753 CDL model and parameter configurations to improve differentiation of high- and low-resolution codebook performance:

- Increase the number of antenna ports to at least 32.

- Align the angular spread parameters of the 38.753 CDL model with those of 38.901, i.e., ASD=39.0949°, ASA=71.1175°, ZSD=4.0665°, ZSA= 10.4245°.

- Apply power boosting for CSI-RS based channel estimation.

- References

[1] 3GPP TR 38.901 V19.0.0, "Study on channel model for frequencies from 0.5 to 100 GHz", July 2025.

[2] 3GPP TR 38.753 V0.1.0, " Study on spatial channel model for demodulation performance Requirements for NR", June 2025.

[3] R4-2508624, “Way Forward for [115][321] NR_SCM,” Nokia, 3GPP RAN4#115, May 2025.

[4] X. Fu, D. Le Ruyet, R. Visoz, V. Ramireddy, M. Grossmann, M. Landmann, W. Quiroga, "A Tutorial on Downlink Precoder Selection Strategies for 3GPP MIMO Codebooks," in IEEE Access, vol. 11, pp. 138897-138922, 2023 (URL).

[5] S. Sesia, I. Toufik, and M. Baker, “LTE: The UMTD Long Term Evolution from Theory to Practice,” 2nd ed. Wiley, 2011.

[6] L. Berrah, R. Visoz, D. Le Ruyet, “Noise-Robust CSI Prediction for 3GPP Release 18 Enhanced Type-II Doppler Codebook,” IEEE PIMRC 2025

# Annex A: Simulation parameters

This Section provides simulation assumptions for SU PMI performance evaluation. It also details CDL channel model parameters and AAV configurations @BS and @UE.

| BS antenna array configuration | M,N,P,Ms,Ns=2,8,2,1,1-32Tx |
| --- | --- |
|  | M,N,P,Ms,Ns=1,4,2,1,1- 8Tx |
|  | M,N,P,Ms,Ns=1,2,2,1,1- 4Tx |
| BS antenna spacing | dH,dV=0.5,0.5 λ |
| UE antenna configuration | M,N,P,Ms,Ns=1,2,2,1,1-4Rx |
| UE antenna spacing | dH,dV=0.5, - λ |
| BS antenna polarization slant angles | (+45,-45°) |
| UE antenna polarization slant angles | (0,90°) |
| BS antenna radiation pattern | Defined by Table 7.3-1 in TR 38.901 |
| UE antenna radiation pattern | Omnidirectional |
| Antenna pannel placement | YZ Plane |
| BS height | 25 m |
| UE height | 1.5 m |
| Distance BS-UE (D2D) | 100 m |
| BS antenna orientation | α=0°, β=10°,γ=0° |
| UE antenna orientation | α=180°, β=0°,γ=0° |
| UE speed | 3km/h |
| UE travel direction | ϕv,θv=(65°,90°) |
| MCS/RI/PMI delay | 7 ms |
| Carrier frequency | 3.5 GHz (FR1) |
| Bandwidth | 40 MHz |
| Subcarrier Spacing | 30 kHz |
| Number of Sub-bands | N3=14 |
| Sub-band size | NSB=8 PRBs |
| Total number of PRBs (40MHz, SCS=30kHz) | TBW=106 |
| Rank | 4 |
| Modulation and coding scheme (MCS) | MCS 7 from Table 2 for PDSCH (corresponding to MCS 13 from Table 1) |
| Retransmission type | Chase |
| Max. number of retransmissions | NRTmax=4 |
| Channel model | 3GPP UMa CDL-C as described in TR 38.753 with truncation and TR 38.901 rms Delay Spread=365ns |
| Polarization model | Model-1 in TR 38.901 |
| Detector at the receiver | LMMSE |
| eType-II/Type-I spatial domain basis (W1) | N1,N2,O1,O2=(8,2,4,4) for 32Tx |
|  | N1,N2,O1,O2=(4,1,4,1) for 8Tx |
|  | N1,N2,O1,O2=(2,1,4,1) for 4Tx |
| eType-II configuration | paramCombination-r16=6 L=4,pν=1/4,β=1/2 |
| cqi-FormatIndicator | Wideband |
| pmi-FormatIndicator | Not configured for eType II Wideband for Type I |
| CSI-RS channel estimation | Row 17 for 32Tx, CDM4 (FD2, TD2), density ρ=1, First sub-carrier index k0,k1,k3,k4=(2,4,6,8), First OFDM symbol index (l0,l1)=(5,12) |
|  | Row 8 for 8Tx, CDM4 (FD2, TD2), density ρ=1, First sub-carrier index k0,k1=(4,6), First OFDM symbol index l0=5 |
|  | Row 4 for 4Tx, FD-CDM2, density ρ=1, First sub-carrier index k0=4, First OFDM symbol index l0=5 |
| CSI-RS EPRE | 1 |
| DMRS channel estimation | Type-1 single-symbol PRB-bundle size of 2 PRBs |

# Annex B: Channel estimation algorithms

## CSI-RS based channel estimation

We follow the approach in [5] denoted mismatched LMMSE assuming that the channel PDP is uniform and that the RMS delay spread is known to the receiver. More details can be found in [6], Section III.B.

## DMRS based channel estimation

Since the PRB bundle is limited to 2 or 4, we simply assume that the channel remains the same within a PRB bundle. A simple averaging of the Least Square estimates over the PRB bundle is performed.

# Annex C: CDL-C channel coefficients generation

This appendix summarizes the steps for generating angles, phases, and channel coefficients according to the 3GPP TR 38.901 and TR 38.753 specifications for CDL channel models.

## Step 1: Generate departure and arrival angles

For the CDL 901 channel model, the azimuth angle of arrival (AOA) of the m-th sub-path within the n-th cluster is given by

ϕn,m,AOA =ϕn,AOA+cASAαm,

where

- ϕn,AOA​ is the cluster AOA given in Table 7.7.1-3 in TR 38.901 and Table 5.1.4.3-1 in TR 38.753 (with truncation to 12 significant clusters),

- cASA​ is the cluster-wise root-mean-square (rms) azimuth spread of arrival given in Table 7.7.1-3 in TR 38.901 and Table 5.1.4.3-1 in TR 38.753 (with truncation),

- αm​ is the ray offset angle within the cluster, as per Table 7.5-3 in TR 38.901.

Similar procedures apply for azimuth angle of departure (AOD) ϕn,m,AOD, zenith angle of arrival (ZOA) θn,m,ZOA, and zenith angle of departure (ZOD) θn,m,ZOD.

The provided CDL table in specification include the delay scaling for RMS delay of 365ns and desired angle scaling.

## Step 2: Coupling of ray angles within clusters

- For CDL 901, AOD​ are coupled randomly to AOA within each cluster. Similarly, ZOD are randomly coupled with ZOA​, and AODs are coupled with ZODs within the cluster.

- In contrast, CDL 753 applies a fixed coupling pattern for ray angles as specified in Table 5.1.4.2-1 in TR 38.753.

## Step 3: Initial phases for polarization matrices

- For 901, initial phases {Φn,mθθ,Φn,mθϕ,Φn,mϕθ,Φn,mϕϕ}  for each ray m and cluster n and for the four polarization components are drawn independently from a uniform distribution over [−π,π].

- For 753, fixed initial phases per ray and polarization are used, as defined in Table 5.1.4.2-2 of TR 38.753.

## Step 4: Cross-Polarization Power Ratios (XPR)

Generate the cross-polarization power ratios (XPR) for each ray m of each cluster n as

κn,m=10X/10,

with X = 7 dB for both CDL-C 901 and 753.

## Step 5: Channel coefficients generation

For each cluster n, transmit antenna element s, and receive antenna element u, the instantaneous channel coefficient at time t is

hu,s,nt=mαn,mejβs,n,m+βu,n,m+γn,mt.

- Coefficients αn,m

αn,m=PnMFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθ κn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD.

- Frx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOA is the UE antenna element field pattern in GCS for antenna element u.

- Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD is the BS antenna element field pattern in GCS for antenna element s.

- expjΦn,mθθ κn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ is the 2x2 polarization matrix.

The transmit antenna element patterns are obtained from the local coordinate system (LCS) rotated to the global coordinate system (GCS). The standard defines two polarization models that we describe hereafter:

Model-1

The antenna element field pattern for a purely vertical polarized antenna is given as

F''θ''θ'',ϕ''F''ϕ''θ'',ϕ'=A ''θ'' ,ϕ'' 0,

where A ''θ'',ϕ' is generated according to Table 7.3-1 in TR 38.901 for sector antenna, which defines the 3D element pattern as a function of zenith and azimuth angles in LCS''.

The LCS’’ is defined as the coordinate system of a single antenna element, that is, transformation include polarization angles and rotations of the antenna panel (, , ). Transformation to the GCS is given as

Fθθ ,ϕFϕθ,ϕ=cos ψ-sinψsinψcosψF''θ''θ'',ϕ''F''ϕ''θ'',ϕ'',

Here, the angle ψ is expressed as a function of mechanical orientation (, , ) and azimuth/zenith angles in GCS (, ), and is given by

We define in this case two rotations for the slant angles ζ0 and ζ1.

Example: for the Pannel orientations α,β,γ=(0,10°,0) at BS and α,β,γ=(180°,0,0) at the UE, when considering cross-polarization at BS and dual-polarized antennas at UE (vertical and horizontal), rotations are:

- LCS’’ of the polarization +45° at BS R1=(0,10°,+45°).

- LCS’’ of the polarization -45° at BS  R2=(0,10°,-45°).

- LCS’’ of vertical polarization at UE  R3=(180°,0,0).

- LCS’’ of the polarization +90° at UE R4=(180°,0,90°).

Model-2

The antenna element field patterns are given by

F'θ'θ',ϕ'F'ϕ'θ',ϕ'=A 'θ',ϕ'cosζsinζ ,

Here ζ is the polarization slant angle, A 'θ',ϕ' is generated according to Table 7.3-1 in TR 38.901 for sector antenna while considering A 'θ',ϕ'=A ''θ'',ϕ' with θ''=θ' and ϕ''=ϕ'.  Here, the single-primed angles are expressed in LCS’ that only considers antenna panel orientation. Then, the fields are transformed to GCS as follows

Fθθ ,ϕFϕθ,ϕ=cos ψ-sinψsinψcosψF''θ''θ'',ϕ''F''ϕ''θ'',ϕ'',

We define in this case a single rotation at BS and at UE that do not consider polarization slant angles.

Example: for the Pannel orientations α,β,γ=(0,10°,0) at BS and α,β,γ=(180°,0,0) at the UE, rotations are:

- LCS’ at BS R5=(0,10°,0).

- LCS’ at UE  R6=(180°,0,0).

- Coefficients βs,n,m/βu,n,m

Figure: phase difference between rays in a 2D illustrative setting

βs,n,m=2πλrn,mD.dstx,

where the spherical unit vector of departure in BS LCS is defined as

rn,mD=sinθn,m,ZODcosϕn,m,AODsinθn,m,ZODsin⁡(ϕn,m,AOD)cosθn,m,ZOA .

And the location vector of transmit antenna element s is given in BS LCS by

dstx=0nHsΔHBSnVsΔVBS ,

the ( nHs,nVs)  can be deduced from the antenna numbering of TR 38.897 as

The location vector is transformed to GCS by considering the panel orientation at BS (rotation R5).  The coefficients βu,n,m are defined in the same manner by considering arrival angles and orientations of the antenna array at the UE side (rotation R6).

- Coefficients γn,m

γn,m=2πλrn,mATv,

where the spherical unit vector of arrival in GCS is given by

rn,mA=sinθn,m,ZOAcosϕn,m,AOAsinθn,m,ZOAsin⁡(ϕn,m,AOA)cosθn,m,ZOA ,

and the velocity vector is defined in GCS as

v=Vm sinθvcosϕvsinθvsinϕvcosθvT,

here Vm is the UE velocity and (ϕv,θv) represents the travelling direction.

The channel impulse response for transmit antenna s and receive antenna u is given as

hu,st,τ=nhu,s,ntδτ-τn,

where τn represents the delay for cluster n.

In TR 38.753, absolute delays are given for a rms delay spread (DS) of 365 ns, while in TR 38.901, the delays need to be scaled by the rms DS.

- Antenna numbering

The antenna ports numbering s should follow the precoding design of TS38.214 Subclause 5.2.2.2. Therefore, the antenna elements or ports for a given polarization are numbered in an increasing order from bottom to top, column by column following the order defined in TR38.897. The two polarizations are dealt with independently and are associated to a different LCS’’. Finally, the rth row hnr of matrix Hn∈CNR×2N1N2, where N1 is the number of CSI-RS ports in the horizontal dimension, N2 in the vertical dimension and NR  is the number of receive antennas, is defined by the stacking of the row hnr,0∈CN1N2 associated to the first polarization 0 and hnr,1 associated to the second polarization, i.e.,  hnr=hnr,0hnr,1 where hnr,p=[hr,0,n⋯hr,N1N2-1,n] are computed independently for p=0,1.