3GPP TSG-RAN WG4 Meeting #116	R4-2509160

Bangalore, India, 25-29 August, 2025

Source:	Nokia

Title:	On LTM event-triggered L1 reporting

Agenda item:	7.25.3.1

Document for:	Discussion

Introduction

In the last meeting, RAN4 agreed CRs for the LTM event-triggered reporting delay requirements, including also event-triggered periodic reporting requirements. In this contribution we discuss the issues that remained open regarding event-triggered L1 reporting requirements.

Discussion

Requirement applicability

The following agreements and proposals were made earlier regarding applicability of cell switch delay requirements:

Issue 2-4: Impact on other LTM requirements Agreement in RAN4#114bis: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS Candidate options: - Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei) - For FR1: - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met - The related cell switch delay and TCI state activation requirements are applicable - The above optimization is not applicable to FR2 - Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia) - Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report. - Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia) - Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple) - Number of active TCI does not exceed that UE can support. - Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK) - Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE) - Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///) - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

One company pointed out in their contribution in the last meeting that leave condition is mandatory in the LTM L1 event-triggered reporting configuration. Hence, including such condition has no meaning in the RAN4#114bis agreement for requirement applicability.

Leave condition configuration is mandatory and hence always present in the event-triggered reporting configuration.

As we have proposed, in our view it is sufficient to define that the condition continues to apply after the report is sent. However, some companies wanted to include the leave condition in the requirement. We believe that the intention of the “leaving condition of the event is configured and corresponding condition is not met” scenario was to refer to on leave reporting being configured and on leave report not being sent after the event-triggered L1 report. To respond to companies’ concerns, we believe that the agreement can be updated to:

In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting and on leave reporting, and the UE has sent an event-triggered L1 report for the target cell and has not sent on leave report before reception of the cell switch command.

When on leave reporting is not configured, requirement applicability could be extended to apply for a longer time than the 1280 ms the requirements apply currently. We can consider e.g. 5 second applicability based on L1 event-triggered report without on leave reporting configuration.

In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting without on leave reporting, and the UE has sent an event-triggered L1 report for the target cell within [5 seconds] before reception of the cell switch command

Furthermore, it was pointed out that in FR2, if event-triggered report is followed by TCI state activation, the UE may be allowed to stop L1 measurements on cells without active TCI states. Hence, cell switch delay applicability cannot be guaranteed based on the L1 report after TCI state activation for these cells. Here the options are either to limit the enhanced applicability outside of this scenario or keep the legacy requirements for FR2.

Either limit proposals 1 and 2 to the scenario where the UE is required to perform L1 measurements on the target cell according to measurement requirements, or limit proposals 1 and 2 to FR1 only.

DL synchronization with event-triggered L1 reporting

When UE is configured with event-triggered L1 reporting, the UE may send an event-triggered L1 report, which may be followed by TCI state activation. Alternatively, TCI state activation may happen based on L3 report, and L1 report may follow after this, as has been brought up by one company in the last meeting. Additionally, there may be a second L1 report triggered after TCI state activation, if cell switch command is not sent shortly after TCI state activation command.

For the scenario where TCI state activation is followed by a L1 report, the UE may no longer have DL synchronization with the activated TCI state, as the current requirements allow the UE to stop keeping track of the DL sync after 160/480 ms in most cases, and event-triggered reporting condition may not become fulfilled within this time. When the UE sends an event-triggered report associated to an activated TCI state, we think it makes sense that the UE would renew the DL synchronization for this TCI state based on the report, if the UE does not have the DL sync already. This should be captured as an additional condition for Tfirst-RS and TRS-proc = 0 as:

Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command.

UE capability

Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

For L3 event-triggered reporting, RAN4 has defined capabilities for the support of event-triggered reporting in section 9.1.4, where the UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 9.1.4.2-1.

We are not aware that RAN1 would have had any discussions related to the number of supported events, so RAN4 should take care of this. The L1 events the UE shall be able to support should be added on top of the existing L3 events i.e. introduction of L1 events shall not impact the existing UE capabilities to support L3 events. Hence, we think RAN4 needs to define either a new table for L1 events to be supported or add new rows to the existing Ecat table to cover intra- and inter-frequency LTM events to be supported. The supported number of events has to be agreed, and both SSB and CSI-RS based events shall be included.

Introduce the number of supported LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

Conclusion

The following Observations and Proposals were made:

Observation 1: Leave condition configuration is mandatory and hence always present in the event-triggered reporting configuration.

Proposal 1: In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting and on leave reporting, and the UE has sent an event-triggered L1 report for the target cell and has not sent on leave report before reception of the cell switch command.

Proposal 2: In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting without on leave reporting, and the UE has sent an event-triggered L1 report for the target cell within [5 seconds] before reception of the cell switch command

Proposal 3: Either limit proposals 1 and 2 to the scenario where the UE is required to perform L1 measurements on the target cell according to measurement requirements, or limit proposals 1 and 2 to FR1 only.

Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command.

Proposal 5: Introduce the number of supported LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.