3GPP TSG-RAN WG4 Meeting #116	R4-2509163

Bangalore, India, 25-29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for UEIBM requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UEIBM requirements are not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TCI state switching delay with DL sync based on UEIBM report - Editorial change for event-triggered reporting delay - Addition of reporting delay requirements for the window-based approach. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain unfinished. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.13, 9.5.3.X, 9.13.3.X , New 9.5.3.X2, New 9.13.3.X2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1

### 8.15.3	MAC-CE based downlink TCI state switch delay

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in DL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in clause 8.15.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch.

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk = 0 under the following conditions:

-	target TCI state is not in the active TCI state list for PDSCH/PDCCH, or

-	UE configured with CSI-ReportConfig with eventType-r19 has sent an event-triggered L1-RSRP measurement report for the RS associated to the target TCI state, and the time between sending the report and receiving the MAC-CE activation command is at least Tfirst-SSB_UEIBR + TSSB-proc ms, where

-	Tfirst-SSB_UEIBR is time to first SSB transmission after the UE has transmitted the measurement report; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state,

1 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc)) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .

Where

-	T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

-	TOuk = 1 for CSI-RS based L1-RSRP measurement, and 0 for SSB based L1-RSRP measurement when TCI state switching involves QCL-TypeD

-	TOuk = 1 when TCI state switching involves other QCL types only

-	Tfirst-SSB is time to first SSB transmission after L1-RSRP measurement when TCI state switching involves QCL-TypeD;

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE for other QCL types;

-	The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

Change #2

9.5.3.X	Event Triggered Reporting [for the UE initiated beam management]

For a UE configured with a CSI-ReportConfig with eventType-r19 and and with dl-OrJointTCI-StateList，reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than  Tevaluate +  Tfirst_UL_channel, where:

- Tevaluate is the maximum of L1-RSRP measurement periods (TL1-RSRP_Measurement_Period_SSB or TL1-RSRP_Measurement_Period_CSI-RS) on the beams corresponding to the event:

- For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period.

- If the newBeamResourceSet-r19  is based on SSB, or if the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is based on SSB:

- the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-2 assuming TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

- If the reference signal in the indicated TCI state and newBeamResourceSet-r19 are all based on CSI-RS:

- the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the reference signal in the indicated TCI state for the current beam.

- the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-2 assuming TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

- Tfirst UL channel is from the time point at RS which triggers the L1 reporting to the time point at next PUCCH occasion

Editor’s note: FFS on Event-1 and event-7.

Change #3

9.13.3.X	Event triggered reporting [for UE initiated beam management]

For a UE configured with a CSI-ReportConfig with eventType-r19 and and with dl-OrJointTCI-StateList，reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clause 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report after condition is met over the air and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than Tevaluate +  Tfirst_UL_channel, where:

- Tevaluate is the maximum of L1-RSRP measurement periods (TL1-RSRP_Measurement_Period_SSB or TL1-RSRP_Measurement_Period_CSI-RS) on the beams corresponding to the event:

  - For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, UE shall perform L1-RSRP measurement based on current beam and new beam’s L1-RSRP measurement period.

  - If the SS/PBCH block is QCLed with the reference signal in the indicated TCI state, the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB_CDP with TReport. = 0, and TSSB_CDP is the periodicity of the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam

  - If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP with TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

- Tfirst UL channel is the time to first PUCCH occasion after event triggered condition is realised at the UE.

Editor’s note: FFS on Event-1 and event-7.

Change #4

9.5.3.X2 Event -triggered reporting delay with eventInstanceCount-r19 > 1

For a UE configured with a CSI-ReportConfig with eventType-r19 and with dl-OrJointTCI-StateList and [eventInstanceCount-r19 > 1]， reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

When a previous timer is not running for the beam that will trigger the report, the event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the time when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than Tevaluate + Ttimer + Tfirst_UL_channel.

When a previous timer based on which a report is not triggered is running for the beam that will trigger the report, the event triggered L1-RSRP measurement reporting delay is defined as the time between the start of the timer within which a measurement report will be triggered and the time when the UE transmits the first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than Ttimer + Tfirst_UL_channel.

Where:

- Tevaluate is as defined in clause 9.5.3.X

- Ttimer is the time from the start of the timer until M event occasions have occurred during the timer. The value of Ttimer shall not be larger than the configured timer duration.

- Tfirst UL channel is the time from the time point at RS at which M event occasions become fulfilled to the next PUCCH occasion.

Change #5

9.13.3.X2 Event -triggered reporting delay with eventInstanceCount-r19 > 1

For a UE configured with a CSI-ReportConfig with eventType-r19 and with dl-OrJointTCI-StateList and [eventInstanceCount-r19 > 1]，reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

When a previous timer is not running for the beam that will trigger the report, the event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the time when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than Tevaluate + Ttimer + Tfirst_UL_channel.

When a previous timer based on which a report is not triggered is running for the beam that will trigger the report, the event triggered L1-RSRP measurement reporting delay is defined as the time between the start of the timer within which a measurement report will be triggered and the time when the UE transmits the first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than Ttimer + Tfirst_UL_channel.

Where:

- Tevaluate is as defined in clause 9.13.3.X

- Ttimer is the time from the start of the timer until M event occasions have occurred during the timer. The value of Ttimer shall not be larger than the configured timer duration.

- Tfirst UL channel is the time from the time point at RS at which M event occasions become fulfilled to the next PUCCH occasion.

End of changes