3GPP TSG-RAN WG4 Meeting #116	R4-2509166

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.13.3

Source:	MediaTek Inc.

Title:	Simulation results for UE demodulation requirements with 8Rx MMSE-IRC receiver

Document for:	Information

- Introduction

In this contribution, we provide our simulation results based on the agreed assumptions [1]-[3].

- Simulation results for PDSCH requirements with inter-cell interference

The simulation results are based on Table A in Annex.

Table 1. Alignment results for 8Rx MMSE-IRC receiver with inter cell interference

| TC# | Receiver | Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model | Antenna correlation | SNR [dB] @ 70% Max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 8.2 |
| 2 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 8.3 |
| 3 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 10.8 |
| 4 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 14.4 |
| 5 | Baseline | TDD 30KHz SCS | 40 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 8.7 |
| 6 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 8.6 |
| 7 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 11.1 |
| 8 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 14.9 |

Table 2. Impairment results for 8Rx MMSE-IRC receiver with inter cell interference

| TC# | Receiver | Duplex Mode and SCS | CHBW | Scenario | MIMO | Rank (target UE) | MCS (target UE) | INR1 | INR2 | Channel Model | Antenna correlation | SNR [dB] @ 70% Max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 10.7 |
| 2 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 10.8 |
| 3 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 13.3 |
| 4 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 16.9 |
| 5 | Baseline | TDD 30KHz SCS | 40 MHz | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 | TDLA30-10 | ULA Low | 11.2 |
| 6 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 11.1 |
| 7 | Simplified |  |  | HomNet | 2T8R | 2 | 17 | 7.77 | 2.29 |  |  | 13.6 |
| 8 |  |  |  | HetNet | 4T8R | 4 | 13 | 7.58 | N/A |  |  | 17.4 |

- Simulation results for PDSCH requirements with intra-cell interference

The simulation results are based on Table B in Annex.

Table 3. Alignment results for 8Rx MMSE-IRC receiver with intra-cell inter-user interference

| TC Number | Receiver | Duplex Mode and SCS | CHBW | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Modulation order for the co-scheduled UE | SNR [dB] @ 70% Max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 11.2 |
| 2 | Simplified |  |  |  |  |  |  |  |  |  | 13.5 |
| 3 | Baseline | TDD 30KHz SCS | 40 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 11.2 |
| 4 | Simplified |  |  |  |  |  |  |  |  |  | 13.6 |

Table 4. Impairment results for 8Rx MMSE-IRC receiver with intra-cell inter-user interference

| TC Number | Receiver | Duplex Mode and SCS | CHBW | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Modulation order for the co-scheduled UE | SNR [dB] @ 70% Max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | FDD 15KHz SCS | 10 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 13.7 |
| 2 | Simplified |  |  |  |  |  |  |  |  |  | 16.0 |
| 3 | Baseline | TDD 30KHz SCS | 40 MHz | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | 13.7 |
| 4 | Simplified |  |  |  |  |  |  |  |  |  | 16.1 |

- Simulation results for CQI requirements with inter-cell interference

The simulation results are based on Table C in Annex.

| Figure 1. Throughput ratio (gamma) versus SINR | Figure 2. BLER versus SINR |
| --- | --- |

Table 5. Summary for the simulation results for 8Rx MMCS-IRC CQI requirement (FDD)

| Duplex Mode and SCS | CHBW | MIMO (Serving Cell) | Rank | INR (dB) | SINR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TP Gain (MMSE-IRC) | BLER (MMSE-IRC) |
| Baseline Receiver |  |  |  |  |  |  |  |
| FDD; 15KHz | 10MHz | 2x8 | 2 | 10.04 | -4 | 3.27 | 0.08 |
|  |  |  |  |  | -3 | 2.89 | 0.08 |
|  |  |  |  |  | -2 | 2.82 | 0.08 |
|  |  |  |  |  | -1 | 2.57 | 0.08 |
|  |  |  |  |  | 0 | 2.20 | 0.08 |
|  |  |  |  |  | 1 | 1.99 | 0.08 |
|  |  |  |  |  | 2 | 1.83 | 0.08 |
|  |  |  |  |  | 3 | 1.75 | 0.08 |
|  |  |  |  |  | 4 | 1.65 | 0.08 |
|  |  |  |  |  | 5 | 1.59 | 0.08 |
|  |  |  |  |  | 6 | 1.52 | 0.08 |
|  |  |  |  |  | 7 | 1.45 | 0.08 |
|  |  |  |  |  | 8 | 1.35 | 0.08 |
|  |  |  |  |  | 9 | 1.28 | 0.08 |
|  |  |  |  |  | 10 | 1.21 | 0.08 |
| Simplified Receiver |  |  |  |  |  |  |  |
| FDD; 15KHz | 10MHz | 2x8 | 2 | 10.04 | -4 | 3.22 | 0.07 |
|  |  |  |  |  | -3 | 2.71 | 0.06 |
|  |  |  |  |  | -2 | 2.55 | 0.06 |
|  |  |  |  |  | -1 | 2.45 | 0.07 |
|  |  |  |  |  | 0 | 2.28 | 0.08 |
|  |  |  |  |  | 1 | 2.02 | 0.08 |
|  |  |  |  |  | 2 | 1.90 | 0.08 |
|  |  |  |  |  | 3 | 1.83 | 0.08 |
|  |  |  |  |  | 4 | 1.77 | 0.08 |
|  |  |  |  |  | 5 | 1.70 | 0.09 |
|  |  |  |  |  | 6 | 1.60 | 0.10 |
|  |  |  |  |  | 7 | 1.54 | 0.11 |
|  |  |  |  |  | 8 | 1.50 | 0.12 |
|  |  |  |  |  | 9 | 1.38 | 0.13 |
|  |  |  |  |  | 10 | 1.27 | 0.14 |

| Figure 3. Throughput ratio (gamma) versus SINR | Figure 4. BLER versus SINR |
| --- | --- |

Table 6. Summary for the simulation results for 8Rx MMCS-IRC CQI requirement (TDD)

| Duplex Mode and SCS | CHBW | MIMO (Serving Cell) | Rank | INR (dB) | SINR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TP Gain (MMSE-IRC) | BLER (MMSE-IRC) |
| Baseline Receiver |  |  |  |  |  |  |  |
| FDD; 15KHz | 10MHz | 2x8 | 2 | 10.04 | -4 | 3.28 | 0.08 |
|  |  |  |  |  | -3 | 2.87 | 0.08 |
|  |  |  |  |  | -2 | 2.87 | 0.08 |
|  |  |  |  |  | -1 | 2.63 | 0.08 |
|  |  |  |  |  | 0 | 2.26 | 0.08 |
|  |  |  |  |  | 1 | 2.05 | 0.08 |
|  |  |  |  |  | 2 | 1.85 | 0.08 |
|  |  |  |  |  | 3 | 1.79 | 0.08 |
|  |  |  |  |  | 4 | 1.66 | 0.08 |
|  |  |  |  |  | 5 | 1.58 | 0.08 |
|  |  |  |  |  | 6 | 1.54 | 0.08 |
|  |  |  |  |  | 7 | 1.47 | 0.08 |
|  |  |  |  |  | 8 | 1.37 | 0.08 |
|  |  |  |  |  | 9 | 1.33 | 0.08 |
|  |  |  |  |  | 10 | 1.23 | 0.08 |
| Simplified Receiver |  |  |  |  |  |  |  |
| FDD; 15KHz | 10MHz | 2x8 | 2 | 10.04 | -4 | 3.06 | 0.09 |
|  |  |  |  |  | -3 | 2.31 | 0.08 |
|  |  |  |  |  | -2 | 2.43 | 0.08 |
|  |  |  |  |  | -1 | 2.46 | 0.09 |
|  |  |  |  |  | 0 | 2.18 | 0.10 |
|  |  |  |  |  | 1 | 1.87 | 0.10 |
|  |  |  |  |  | 2 | 1.79 | 0.10 |
|  |  |  |  |  | 3 | 1.73 | 0.10 |
|  |  |  |  |  | 4 | 1.62 | 0.11 |
|  |  |  |  |  | 5 | 1.49 | 0.11 |
|  |  |  |  |  | 6 | 1.45 | 0.12 |
|  |  |  |  |  | 7 | 1.42 | 0.13 |
|  |  |  |  |  | 8 | 1.39 | 0.13 |
|  |  |  |  |  | 9 | 1.31 | 0.14 |
|  |  |  |  |  | 10 | 1.21 | 0.15 |

- Conclusion

In this contribution, we shared simulation results for 8Rx MMSE-IRC receiver considering interference.

- Reference

- R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, Nokia

- R4-2502275, Way forward for [114][324] NR_demod_Ph5_Part2_UE, Nokia

- R4-2504694, Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, Nokia

- Annex: Simulation Assumptions

Table A. Simulation assumptions for PDSCH requirements with inter cell interference

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for HomoNet Disabled for HetNet |
| Duplex mode |  |  | FDD/TDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 2,4 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for HomoNet N/A for HetNet |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 in 38.101-4 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for HomoNet 7.58 for HetNet | 2.29 for HomoNet N/A for HetNet |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 for FDD 8 for TDD |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 for FDD Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 for TDD |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 in 38.101-4 |  |  |  |  |  |

Table B. Simulation assumptions for PDSCH requirements with intra cell interference

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD/TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | 1000 | 1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 | 1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Single Panel Type I, For Rank 2+2: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 in 38.101-4 |  |
| Number of HARQ Processes |  |  | 4 for FDD 8 for TDD | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 for FDD Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 for TDD | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

Table C. Simulation assumptions for CQI requirements with inter cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10/40 | 10/40 |
| Duplex Mode |  |  |  | FDD/TDD | FDD/TDD |
| Subcarrier spacing |  |  | kHz | 15/30 | 15/30 |
| SINR |  |  | dB | TBD | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 | Row 5,6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 for FDD 10/1 for TDD | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 for FDD 10/1 for TDD | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 for FDD 10/1 for TDD | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 for FDD 10/9 for TDD | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 010000 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 for FDD 9.5 for TDD | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-2 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2x8 | 1x8 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |