3GPP TSG-RAN WG4 Meeting #116	R4-2509167

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.1.4

Source:	MediaTek Inc.

Title:	Discussion on demodulation and CSI requirements for 6Rx UE

Document for:	Discussion

# Introduction

Based on the WF of demodulation and CSI requirements for 6Rx UE [1], we provide our perspectives on the unresolved issues and simulation results in this contribution.

- Discussion

## 2.1 General

Issue 1-1: Test applicability rule for 6Rx tests - Proposals - Option 1: Introduce UE declaration and design proper test applicability rule in the specification as per the objective of perf part in RP-250780 if no explicit UE capability signalling to differentiate UE type introduced by RAN2. - Option 2: Introduce test applicability rule only for 6 layers cases - Option 3: Postpone the discussion after RAN2 finishes the UE capability signalling design Issue 1-2: Test applicability rule for different number Rx antenna ports - Proposals - Option 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass. - Other options Issue 1-3: Specification structure - Proposals - Option 1: Add a new clause for 6Rx - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

Since the 6-layer capability is still under active discussion in RAN2, it would be premature to finalize the test applicability rules for 6Rx in RAN4 at this stage. There are still several unresolved issues and uncertainties surrounding the test parameters for 6Rx, which require further clarification and alignment. Consequently, RAN4 can prioritize requirements design for now and address the test applicability rules at a later stage.

Regarding the specification structure, modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance. Such changes could lead to unnecessary confusion and inefficiencies in managing updates and revisions. To ensure clarity and streamline the process, we propose adding new clauses specifically for 6Rx. This approach not only preserves the integrity of the existing structure but also provides a dedicated space to address the unique requirements and parameters associated with 6Rx, facilitating better organization and future scalability.

Observation 1: The 6-layer capability is still being discussed in RAN2 and numerous unresolved issues remain regarding the test parameters for 6Rx.

Proposal 1: RAN4 can prioritize test case design for now and address the test applicability rules at a later stage.

Observation 2: Modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance.

Proposal 2: Do not change the current clause numbering and add new clauses for 6Rx requirements.

## 2.2 PDSCH requirements

4 layers requirements

Issue 2-2-1: MIMO correlation for 4 MIMO layers for Handheld UE testing - Agreement - Introduce correlation matrix for 6Rx in the specification: RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β1625*β925*β425*β125*1 - Proposal - Option 1: with α = 0 and β = 0.07179 - Option 2: with α = 0.3 and β = 0.07179 - Option 3: with α = 0 and β = 0 Issue 2-2-3: Test cases for 4 MIMO layers for FWA and Handheld UE - Proposals - Option 1: Define one single test case - Option 1a: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE - Option 1b: One single test case for 4 MIMO layers corresponding to a Handheld UE - Option 2: Define two sets of 4 layers tests with different MIMO correlation Issue 2-2-6: MCS for 4 MIMO layer case for Handheld UE with ULA medium (If agreed) - Agreement - MCS 13 (Table 1)

Handheld UE and fixed wireless access (FWA) devices differ significantly in terms of form factor and mobility. FWA equipment is generally larger and intended for stationary use, providing a stable and reliable internet connection to fixed locations such as homes, offices, and buildings. These devices are often mounted on structures or installed in designated areas to ensure optimal performance. In contrast, handheld UE devices feature a compact form factor for ease of portability, allowing users to conveniently carry and operate them in various environments. Given that differences in form factor lead to variations in antenna placement, which in turn affect MIMO correlation, we propose defining two sets of requirements for 4L6R, each tailored to different MIMO correlation scenarios.

Observation 3: Handheld UE and FWA exhibit significant differences in form factor and mobility.

Proposal 3: Define two sets of requirements for 4L6R, each tailored to different MIMO correlation scenarios for FWA and Handheld UE.

At the last meeting, RAN4 agreed adopting the legacy approach to introduce the correlation matrix for 6Rx UE; however, the specific correlation values α and β have yet to be determined. It was also agreed that MCS 13 will be used to define requirements (if introduced). We present simulation results with MCS13 and different α and β in Figure 1. Since assuming β = 0 at the UE side is overly idealistic, we are comfortable considering either “α = 0 and β = 0.07179” or “α = 0.3 and β = 0.07179” to define requirements for 4T6R4L with non-zero MIMO correlation. The required SNR achieving 70% maximum throughput are summarized in Table 1.

Figure 1. Simulation results for 4T6R4L with MCS13 (Table 1) and non-zero MIMO correlation.

Table 1. Summary for the required SNR achieving 70% maximum throughput, 4T6R4L with ULA medium

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 4 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0 and β = 0.07179 | 70 | 16.05 |
| Rank 4 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0.3 and β = 0.07179 | 70 | 23.55 |

Observation 4: For 4T6R4L ULA with non-zero MIMO correlation, it can be shown that “α = 0 and β = 0.07179” or “α = 0.3 and β = 0.07179” are testable when considering MCS13 (Table 1).

Proposal 4: Use “α = 0 and β = 0.07179” to define requirements for 4T6R4L handheld UE with non-zero MIMO correlation.

Issue 2-2-5: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low - Agreement: Down select the following two options in the next RAN4 meeting - Option 1: MCS 20 (Table 2) - Option 2: MCS 17 (Table 1)

Regarding the MCS selection for 4T6R4L with ULA Low, our simulation results are presented in Figure 2. It can be seen that both MCS 20 (Table 2) and MCS 17 (Table 1) are feasible for testing. We are comfortable proceeding with either MCS 20 (Table 2) and MCS 17 (Table 1). The required SNR achieving 70% maximum throughput are summarized in Table 2.

Figure 2. Simulation results for 4T6R4L with different MCS options and ULA low

Table 2. Summary for the required SNR achieving 70% maximum throughput, 4T6R4L with TDLA30-10 ULA Low

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 4 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 4x6 ULA Low | 70 | 12.74 |
| Rank 4 | 40 / 30 | MCS20 (MCS Table 2) | TDLA30-10 | 4x6 ULA Low | 70 | 24.11 |

Observation 5: For 4T6R4L TDLA30-10 ULA low, it can be shown that MCS 17 (Table 1) and MCS20 (Table 2) are testable.

Proposal 5: Use MCS17 (Table 1) to define requirements for 4T6R4L FWA with zero MIMO correlation.

6 layers requirements

Issue 2-3-1: Test applicability rule for FWA - Proposals - Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests - Other options Issue 2-3-3: DMRS configuration for 6 MIMO layers case - Proposals - Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-18 eType1 DMRS 1+1

To reduce the testing burden, we propose to introduce test applicability rules for UE skipping FWA 4 layers requirements if it can pass the FWA 6 layers requirements. As for DMRS configuration, we propose using Rel-15 2+2 to define requirements for FWA 6R6L as Rel-18 enhanced DMRS needs additional UE capabilities.

Proposal 6: Introduce test applicability rules to let UE skip FWA 4 layers requirements if it can pass the FWA 6 layers requirements.

Proposal 7: Use Rel-15 2+2 DMRS configuration to define 8T6R6L FWA requirements.

Based on the assumption of Rel-15 2+2 DMRS and agreed MCS 17 (Table 1), we show the simulation result for 8T6R6L in Figure 3 and summarize the required SNR in Table 3.

Figure 3. Simulation results for 8T6R6L with MCS 17 (Table 1) with TDLA30-10 ULA low

Table 3. Summary for the required SNR achieving 70% maximum throughput (FWA 8T6R6L with TDLA30-10 ULA Low)

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 6 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 8x6 ULA Low | 70 | 18.63 |

## 2.3 SDR requirements

Issue 3-1-1: Static channel and precoding for 6Rx - For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): - Option 1: H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11 - Option 2: H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

There were two options left for 8T6R static channel. We support Option 1 to have no scaling factor in static channel matrix definition.

Proposal 8: Use the following channel matrix to define static channel for 8T6R

H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Issue 3-1-1: Static channel and precoding for 6Rx - For 4T6R (Random precoding): H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j Issue 3-1-2: SDR requirements for 1024QAM with 4 layers - Proposals - Option 1: Not Define SDR requirements for 1024QAM with 4 layers - Option 2: FFS SDR 1024QAM 4-layer requirements after 6Rx static channels are determined

Based on the predetermined static channel configuration for 4T6R, we have conducted simulations to assess the feasibility of SDR requirements for 4T6R4L 1024QAM. The results of these simulations are depicted in Figure 4. The result demonstrates that the SNR required to achieve 85% throughput is below 29 dB. Given that the maximum achievable SNR for 1024 QAM is 32.04 dB (corresponding to an EVM of 2.5%), and considering the margin for RF impairments, it is appropriate to designate MCS23 as the highest achievable MCS for 1024QAM with 4 MIMO layers.

Figure 4. SDR simulation results for 4T6R4L MCS 23 (Table 4)

Observation 6: For 6Rx SDR requirements with 1024QAM and 4 layers, the SNR required to achieve 85% throughput is below 29 dB if MCS 23 in Table 4 is used.

Proposal 9: Given that the maximum achievable SNR for 1024 QAM is 32.04 dB (corresponding to an EVM of 2.5%), RAN4 can consider MCS 23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers.

Currently, there are two UE capabilities related to 1024QAM defined in TS38.306, which are pdsch-1024QAM-FR1-r17 and pdsch-1024QAM-2MIMO-FR1-r17. If RAN4 agreed to define SDR 1024QAM 4-layer requirements for 6Rx, we suggest defining new UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules.

| pdsch-1024QAM-2MIMO-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-FR1-r17. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| pdsch-1024QAM-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-2MIMO-FR1-r17. | Band | No | N/A | FR1 only |

Proposal 10: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced.

## 2.4 CSI requirements

Issue 4-1-1: CQI test for 6Rx - Proposals - Option 1: Not define CQI test - Option 2: Only define one CQI requirements with rank 4 under static channel Issue 4-1-2: RI test for only Rank 6 - Proposals - Option 1: Not define RI tests for 6Rx - Option 2: Define RI test for 6Rx

For the CSI requirements, we believe there is minimal difference between 6Rx and 2Rx, as well as between 4Rx and 8Rx. Consequently, we suggest not defining specific CQI and RI requirements

Proposal 11: Not to define CQI requirements for 6Rx.

Proposal 12: Not to define RI requirements for 6Rx.

# Conclusion

In this contribution, we provide our views about performance requirements for 6Rx UEs. Observations and proposals are summarized:

Observation 1: The 6-layer capability is still being discussed in RAN2 and numerous unresolved issues remain regarding the test parameters for 6Rx.

Proposal 1: RAN4 can prioritize test case design for now and address the test applicability rules at a later stage.

Observation 2: Modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance.

Proposal 2: Do not change the current clause numbering and add new clauses for 6Rx requirements.

Observation 3: Handheld UE and FWA exhibit significant differences in form factor and mobility.

Proposal 3: Define two sets of requirements for 4L6R, each tailored to different MIMO correlation scenarios for FWA and Handheld UE.

Observation 4: For 4T6R4L ULA with non-zero MIMO correlation, it can be shown that “α = 0 and β = 0.07179” or “α = 0.3 and β = 0.07179” are testable when considering MCS13 (Table 1).

Proposal 4: Use “α = 0 and β = 0.07179” to define requirements for 4T6R4L handheld UE with non-zero MIMO correlation.

Observation 5: For 4T6R4L TDLA30-10 ULA low, it can be shown that MCS 17 (Table 1) and MCS20 (Table 2) are testable.

Proposal 5: Use MCS17 (Table 1) to define requirements for 4T6R4L FWA with zero MIMO correlation.

Proposal 6: Introduce test applicability rules to let UE skip FWA 4 layers requirements if it can pass the FWA 6 layers requirements.

Proposal 7: Use Rel-15 2+2 DMRS configuration to define 8T6R6L FWA requirements.

Proposal 8: Use the following channel matrix to define static channel for 8T6R

H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Observation 6: For 6Rx SDR requirements with 1024QAM and 4 layers, the SNR required to achieve 85% throughput is below 29 dB if MCS 23 in Table 4 is used.

Proposal 9: Given that the maximum achievable SNR for 1024 QAM is 32.04 dB (corresponding to an EVM of 2.5%), RAN4 can consider MCS 23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers.

Proposal 10: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced.

Proposal 11: Not to define CQI requirements for 6Rx.

Proposal 12: Not to define RI requirements for 6Rx.

# Reference

- R4-2508727, Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon

# Annex: Simulation Assumptions

Table 1. Simulation assumptions for PDSCH requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: Refer to Issue 2-2-1 and 2-2- - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers - ULA Low: Option 1: MCS20 (Table 2); Option 2: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-3 |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  |  | SNR@70% max TP |