3GPP TSG-RAN WG4 Meeting # 116	R4-2509190

Bengaluru (Bangalore), India, 25th – 29th August 2025

Agenda Item:	7.9.3.3

Source: 	Nokia

Title: 	Transmitter co-existence spurious emission requirements

Document for:	Approval

# 1.	Introduction

The revised WI on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at TSG RAN#106 [1]. One of the objectives of this WI is related to the request from ETSI/TFES in [2]:

Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz..

This topic was discussed at TSG RAN4#115 [3] and the WF was agreed [4]. System level simulation results in urban macro scenario were provided in [5]. This contribution provides system level simulation results in dense urban scenario to progress this topic in RAN4.

# 2.	Discussion

It was agreed in the WF [4]:

## Sub-topic 1-1: Evaluation methodology Way forward: - Keep both MCL and system level evaluations. - Derivation approach of the final conclusions (i.e., Tx spurious emissions co-existence limits, applicable frequency ranges, etc.) is FFS. ## Sub-topic 1-2: Antenna / array gain considerations Way forward: The out-of-band antenna gain information is to be provided together with the simulation results. ## Sub-topic 1-3: Performance metrics Way forward: Use 99th percentile of the coupling loss CDF as a starting point. ## Sub-topic 1-4: Timeline Way forward: Aim for conclusion of the Tx spurious emissions co-existence levels re-evaluation not later that the end of 2025 (due to timeline in ETSI MSG TFES).

Note that the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL (UE receiver) protection [6] was derived with the worst case of MCL of 53 dB for the microcell environment [7]:

- 53 dB for the microcell environment With the worse case of MCL = 53 dB the spurious level can be derived as follows: Spurious Level – MCL < Thermal Noise Noise Figure of the UE receiver: NF = 9dB Spurious Level < Thermal Noise + MCL = kTo + NF + MCL = –174dBm/Hz + 9dB + 53dB Spurious Level < –52dBm/1MHz

To provide a like-for-like comparison, the CL from the system level simulation using the BS antenna characteristics at 7125 - 8400 MHz frequency range (instead of 6425 MHz which are yet to be agreed on) with the simulation assumptions in dense urban environment at 14800 - 15350 MHz frequency range in TR 38.922 [8] and the propagation loss calculated at the victim carrier frequency of 6425 MHz are provided in figures 1 to 3 below. As the AAS antenna gain pattern in the out-of-band frequency range has not been agreed in RAN4, the simulation results are provided here with 0 (totally uncorrelated), 0.5 (half correlated) and 1 (totally correlated) correlation between 2 antenna elements at the aggressor (2 GHz or 3.5 GHz) frequency band in view from the victim (6425 MHz) frequency band.

(a) Overall								(b) Zoom-in

Figure 1: CL using 7125 - 8400 MHz BS antenna characteristics at 6425 MHz carrier frequency (0 correlation)

(a) Overall								(b) Zoom-in

Figure 2: CL using 7125 - 8400 MHz BS antenna characteristics at 6425 MHz carrier frequency (0.5 correlation)

(a) Overall								(b) Zoom-in

Figure 3: CL using 7125 - 8400 MHz BS antenna characteristics at 6425 MHz carrier frequency (1 correlation)

It can be observed from figures 1 and 2 that the MCL in the simulated dense urban environment is about 55 dB with 0 correlation, about 53 dB with 0.5 correlation, and about 50 dB with 1 correlation. Therefore, a real-life scenario where the correlation is between 0 and 1, the MCL in the dense urban environment is between 50 dB and 55 dB. As such the 53 dB MCL (corresponds to 0.5 correlation in the simulation results) used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency.

# 3.	Conclusion

This contribution has provided system level simulation results in dense urban scenario to progress the objective of the WI for transmitter co-existence spurious emission requirements in RAN4.

Observation: The 53 dB MCL used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency.

Proposal: To keep the 53 dB MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC.

[2]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[3]	R4-2507835, “Topic summary for [115][303] NR_BS_RF_Part2_CLTA_SE”, Huawei.

[4]	R4-2508762, “Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: Tx spur co-ex”, Huawei.

[5]	R4-2507635, “On TX co-existence spurious emissions requirements”, Nokia.

[6]	3GPP TS 38.104 v19.1.0: “NR; Base Station (BS) radio transmission and reception”.

[7]	R4-99a01, “BS TDD Spurious Emission Requirements for Co-Existence UTRA-FDD/ UTRA-TDD”, Siemens.

[8]	3GPP TR 38.922 v19.1.0: “NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”.