3GPP TSG-RAN WG4 Meeting #116	R4-2509193

Bengaluru, India, 25-29 August 2025

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for SCM Demod for Comparison

Agenda item:	 7.12.3

Document for:	Information

- Introduction

In the previous RAN4 Meeting #115s, we contributed to the SCM Study Item sharing considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

According to the outcome of the discussion during the meeting and based on the approved Way Forward [2], we provide in this contribution simulation results for comparison

The data can be found in the Excel files annexed to this contribution:

- Annex A - Results collection table for NR SCM CDL Option 1

- Annex B - Results collection table for NR SCM CDL Option 2

- Annex C - Results collection table for NR SCM eTDL4C1T

- Annex D - Results collection table for NR SCM Legacy TDL

# Simulation Parameters

## CDL Option 1: Based on 38.901 CDL-C Profile

Table 1

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -38.222 | -96.938 | 96.764 | 98.141 |
| 2 | 76.6135 | -1.25 | -21.829 | 118.89 | 98.451 | 71.056 |
| 3 | 80.9935 | -3.4684 | -21.829 | 118.89 | 98.451 | 71.056 |
| 4 | 85.0085 | -5.2294 | -21.829 | 118.89 | 98.451 | 71.056 |
| 5 | 79.424 | -2.5215 | -34.158 | -124.42 | 100.86 | 67.561 |
| 6 | 232.359 | 0 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 7 | 235.352 | -2.2185 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 8 | 239.44 | -3.9794 | -5.9189 | 171.16 | 99.173 | 76.647 |
| 9 | 240.316 | -7.4215 | 44.223 | 51.882 | 106.4 | 62.843 |
| 10 | 289.6275 | -7.1215 | -50.55 | 63.395 | 94.476 | 56.552 |
| 11 | 299.7745 | -10.7215 | 49.113 | -42.068 | 107.48 | 69.832 |
| 12 | 340.764 | -11.1215 | -73.004 | 43.065 | 92.308 | 50.785 |
| 13 | 448.4025 | -5.1215 | -44.214 | 65.055 | 104.59 | 103.38 |
| 14 | 477.5295 | -6.8215 | -50.413 | -63.435 | 105.2 | 50.086 |
| 15 | 792.196 | -8.7215 | -60.193 | 78.954 | 91.706 | 51.659 |
| 16 | 989.3325 | -13.2215 | 64.61 | 26.262 | 105.2 | 121.03 |
| 17 | 1554.4985 | -13.9215 | 62.199 | -22.591 | 93.994 | 53.931 |
| 18 | 1679.1095 | -13.9215 | 55.036 | -1.6394 | 91.826 | 61.619 |
| 19 | 2003.923 | -15.8215 | -76.31 | -19.791 | 90.742 | 37.505 |
| 20 | 2046.8105 | -17.1215 | 57.378 | 4.4803 | 108.21 | 53.057 |
| 21 | 2301.8725 | -16.0215 | 58.136 | 0.22765 | 91.706 | 35.757 |
| 22 | 2422.651 | -15.7215 | 67.296 | -4.8548 | 91.585 | 52.882 |
| 23 | 2570.5855 | -21.6215 | 76.181 | -28.296 | 106.4 | 46.417 |
| 24 | 3158.0895 | -22.8215 | -91.394 | 29.27 | 109.53 | 44.669 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3775 | 15.558 | 3.6133 | 12.232 | 7 |  |

## CDL Option 2: CDL-C profile after Cluster Reduction

Table 2

| Cluster # | Cluster delay [ns] |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 |  | -7.4 |  | -38.2 |  | -96.9 |  | 96.8 |  | 98.1 |
| 2 | 132 |  | -1.2 |  | -21.8 |  | 118.9 |  | 98.5 |  | 71.1 |
| 3 | 138 |  | -5.5 |  | -34.2 |  | -124.4 |  | 100.9 |  | 67.6 |
| 4 | 402 |  | 0.0 |  | -5.9 |  | 171.2 |  | 99.2 |  | 76.6 |
| 5 | 416 |  | -10.4 |  | 44.2 |  | 51.9 |  | 106.4 |  | 62.8 |
| 6 | 502 |  | -10.1 |  | -50.6 |  | 63.4 |  | 94.5 |  | 56.6 |
| 7 | 518 |  | -13.7 |  | 49.1 |  | -42.1 |  | 107.5 |  | 69.8 |
| 8 | 776 |  | -8.1 |  | -44.2 |  | 65.1 |  | 104.6 |  | 103.4 |
| 9 | 826 |  | -9.8 |  | -50.4 |  | -63.4 |  | 105.2 |  | 50.1 |
| 10 | 1372 |  | -11.7 |  | -60.2 |  | 79.0 |  | 91.7 |  | 51.7 |
| 11 | 1712 |  | -16.2 |  | 64.6 |  | 26.3 |  | 105.2 |  | 121.0 |
| 12 | 2906 |  | -16.9 |  | 55.0 |  | -1.6 |  | 91.8 |  | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 1.4 |  | 15.6 |  | 3.6 |  | 12.2 |  | 7 |  |

## E-TDL Channel with 4 Clusters and 1 TDLC Instance (eTDL4C1T)

Table 3: 4 clusters and 1 TDLC channel instance

| TDLC-300 / XP-high / 100Hz Doppler |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | -π | -π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | π/2 | π/2 |