3GPP TSG-RAN WG4 Meeting #116							         	       	R4-2509230

# Bengaluru, India, 25th – 29th August, 2025

Title: 	MPR input for PC2 NB-IoT-NTN

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.6.3.3.1	Tx requirements	[NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In RAN#115, we provided a preliminary evaluation in [1] of the PC2 NB-IoT-NTN MPR for the corner case allocations. A way forward [2] captured a few agreements on the 12-tone and 1-tone 15KHz SCS edge and center allocations. It was observed that most of the allocations have not been evaluated. In this contribution, we provide the MPR evaluations for all the applicable allocations.

# Discussion

## 2.1 Way forward input on PC2 NB-IoT-NTN

The way forward [2] lists the open items for PC2 NB-IoT based IoT-NTN MPR and A-MPR for single tone and multi-tone allocations, and is copied below in italics.

Issue 1-1-2: MPR requirements for NB-Iot based Iot-NTN PC2

- Agreement:

  - Unified requirements for pi/2 BPSK and pi/4 QPSK for single Tone

  - The following structure and requirements considered as starting point. Further refinements on tables and wording can be made later

- Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |
| --- | --- | --- |
| SCS | 15KHz | 3.75kHz |
| Tone positions for 1 Tone allocation | [0-11] | [0-47] |
| MPR | [0] dB | TBD dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |

- Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ TBD dB | TBD dB |  | ≤ TBD dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ TBD dB |  | ≤ TBD dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| Note 1: For multi-tone allocation, only QPSK and 15KHz SCS can be supported |  |  |  |  |

Issue 1-1-5: A-MPR for NB-Iot based Iot-NTN PC2

- Agreement:

  - Wait for PC3 A-MPR baseline before concluding PC2 A-MPR for NS_04N and NS_05N (band 254)

  - No A-MPR is needed for [NS_03N] and NS_02N for NB-IoT PC2 in square brackets

  - Define [2] dB A-MPR increase for NS_24 to support NB-IoT PC2 compared to PC3

    - The exact value can be revisited once MPR is agreed.

As observed, except for the 12-tone and 1-tone 15KHz SCS edge and center allocations, most of the allocations have not been evaluated. In this contribution, we provide the MPR and A-MPR evaluations for all the applicable allocations.

## 2.2 PC2 NB-IoT-NTN measurements based on PC3 NR PA

Further measurements were performed on the same NR PC3 PA used in [1] to include the missing single-tone and multi-tone allocations. The PA calibrations compared to [1] were also reconsidered and measured at the nominal VCC which is more appropriate for the low PAPR waveforms:

- QPSK 3, 6 and 12-tone allocations. Only the allocations on the left side of the carrier were measured.

- Single-tone Pi/4 QPSK and Pi/2 BPSK 15kHz all positions (0 to 11).

- Single-tone Pi/4 QPSK and Pi/2 BPSK 3.75kHz every 4 positions (1,4,8,12,16,20,23).

- Post-PA losses of 3dB (with 0.5dB correction since the measured PA has an integrated post PA switch).

- With 28dB and 34dB image impairments to check the influence with the baseline carrier leakage, CIM3 and CIM5.

- Performed tests: Occupied BW, UTRA ACLR, NB-IoT SEM, 1dB compression point and compression level at the SEM limit.

- IBE could not be tested directly, but for 15kHz tone the in-band spectrum in 15kHz bins can be measured and represent a worst case versus IBE which is after FFT and BB corrections.

Observation on occupied BW: In our measurement campaign we found that the 12-tone input waveform from the measurement instrument would not meet the occupied BW and that there was no impact from the PA. Since the BB waveform should be designed to meet the occupied BW by applying the proper digital filtering, we ignored this fail as it is not related to the PA.

The raw data for all the cases can be found in the Annex 1.

Observations on raw results:

- In all the cases, the limitation comes from the NB-IoT SEM limit.

- Many backoff values compared to 26dBm are negative, denoting a possibility for boosting.

- Most of the inner single-tone allocations can reach up to 28dBm (2dB boosting) but requires saturating the PA (into >4dB compression).

- Other than for the 3.75kHz edge allocations, the improved image rejection has little benefit.

- There is a slight difference between Pi/4 QPSK and Pi/2 BPSK

Given the negligible impact of image rejection we focussed our evaluation on the 28dB image rejection numbers summarized in Table 1.

Table 1: Evaluated MPR per waveforms (boosting highlighted in yellow)

|  | Tone position and MPR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SC# 3.75kHz | 0-3/44-47 | 4-7/40-43 | 8-11/36-39 | 12-15/32-35 | 16-19/28-31 | 20-27 |
| 1 tone Pi/2 BPSK | 0.5 | -1.5 | -2 | -2 | -2 | -2 |
| 1 tone Pi/4 QPSK | 0.5 | -2 | -2 | -2 | -2 | -2 |
| SC# 15kHz | 0/11 | 1/10 | 2/9 | 3/8 | 4/7 | 5/6 |
| 1 tone Pi/2 BPSK | 0 | -1 | -1.5 | -1.5 | -2 | -2 |
| 1 tone Pi/4 QPSK | -1 | -2 | -2 | -2 | -2 | -2 |
| SC# 15kHz | 0-2/9-11 |  |  | 3-5/6-8 |  |  |
| 3 tone QPSK | -0.5 |  |  | -1 |  |  |
| SC# 15kHz | 0-5/6-11 |  |  |  |  |  |
| 6 tone QPSK | 0 |  |  |  |  |  |
| SC# 15kHz | 0-11 |  |  |  |  |  |
| 12 tone QPSK | 2 |  |  |  |  |  |

Observations:

- Other than for some edge allocations, boosting is feasible for 3-tone and 1-tone allocation

- For Pi/4 QPSK vs Pi/2 BPSK, outside some 0.5dB rounding, the difference in MPR is small.

- For 3.75kHz versus 15KHz single tone, the main difference is for the edge allocations (PSD)

## 2.3 Estimation of IBE

In band emissions for NB-IoT could not be tested directly in our setup, but for 15kHz SCS single-tone and multi-tone cases, the in-band spectrum in 15kHz bins can be measured. Although it does not provide the exact IBE value that is calculated after FFT and BB corrections, it represents a worst-case evaluation of IBE and permits to investigations into whether this might be a limiting factor for MPR instead of the NB-IoT SEM. The evaluation can be done for 3 and 6-tone QPSK and 15kHz SCS 1-tone Pi/4. The Table 2 below provides the power ratio [dB] between non-allocated tones and allocated tones for:

- QPSK 3-tone edge, 3-tone centre and 6-tone

- Pi/4 QPSK 15kHz 1-tone position 0 to 5

- At the maximum measured power

Table 2: Power ratio [dB] between non-allocated tones and allocated tones

| Non-Allocated | Tone position |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocated | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| 3tone 0-2 | 0.0 | -0.9 | 0.9 | 8.9 | 15.6 | 18.9 | 20.5 | 22.9 | 23.9 | 23.8 | 24.4 | 25.5 |
| 3tone 3-5 | 19.6 | 15.9 | 9.3 | -0.3 | -0.7 | 1.0 | 9.1 | 15.1 | 18.7 | 21.2 | 22.9 | 24.3 |
| 6tone 0-5 | 0.5 | -0.4 | 0.3 | -0.5 | -0.2 | 0.3 | 8.5 | 14.9 | 17.1 | 18.2 | 19.8 | 21.9 |
| 1tone 0 | 0.0 | 9.5 | 18.1 | 21.8 | 24.2 | 24.8 | 25.8 | 27.8 | 29.0 | 30.1 | 30.7 | 28.4 |
| 1tone 1 | 9.3 | 0.0 | 9.3 | 18.0 | 21.9 | 23.8 | 25.2 | 26.9 | 28.0 | 29.1 | 27.5 | 30.7 |
| 1tone 2 | 18.4 | 9.5 | 0.0 | 9.4 | 18.1 | 21.6 | 23.9 | 26.1 | 27.5 | 26.8 | 28.6 | 30.2 |
| 1tone 3 | 22.0 | 18.2 | 9.4 | 0.0 | 9.4 | 18.1 | 21.6 | 24.3 | 25.2 | 27.5 | 29.0 | 29.4 |
| 1tone 4 | 24.5 | 21.6 | 18.3 | 9.5 | 0.0 | 9.3 | 17.9 | 21.8 | 24.3 | 26.2 | 27.8 | 29.0 |
| 1tone 5 | 26.4 | 24.5 | 22.1 | 18.1 | 9.3 | 0.0 | 9.4 | 18.3 | 21.8 | 24.4 | 26.2 | 27.7 |

Observation: This Table can be used to compare with other IBE input in the meeting.

## 2.4 MPR and boosting option proposal

Although our evaluation shows that many allocations can benefit from boosted power, there are multiple reasons why this cannot be a mandated performance for all UEs and rather be an optional capability:

- For up to 2dB boosting, the PA need to be pushed in hard compression (>4dB compression)

- This requires enough drive from the transceiver (although less than for PC1 operation)

- The output power needs to be properly controlled for the different allocations to meet the SEM, which is more difficult when the PA is saturated (although this is similar for the GSM PA power control)

- With up to 2dB boosting, IBE and EVM may depend on specific waveform design (CFR, spectrum shaping) that is common but cannot be mandated.

Separately, without boosting, there is much less difference between the 1-tone waveform due to 3.75kHz or 15kHz SCS, tone position, and Pi/2 BPSK of Pi/4 QPSK resulting in a simplified MPR table.

Proposal on MPR table: The following table is used for PC2 NB-IoT-NTN baseline requirement which requires MPR only for 12-tone and the four edge position 3.75kHz SCS 1-tone allocations

Table 3: Proposed baseline MPR for PC2 NB-IoT-NTN

| Waveform SCS and number of tones | Tone position and MPR (dB) |  |
| --- | --- | --- |
| SC# 15kHz/3.75kHz | 0-11/0-471 |  |
| 1 tone Pi/2 BPSK or Pi/4 QPSK | 0 |  |
| SC# 15kHz | 0-2 and 9-11 | 3-5 and 6-8 |
| 3 tone QPSK | 0 | 0 |
| SC# 15kHz | 0-5 and 6-11 |  |
| 6 tone QPSK | 0 |  |
| SC# 15kHz | 0-11 |  |
| 12 tone QPSK | 2 |  |
| Note 1: except for 0-3 and 44-47 3.75kHz tone positions where MPR is 0.5dB |  |  |

As there is a significant opportunity to boost many allocation types which is critical for NTN coverage, we also propose to introduce an optional boosting capability with an associated table. As boosting may also be valid for PC3 or for TN, the NB-IoT boosting capability should not be tied to a specific power class or NTN. To avoid too much complexity for this optional boosting, we only used 1dB power boosting steps.

Proposal on boosting:

- A NB-IoT boosting UE capability is introduced in Release 19 that should be valid regardless of the power class or NTN or TN operation.

- Power boosting is introduced in PCmax requirement for NB-IoT in TN and NTN specifications

- The following table is used for PC2 NB-IoT-NTN optional boosting requirement.

Table 4: Proposed optional power boosting for PC2 NB-IoT-NTN

| Waveform SCS and number of tones | Tone position and power boosting (dB) |  |
| --- | --- | --- |
| SC# 15kHz/3.75kHz | 1-2 and 9-10/4-7 and 40-43 | 3-8/8-39 |
| 1 tone Pi/2 BPSK or Pi/4 QPSK | 1 | 2 |
| SC# 15kHz | 3-5 and 6-8 |  |
| 3 tone QPSK | 1 |  |

# Conclusion

In this contribution, we evaluated the emissions for the missing allocations in our earlier measurements in [1] and make the following proposals.

Proposal on MPR table: The following table is used for PC2 NB-IoT-NTN baseline requirement which requires MPR only for 12-tone and the four edge position 3.75kHz SCS 1-tone allocations

Table 5: Proposed baseline MPR for PC2 NB-IoT-NTN

| Waveform SCS and number of tones | Tone position and MPR (dB) |  |
| --- | --- | --- |
| SC# 15kHz/3.75kHz | 0-11/0-471 |  |
| 1 tone Pi/2 BPSK or Pi/4 QPSK | 0 |  |
| SC# 15kHz | 0-2 and 9-11 | 3-5 and 6-8 |
| 3 tone QPSK | 0 | 0 |
| SC# 15kHz | 0-5 and 6-11 |  |
| 6 tone QPSK | 0 |  |
| SC# 15kHz | 0-11 |  |
| 12 tone QPSK | 2 |  |
| Note 1: except for 0-3 and 44-47 3.75kHz tone positions where MPR is 0.5dB |  |  |

Proposal on boosting:

- A NB-IoT boosting UE capability is introduced in Release 19 that should be valid regardless of the power class or NTN or TN operation.

- Power boosting is introduced in PCmax requirement for NB-IoT in TN and NTN specifications

- The following table is used for PC2 NB-IoT-NTN optional boosting requirement.

Table 6: Proposed optional power boosting for PC2 NB-IoT-NTN

| Waveform SCS and number of tones | Tone position and power boosting (dB) |  |  |
| --- | --- | --- | --- |
| SC# 15kHz/3.75kHz | 1-2/4-7 | 3-8/8-39 | 9-10/40-43 |
| 1 tone Pi/2 BPSK or Pi/4 QPSK | 1 | 2 | 1 |
| SC# 15kHz | 3-5 and 6-8 |  |  |
| 3 tone QPSK | 1 |  |  |

# References

[1] R4-2505834 NR and GSM PAs measurements for NB-IoT-NTN HPUE, Skyworks Solutions Inc., RAN4#115

[2] R4-2508122 WF on NR_IoT_NTN_HPUE_part2, Vivo, RAN4#115

# Annex 1: Raw measurement data

Table A1 provides the RAW measured data for the backoff needed to meet the NB-IoT SEM mask, which is the limiting factor for all the measured single-tone and multi-tone allocations, modulations and SCS, and the associated PA compression level. Data is available for 28 and 34dB image rejection.

Table A 1: Backoff and compression level at SEM limit for 28 and 34dB image rejection vs waveforms

| Waveform |  |  |  | 28dB image |  | 34dB image |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mod | SCS | #Tone | Tone# | comp | Backoff | comp | Backoff |
| QPSK | 15kHz | 3 | 0 | 2.5 | -0.6 | 2.1 | -0.6 |
|  |  | 3 | 3 | 2.6 | -1 | 2.7 | -1 |
|  |  | 6 | 0 | 2.1 | 0 | 2.1 | 0 |
|  |  | 12 | 0 | 0.8 | 1.7 | 0.6 | 1.8 |
| Pi/2 BPSK |  | 1 | 0 | 2.3 | -0.4 | 2.4 | -0.5 |
|  |  | 1 | 1 | 2 | -1.5 | 1.8 | -1.5 |
|  |  | 1 | 2 | 2.6 | -1.6 | 3.1 | -1.8 |
|  |  | 1 | 3 | 3.6 | -1.9 | 3.8 | -2 |
|  |  | 1 | 4 | 3.3 | -2 | 4.2 | -2 |
|  |  | 1 | 5 | 4.2 | -2.1 | 4.4 | -2.1 |
| Pi/4QPSK |  | 1 | 0 | 2.4 | -1.1 | 2.6 | -1 |
|  |  | 1 | 1 | 2 | -2 | 2.5 | -2.1 |
|  |  | 1 | 2 | 2.7 | -2.1 | 3.3 | -1.9 |
|  |  | 1 | 3 | 3.2 | -2 | 3.4 | -2 |
|  |  | 1 | 4 | 3.4 | -1.8 | 3.2 | -2 |
|  |  | 1 | 5 | 2.9 | -1.9 | 3 | -1.9 |
| Pi/2 BPSK | 3.75kHz | 1 | 0 | 2 | 0.5 | 3 | 0.1 |
|  |  | 1 | 4 | 2.1 | -1.6 | 2.7 | -2 |
|  |  | 1 | 8 | 4.2 | -2 | 4.2 | -2 |
|  |  | 1 | 12 | 4.5 | -2.1 | 4.6 | -2.4 |
|  |  | 1 | 16 | 3.7 | -1.9 | 5 | -2 |
|  |  | 1 | 20 | >5 | -2.3 | >6 | -2.3 |
|  |  | 1 | 23 | >6 | -2.3 | >6 | -2.4 |
| Pi/4QPSK |  | 1 | 0 | 1.5 | 0.3 | 4 | -0.2 |
|  |  | 1 | 4 | 3 | -2 | 2.4 | -2 |
|  |  | 1 | 8 | 3.8 | -2 | 4.4 | -2 |
|  |  | 1 | 12 | 4.2 | -2.2 | 4.3 | -2.2 |
|  |  | 1 | 16 | 4.8 | -2.2 | 4.8 | -2.1 |
|  |  | 1 | 20 | 5 | -2.2 | 4.8 | -2.1 |
|  |  | 1 | 23 | 5.2 | -2.1 | 4.7 | -2.1 |