3GPP TSG RAN WG4 Meeting #116		R4-2509246

Bengaluru, India, August 25th – 29th , 2025

Title: 	Discussion on LB-LB CA via switching

Agenda Item:	7.3.3

Source: 	CATT

Document for:	Approval

1 Introduction

In this May meeting, RAN1 has concluded this Work Item with the agreed CR [1] and send all the conclusions to RAN4 and RAN2 via the LS [2]. In the RAN4#115 meeting, some outcomes including Switching time mask, switching periods and switching period location have been captured into the approved TP [3]. However, according to the WF [4], there are some open issues to be solved in the RAN4#116 meeting. In this paper, we’d like to discuss these open issues.

2 Discussion

2.1 The applicable band combinations

Referring to the latest specification [5] in the clause 5.2A.2, the exemplary low-low band combination CA_n5-n29 has been captured into the specification as well as the higher order combinations CA_n5-n25-n29/CA_n5-n29-n66/ CA_n5-n29-n77/ CA_n5-n25-n29-n66 which include CA_n5-n29. Based on the current specification, it can be understood that UE could support the con-current operation in the UL band n5 and both DL band n5 and n29 for these band combination.

Observation 1: current specification has captured the exemplary low-low band combination CA_n5-n29 as well as the higher order combinations CA_n5-n25-n29/CA_n5-n29-n66/ CA_n5-n29-n77/ CA_n5-n25-n29-n66 which include band n5 and n29.

Observation 2: in current specification, it’s allowed for UE to support the con-current operation in the UL band n5 and both DL band n5 and n29 for these band combinations (CA_n5-n29/ CA_n5-n25-n29/ CA_n5-n29-n66/ CA_n5-n29-n77/ CA_n5-n25-n29-n66).

As RAN4 is approaching to conclude this low-low bands CA via switching WI, RAN4 need to discuss how to explicitly indicate which band combinations can support the new feature, i.e. low-low bands CA via switching. There are two options to be further discussed.

Option 1: Creating a new table/clause to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

For example: clause 5.2A.3 Inter-band CA via switching

Option 2: Creating a NOTE in the clause 5.2A.2.1 to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

For example: NOTE X: it’s allowed for UE to support this low-low bands CA combination via switching by indicate the UE capability [switchingPeriodForFDD-SDL].

Proposal 1: RAN4 can discuss the following two options and decide how to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

Option 1: Creating a new table/clause to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

For example: clause 5.2A.3 Operating bands for low-low bands CA via switching

Option 2: Creating a NOTE in the clause 5.2A.2.1 to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

For example: NOTE X: it’s allowed for UE to support this low-low bands CA combination via switching by indicate the UE capability [switchingPeriodForFDD-SDL].

In this WID [6], only low-low bands CA via switching are considered. Specifically, Sub-1GHz FDD band + SDL band combinations were considered in this Work Item. Thus, the higher order band combinations including the Sub-1GHz FDD + SDL parts should not be considered in this Work Item. In addition, the reason why operators would like to introduce this feature is to improve the network traffic volumes in both urban and rural scenarios in which Sub-1GHz FDD+SDL bands need to provide the enough traffic volumes according to the Motivation [7]. However, for the scenarios where 2GHz bands and C bands can provide the traffic volumes, it seems that the necessities for improving the network traffic volumes are less urgent than the scenarios where only Sub-1GHz FDD+SDL bands can provide the traffic volumes.

Observation 3: In this Work Item [6], only two low-low bands CA via switching was considered, and the higher order band combinations including the Sub-1GHz FDD + SDL parts are out of the scope of this Work Item.

Based on the discussion in the previous RAN4 meetings, it seems that the only RF requirements Time mask for low-low bands CA via switching will be specified generally. It means that no band combination specific requirements will be specified. Thus, it’s a low hanging fruit for RAN4 to extend this feature to the other FDD+SDL low-low bands CA combinations. As CA_n29-n71 has been captured in the specification with specific BCS and delta T and R requirements, it’s proposed to capture both CA_n29-n71 and CA_n5-n29 to support this new feature, i.e. low-low bands CA via switching.

Proposal 2: it’s a low hanging fruit for RAN4 to extend this feature to the other FDD+SDL low-low bands CA combinations in Rel-19, at least for CA_n29-n71 which has been captured in the specification.

2.2 The minimum requirements of switching gaps

According to the approved TP [3], RAN4 reached some consensus on the actual switching gap length for both switching directions as shown below. When we put all the parameters into the following two formulas, we can get the actual switching gap length in the OFDM symbol unit per RAN1’s agreement in the feature list [8].

FDD to SDL switch: actual switching gap length > Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period. SDL to FDD switch: Actual switching gap length > Tswitch + TTA + MRTD + SDL trailing transient period + FDD UL leading transient period

However, based on our analysis, there are some points to be improved for these two formula.

- Whether to include SDL leading transient period:

Generally, the transient period is specified for transmitter in RF spec. RAN4 never specify the transient period requirements for the receiver. As SDL band only has a receiver without a transmitter, it’s not necessary to consider SDL transient period when calculating the switching gap.

Observation 4: it’s not necessary to consider SDL transient period when calculating the switching gap.

- The adjustment about the FDD UL trailing end transient period for FDD to SDL switching scenario:

Based on the current specification, NTA, offset is configured as 25600 (13µs) by default. That means TTA is larger than the FDD UL trailing end transient period (10µs). Thus, there is no need to consider FDD UL trailing end transient period for this case. The formula should be updated to reflect this case.

Observation 5: for FDD to SDL switching scenario, whether to include FDD UL trailing end transient period (10µs) depends on whether TTA is less than the transient period (10µs).

- The adjustment about receive timing difference:

MRTD is captured in the TR 38.768 based on the approved TP in last meeting. However, from UE minimum requirements’ perspective, the receive timing difference among the closest DL slot timing boundaries (between PCell and SCell) or (between SCell and PCell) could be positive value or negative value, i.e. the value of receive timing difference could be any value between negative MRTD and Positive MRTD.

Observation 6: from UE minimum requirements’ perspective, the receive timing difference among the closest DL slot timing boundaries (between PCell and SCell) or (between SCell and PCell) could be any value between negative MRTD and Positive MRTD.

Proposal 3: according to observation 4, 5 and6, it’s proposed to specify the minimum requirements for switching gap from UE perspective below.

For Switch-from FDD PCell to Switch-to SDL SCell,

Minimum Switching GapF2D=Switching Period+∆T1+∆TRTD,  P2S114*103

For Switch-from SDL SCell to Switch-to FDD PCell,

Minimum Switching GapD2F=Switching Period+∆T2+∆TRTD,  S2P114*103

Where:

Switching Period (µs) is a UE capability indicated by switchingPeriodForFDD-SDL.

∆T1(µs) represents the impacts of switch-from UL carrier:

if Transient period-TTA*106>0,

then ∆T1(μs)=Transient period-TTA*106

if Transient period-TTA*106≤0,

then ∆T1(μs)=0

∆T2(µs) represents the impacts of switch-to UL carrier:

∆T2=transient period+TTA*106

Transient period is 10µs specified in the clause 6.3.3.2 of TS 38.101-1.

TTA(s) is the Timing Advance of FDD UL carrier, specified in the clause 4.3.1 of TS 38.211.

∆TRTD,  P2S(µs) represents a relative receive timing difference among the closest DL slot timing boundaries between PCell and SCell:

∆TRTD,  P2S=receive timingPCell-receive timingSCell

∆TRTD,  S2P(µs) represents a relative receive timing difference among the closest DL slot timing boundaries between SCell and PCell:

∆TRTD,  S2P=receive timingSCell-receive timingPCell

114*103(µs) is the duration of one OFDM symbol with 15kHz SCS of blanked carrier.

x is the smallest integer greater than or equal to x.

The Diagram of switching time mask including Switching Period, Switching Gap, Transient Period and other parameters is shown below.

Figure 1 The diagram of switching time mask for inter-band CA via switching

The band A represent PCell FDD band and the band B represent the SCell SDL band.

2.3 The maximum value of switching gaps

According to the latest RAN1 feature list [11], the maximum values for LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 and LBCA-SwitchinggGap-SCelltoPCell-r19 are still FFS.

Based on our evaluation about switching gap, maximum value of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 should be equal to or larger than three OFDM symbols with 15kHz SCS, no matter what the TTA is .

For LBCA-SwitchinggGap-SCelltoPCell-r19, the maximum value should be equal to or larger than the following values based on the different maximum cell coverage to be supported.

| Maximum Cell Coverage (km) | The maximum value of LBCA-SwitchinggGap-SCelltoPCell-r19 |
| --- | --- |
| 100km | 12 OFDM symbols with 15kHz SCS |
| 21km | 5 OFDM symbols with 15kHz SCS |
| 14km | 4 OFDM symbols with 15kHz SCS |
| 10km | 4 OFDM symbols with 15kHz SCS |
| 0.5km~5km | 3 OFDM symbols with 15kHz SCS |

Proposal 4: From RAN4’s perspective, it’s recommended for RAN1/RAN2 to specify the maximum number of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 and LBCA-SwitchinggGap-SCelltoPCell-r19 considering the following scenarios based on our calculation about switching gap.

Proposal 4-1: maximum value of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 should be equal to or larger than three OFDM symbols with 15kHz SCS, no matter what the TTA is.

Proposal 4-2: For LBCA-SwitchinggGap-SCelltoPCell-r19, the maximum value should be equal to or larger than the following values based on the different maximum cell coverage to be supported.

| Maximum Cell Coverage (km) | The maximum value of LBCA-SwitchinggGap-SCelltoPCell-r19 |
| --- | --- |
| 100km | 12 OFDM symbols with 15kHz SCS |
| 21km | 5 OFDM symbols with 15kHz SCS |
| 14km | 4 OFDM symbols with 15kHz SCS |
| 10km | 4 OFDM symbols with 15kHz SCS |
| 0.5km~5km | 3 OFDM symbols with 15kHz SCS |

2.4 Test methodology for the DL time mask/REFSENS requirements

As a new switching pattern between the PCell and the SCell will be indicated by network IE LBCA-SwitchingPattern referring to clause 24 of TS 38.213, current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for Low-Low bands inter-band CA via switching. That means current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for this case.

The clause 7.3A.2.3 of TS 38.101-1: 7.3A.2.3 Reference sensitivity power level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6. For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2. The reference sensitivity exceptions due to harmonic, harmonic mixing, cross band isolation and power class 2 or power class 3 CA intermodulation interferences are applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO or Tx diversity operation.

Observation 7: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for Low-Low bands inter-band CA via switching.

Apart from REFSENS requirements, it’s very important to verify the DL receiving performance for UE before and/or after the Switching gap. Alternatively, RAN4 can design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Observation 8: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

For the LBCA-SwitchingPattern, it’s proposed to design the DL RMC as ‘00011 00110 01100 11001 11100 11001 10011 00110’. Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots. In addition, one symbol PDCCH is scheduled at the first symbol in each DL slot except for SSB slots. The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown below.

Figure 2 The diagram about DL RMC for Low-Low bands inter-band CA via switching

Proposal 5: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

5-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

5-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

5-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

5-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 2.

3 Summary

3.1 The applicable band combinations

Observation 1: current specification has captured the exemplary low-low band combination CA_n5-n29 as well as the higher order combinations CA_n5-n25-n29/CA_n5-n29-n66/ CA_n5-n29-n77/ CA_n5-n25-n29-n66 which include band n5 and n29.

Observation 2: in current specification, it’s allowed for UE to support the con-current operation in the UL band n5 and both DL band n5 and n29 for these band combinations (CA_n5-n29/ CA_n5-n25-n29/ CA_n5-n29-n66/ CA_n5-n29-n77/ CA_n5-n25-n29-n66).

Proposal 1: RAN4 can discuss the following two options and decide how to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

Option 1: Creating a new table/clause to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

For example: clause 5.2A.3 Operating bands for low-low bands CA via switching

Option 2: Creating a NOTE in the clause 5.2A.2.1 to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching.

For example: NOTE X: it’s allowed for UE to support this low-low bands CA combination via switching by indicate the UE capability [switchingPeriodForFDD-SDL].

Observation 3: In this Work Item [6], only two low-low bands CA via switching was considered, and the higher order band combinations including the Sub-1GHz FDD + SDL parts are out of the scope of this Work Item.

Proposal 2: it’s a low hanging fruit for RAN4 to extend this feature to the other FDD+SDL low-low bands CA combinations in Rel-19, at least for CA_n29-n71 which has been captured in the specification.

3.2 The minimum requirements of switching gaps

- Whether to include SDL leading transient period:

Observation 4: it’s not necessary to consider SDL transient period when calculating the switching gap.

- The adjustment about the FDD UL trailing end transient period for FDD to SDL switching scenario:

Observation 5: for FDD to SDL switching scenario, whether to include FDD UL trailing end transient period (10µs) depends on whether TTA is less than the transient period (10µs).

- The adjustment about receive timing difference:

Observation 6: from UE minimum requirements’ perspective, the receive timing difference among the closest DL slot timing boundaries (between PCell and SCell) or (between SCell and PCell) could be any value between negative MRTD and Positive MRTD.

Proposal 3: according to observation 4, 5 and 6, it’s proposed to specify the minimum requirements for switching gap from UE perspective below.

For Switch-from FDD PCell to Switch-to SDL SCell,

Minimum Switching GapF2D=Switching Period+∆T1+∆TRTD,  P2S114*103

For Switch-from SDL SCell to Switch-to FDD PCell,

Minimum Switching GapD2F=Switching Period+∆T2+∆TRTD,  S2P114*103

Where:

Switching Period (µs) is a UE capability indicated by switchingPeriodForFDD-SDL.

∆T1(µs) represents the impacts of switch-from UL carrier:

if Transient period-TTA*106>0,

then ∆T1(μs)=Transient period-TTA*106

if Transient period-TTA*106≤0,

then ∆T1(μs)=0

∆T2(µs) represents the impacts of switch-to UL carrier:

∆T2=transient period+TTA*106

Transient period is 10µs specified in the clause 6.3.3.2 of TS 38.101-1.

TTA(s) is the Timing Advance of FDD UL carrier, specified in the clause 4.3.1 of TS 38.211.

∆TRTD,  P2S(µs) represents a relative receive timing difference among the closest DL slot timing boundaries between PCell and SCell:

∆TRTD,  P2S=receive timingPCell-receive timingSCell

∆TRTD,  S2P(µs) represents a relative receive timing difference among the closest DL slot timing boundaries between SCell and PCell:

∆TRTD,  S2P=receive timingSCell-receive timingPCell

114*103(µs) is the duration of one OFDM symbol with 15kHz SCS of blanked carrier.

x is the smallest integer greater than or equal to x.

The Diagram of switching time mask including Switching Period, Switching Gap, Transient Period and other parameters is shown below.

Figure 1 The diagram of switching time mask for inter-band CA via switching

3.3 The maximum value of switching gaps

Proposal 4: From RAN4’s perspective, it’s recommended for RAN1/RAN2 to specify the maximum number of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 and LBCA-SwitchinggGap-SCelltoPCell-r19 considering the following scenarios based on our calculation about switching gap.

Proposal 4-1: maximum value of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 should be equal to or larger than three OFDM symbols with 15kHz SCS, no matter what the TTA is.

Proposal 4-2: For LBCA-SwitchinggGap-SCelltoPCell-r19, the maximum value should be equal to or larger than the following values based on the different maximum cell coverage to be supported.

| Maximum Cell Coverage (km) | the maximum value of LBCA-SwitchinggGap-SCelltoPCell-r19 |
| --- | --- |
| 100km | 12 OFDM symbols with 15kHz SCS |
| 21km | 5 OFDM symbols with 15kHz SCS |
| 14km | 4 OFDM symbols with 15kHz SCS |
| 10km | 4 OFDM symbols with 15kHz SCS |
| 0.5km~5km | 3 OFDM symbols with 15kHz SCS |

3.4 Test methodology for the DL time mask/REFSENS requirements

Observation 7: current REFSENS requirements for inter-band CA specified in the clause 7.3A.2.3 of TS 38.101-1 can’t be reused for Low-Low bands inter-band CA via switching, since current DL FDD RMC specified in clause A.3.3 of TS 38.101-1 are not applicable for Low-Low bands inter-band CA via switching.

Observation 8: for Low-Low bands inter-band CA via switching, it’s possible to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

Proposal 5: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together.

5-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’.

5-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots.

5-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots.

5-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 2.

Reference

[1] R1- 2505061, “Introduction of low NR band carrier aggregation via switching for NR”, Samsung

[2] R1-2504869, “LS on Low NR band carrier aggregation via switching”, RAN1

[3] R4-2507932, “TP for 38.768: time masks for DL-only CA via switching”, Ericsson, Xiaomi, MediaTek, Apple

[4] R4-2508118, “WF on UE RF requirements for LB CA via switching”, Apple

[5] TS 38.101-1, v19.1.0

[6] RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS

[7] RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, HiSilicon, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Samsung, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE

[8] R1-2503242, “DRAFT LS on Rel-19 higher layers parameters list Post RAN1#121”, RAN1

[9] R4-2508259, “WF on RRM requirements for NR_LBCA_Sw”, Apple

[10] RP‑251762, “TR 38.768 v1.0.0 on Low NR band carrier aggregation via switching”, Apple

[11] R1-2503243, “Consolidated Rel-19 higher layers parameters list Post RAN1#121”, RAN1