3GPP TSG RAN WG4 Meeting #116		R4-2509248

Bengaluru, India, August 25th – 29th , 2025

Title: 	TP for TR 38.768 to modify the general description

Agenda Item:	7.3.2

Source: 	CATT

Document for:	Approval

1 Introduction

After RAN4#115, TR 38.768 has further been updated to v1.0.0 by capturing TPs approved in this meeting [1]. However, the general description in clause is not aligned with the latest WID [2] and RAN1 agreed CR [3].

Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 WID RP-251315.

24 Downlink carrier aggregation via switching A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots. CR R1- 2505061

In both WID [2] and RAN1 agreed CR [3], only SDL carrier is considered in an Scell, but the general description extend the scope to “DL Scell configured in a DL band of another paired band”, which is not aligned with the scope of WID and RAN1 agreed CR. Thus, it’s proposed to modify the general description in the clause 5.2.1 of TR 38.768 to reflect the accurate scenario where we studied in this WID [2]. In addition, 70 𝜇s UE capability was agreed in the last RAN4 meeting according to the way forward [4]. It’s proposed to reflect the latest agreement in the TR 38.768.

2 Text Proposal

--- < Start of change> ---

5.2.1	General

For what follows we consider switching between an FDD and SDL band for the CA-n5A-29A configuration in Table 4-1 but without loss of generality: one UL is configured in a paired band with the DL Scell configured in in an SDL band and both bands belong to Sub-1GHz bands.

The switching period, defined in units of 𝜇s according to UE capability, is located within a switching gap located within the switched-from carrier. Switching gaps, expressed in units of symbols, are accommodated by the network according to the physical layer design of the feature.

The timing reference for the switching pattern and switching gap at the UE is the DL timing of the Pcell (in the FDD band). The uplink radio frame transmission on the Pcell takes place TTA = (NTA + TAoffset)Tc before the reception of the first detected path (as defined in TS 38.133) in time on the corresponding DL radio frame. This means that the timing advance must be considered at switching events.

The duration of the switching gap is RRC configured. The network ensures that the switching gap is large enough to cover the switching period (35, 70 or 140 𝜇s according to the UE capability), transient periods and the TA, if needed, in both switch directions.

--- < End of change> ---

Reference

[1] RP‑251762, “TR 38.768 v1.0.0 on Low NR band carrier aggregation via switching”, Apple

[2] RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS

[3] R1- 2505061, “Introduction of low NR band carrier aggregation via switching for NR”, Samsung

[4] R4-2508118, “WF on UE RF requirements for LB CA via switching”, Apple