3GPP TSG RAN WG4 Meeting #116		R4-2509255

Bengaluru, India, August 25th – 29th , 2025

Title: 	Discussion on PC1.5 intra-band UL NC CA

Agenda Item:	7.1.2.1.2

Source: 	CATT

Document for:	Approval

1 Background

In the RAN4#114 meeting, RAN4 reached a consensus on the perCC Pcmax, c for PC1.5 intra-band NC CA with dual PA [1]. However, RAN4 didn’t reach any agreements about how to reflect this way forward in the spec referring to the draft CRs [2][3].

1.5 PCMAX,c and PCMAX for intra-band NCCA (with dual PA) Agreement: - PCMAX,C is upper bounded by 26dBm - Detailed spec change is discussed based on dCR provided in future meeting, CR leader is encouraged to take companies’ input into account - For PCMAX, no change of current spec

In this paper, we’d like to discuss this issue and provide a future-proof proposal.

2 Discussion

In the Release 17, RAN4 introduced a per band per BC power class capability ‘ue-PowerClassPerBandPerBC’ to distinguish the different PA capabilities in different UL bands of UL inter-band CA. For example, UL CA_n28A-n41A could support PC2 or higher power limit, but PC3 is supported in band n28A and PC2 is supported in band n41A.

From RAN2 IE design’s perspective or framework of UE BC capabilities reporting, there is no big difference between intra-band NC CA and inter-band CA.

For example, UE has to report twice FeatureSetUplink /FeatureSetUplink-v1710 for band n78A in order to represent UL CA_n78(2A). It’s similar to the UL CA_n28A-n41A that UE has to report one FeatureSetUplink /FeatureSetUplink-v1710 for band n28A and another FeatureSetUplink /FeatureSetUplink-v1710 for band n41A.

|  | ue-PowerClassPerBandPerBC | ue-PowerClassPerBandPerBC | powerClass / powerClass-v1610 |
| --- | --- | --- | --- |
| UL CA_n28A-n41A | 23dBm for band n28A | 26dBm for band n41A | PC2 |
| UL CA_n78(2A) | 26dBm for band n78A | 26dBm for band n78A | PC1.5 |

From future-proof perspective, a clear UE capability indication is very important to consider the forward compatibility in case UE could support 29dBm PC1.5 per CC to achieve the PC1.5 intra-band NC CA in the future.

Thus, it’s proposed to reuse the legacy R17 UE capability ‘ue-PowerClassPerBandPerBC-r17’’ to indicate the 26dBm upper boundary per CC for PC1.5 intra-band NC CA with dual-PA architecture instead of some hard code solutions in the RAN4 RF spec.

Observation 1: From RAN2 IE design’s perspective or framework of UE BC capabilities reporting, there is no big difference between intra-band NC CA and inter-band CA.

Proposal 1: To reuse the legacy R17 UE capability ‘ue-PowerClassPerBandPerBC-r17’’ to indicate the 26dBm upper boundary per CC for PC1.5 intra-band NC CA with dual-PA architecture.

In the current specification, the PpowerClass specified in the clause 6.2.4 of TS 38.101-1 is indicated by per band UE power class capability ue-PowerClass / ue-PowerClass-v1610 / ue-PowerClass-v1700. As current logic for UE power class reporting of one band combination is different from the R15 design, it’s necessary to clarify the definition of PPowerClass under one UL intra-band CA combination.

6.2.4 Configured transmitted power The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost} where PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7]; PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation, without taking into account the tolerance specified in the Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation;

6.2A.4.1.2 Configured transmitted power for Intra-band non-contiguous CA For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX. The configured maximum output power PCMAX,c on serving cell c shall be set as specified in subclause 6.2.4. The configured maximum output power PCMAX,c on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

Proposal 2: The following spec changes in the TS 38.101-1 are proposed to reflect the agreement that “PCMAX,C is upper bounded by 26dBm” for PC1.5 intra-band NC CA with dual PA.

>> Start of Changes <<

6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in clause 6.2.1 apply for power class 3 and other power classes if indicated in clause 5.5A.2. For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 6.2A.1.2-1.

Table 6.2A.1.2-1: UE Power Class for intra-band non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5(dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40(2A) |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41(2A) |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n77(2A) |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n78(2A) |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n79(2A) |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: An uplink CA configuration in which the band has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For intra-band non-contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power class 3 is the default power class unless otherwise stated. NOTE 5: Achieved by UE indicating dualPA-Architecture and ‘PC2’ is indicated by ue-PowerClassPerBandPerBC-r17 in this release. |  |  |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-2 and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.2 is 23dBm or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.2;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.2.

>> Next of Changes <<

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in sub-clause 6.2.4, but with the following definitions for some parameters.

MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively.

PPowerClass is the maximum UE power specified in Table 6.2.1-1 reported by ue-PowerClassPerBandPerBC-r17, without taking into account the tolerance. If ue-PowerClassPerBandPerBC-r17 is absent, PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation reported by the smaller one between ue-PowerClass / ue-PowerClass-v1610 and powerClass / powerClass-v1610 without taking into account the tolerance;

For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

>> End of Changes <<

3 Summary

Observation 1: From RAN2 IE design’s perspective or framework of UE BC capabilities reporting, there is no big difference between intra-band NC CA and inter-band CA.

Proposal 1: To reuse the legacy R17 UE capability ‘ue-PowerClassPerBandPerBC-r17’’ to indicate the 26dBm upper boundary per CC for PC1.5 intra-band NC CA with dual-PA architecture.

Proposal 2: The following spec changes in the TS 38.101-1 are proposed to reflect the agreement that “PCMAX,C is upper bounded by 26dBm” for PC1.5 intra-band NC CA with dual PA.

>> Start of Changes <<

6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in clause 6.2.1 apply for power class 3 and other power classes if indicated in clause 5.5A.2. For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 6.2A.1.2-1.

Table 6.2A.1.2-1: UE Power Class for intra-band non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5(dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40(2A) |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41(2A) |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n77(2A) |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n78(2A) |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n79(2A) |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: An uplink CA configuration in which the band has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of the band are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 3: For intra-band non-contiguous carrier aggregation the maximum power requirement shall apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power class 3 is the default power class unless otherwise stated. NOTE 5: Achieved by UE indicating dualPA-Architecture and ‘PC2’ is indicated by ue-PowerClassPerBandPerBC-r17 in this release. |  |  |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-2 and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.2 is 23dBm or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.2;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.2.

>> Next of Changes <<

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in sub-clause 6.2.4, but with the following definitions for some parameters.

MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively.

PPowerClass is the maximum UE power specified in Table 6.2.1-1 reported by ue-PowerClassPerBandPerBC-r17, without taking into account the tolerance. If ue-PowerClassPerBandPerBC-r17 is absent, PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation reported by the smaller one between ue-PowerClass / ue-PowerClass-v1610 and powerClass / powerClass-v1610 without taking into account the tolerance;

For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

>> End of Changes <<

Reference

[1] R4-2502863, WF on TN HPUE, Samsung, Skyworks, OPPO, LGE, Apple, RAN4#114

[2] R4-2505614, [NR_ENDC_RF_Ph4-Core] DraftCR to 38101-1-j10 on PC1.5 NC ULCA MPR A-MPR and PCmax, Skyworks Solutions Inc.

[3] R4-2505758, Introduction of PC1.5 intra-band UL NC CA, Xiaomi