3GPP TSG-RAN WG4 Meeting #116                                                                R4-2509259

Bengaluru, India, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on 6RX SRS test

Agenda item:	7.1.2.3.3

Document for:	Approval

- Introduction

In RAN4 #115 meeting, a WF [1] on 6Rx and SRS IL imbalance was agreed. However, there are still some remaining issues need to be discussed. In this contribution, we focus on the SRS test issue, share some views and make proposals from some aspects.

- Discussion

The agreement of SRS IL imbalance issue have been made and shown as below:

## <Sub-topic 4-1> Solutions for SRS IL imbalance issue Agreement:(online agreement) - Do not introduce any solution for SRS-IL imbalance issue for UE in Rel-19 - It is FFS on if and how to introduce [R17] RMC and test for SRS. The related discussion should be contribution driven and in the agenda item of R17 maintenance.

2.1 ∆TRxSRS test under MOP limited case

∆TRxSRS was introduced in PCMAX equation from Rel-15 specification with specific requirement, but no corresponding UL RMC is defined in RAN4 specification as well as test method is defined in RAN5 specification. Currently, PCMAX is tested only for PUSCH measurement channel so the items such as MPR in PCMAX equation can be tested except ∆TRxSRS. However, in RAN1 SRS power control, PCMAX which initially not defined for SRS (e.g. MPR) is used.

As the number of switching antennas increase, ∆TRxSRS also increases and the IL imbalance impact on system performance is unneglectable. Although not all requirements have corresponding test, we should still try our best to define the test of ∆TRxSRS unless it is verified that the test is not possible under current circumstance.

------------------------------------------------------TS 38.101-1------------------------------------------------------------------------

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

----------------------Next section---------------

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

------------------------------------------------------TS 38.101-1------------------------------------------------------------------------

Therefore, RAN4 should work on UL RMC on ∆TRxSRS under MOP limited case and at the same time involve RAN5 to request them to work on the test procedure.

Proposal 1: RAN4 should try best to introduce ∆TRxSRS test under MOP limited case. RAN4 can work on UL RMC and at the same time involve RAN5 to request them to work on the test procedure.

2.2 SRS IL imbalance test under MOP non-limited case

UE is required to perform SRS IL imbalance compensation, this kind of compensation can only be performed under MOP non-limited case and is expected to have better imbalance result compared to ∆TRxSRS requirement which only directly reflects the hardware defection. However, there is no such kind of requirement now in the specification and reuse ∆TRxSRS requirement as compensation result is obviously non-reasonable. Therefore, we think RAN4 is not ready to define such kind of test in current stage because of lack of requirements.

Proposal 2: It is proposed not to introduce any test for UE compensation of SRS IL imbalance, i.e. no SRS IL imbalance test under MOP non-limited case in current stage.

2.3 UL RMC

In RAN4, uplink transmission measurement channels are only defined for PUSCH and there is no SRS related measurement channel. In [2], reference measurement channels of SRS transmission power test are proposed, but we think there are still some issues to be discussed and verified before any CR. For example, although SRS is a sequence, after modulated to DFT-s-OFDM waveform, there is still some PAPR which needs to be further verified whether MPR=0 in nearly full RB allocation (current MPR was specified without consideration of SRS sequence), and also the usage of codebook is needed to be carefully discussed.

Proposal 3: UL RMC for SRS power testing need to be studied on some issues, e.g. MPR and codebook issues.

2.4 Other issues

UE vendors may concern about ∆TRxSRS test for earlier release because it is already in mass production stage. And considering our proposal 3, we think it still needs some time to specify the test both in RAN4 and RAN5, so earlier release may be a little difficult, but anyway the test can be performed from release 19. Of course, the test should be applied also to 2RX/4RX/6RX/8RX because they all have requirements in the specification.

Proposal 4: ∆TRxSRS test for MOP limited case can be applied to 2RX/4RX/6RX/8RX from release 19.

- Conclusion

This contribution provides our views on SRS test. The following proposals are concluded as follows:

Proposal 1: RAN4 should try best to introduce ∆TRxSRS test under MOP limited case. RAN4 can work on UL RMC and at the same time involve RAN5 to request them to work on the test procedure.

Proposal 2: It is proposed not to introduce any test for UE compensation of SRS IL imbalance, i.e. no SRS IL imbalance test under MOP non-limited case in current stage.

Proposal 3: UL RMC for SRS power testing need to be studied on some issues, e.g. MPR and codebook issues.

Proposal 4: ∆TRxSRS test for MOP limited case can be applied to 2RX/4RX/6RX/8RX from release 19.

- References

- R4-2508086, WF on 6Rx and SRS IL imbalance

- R4-2506186, Reference measurement channels for SRS transmissions