3GPP TSG-RAN WG4 Meeting #116	R4-2509276

# Bengaluru , IN, 25th – 29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test cases for satellite switching with re-synchronization for less than 5MHz with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases for satellite switching with re-synchronization for less than 5MHz with NTN needs to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the test cases for satellite switching with re-synchronization for less than 5MHz with NTN in clause A.14.2.2.X. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for satellite switching with re-synchronization for less than 5MHz with NTN would be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.14.2.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.14.2.2.X	[RACH-based] Hard Satellite switching with re-synchronization from FR1 to FR1 for less than 5MHz with NTN

A.14.2.2.X.1	Test Purpose and Environment

This test is to verify the requirement for [RACH-based] hard satellite switching with re-synchronization from SAN FR1 to SAN FR1 for unknown target cell operating with 12 PRB SSB bandwidth specified in clause 6.1C.3.

A.14.2.2.X.2	Test Parameters

Supported test configurations are shown in table A.14.2.2.X.2-1. General test parameters as specified in table A.14.2.2.X.2-2 apply except those specified in table A.14.2.2.4.2-2. Target Satellite configuration pattern specified in table A.14.2.2.4.2-3 shall apply. Cell specific test parameters as specified in table A.14.2.2.X.2-4 apply except those specified in table A.14.2.2.4.2-4.

The test procedure specified in clause A.14.2.2.4.2 applies to this test. The Cell 2 is the unknown target cell operating with 12 PRB SSB bandwidth.

Table A.14.2.2.X.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 3 MHz BW |

Table A.14.2.2.X.2-2: General test parameters for [RACH-based] Hard Satellite switching with re-synchronization from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
| Final condition | Active cell |  | Cell 2 | unknown target cell operating with 12 PRB SSB bandwidth |

Table A.14.2.2.X.2-3: Cell specific test parameters for [RACH-based] Hard Satellite switching with re-synchronization from FR1 to FR1 test case

| Parameter |  | Unit | Cell 1Note1 |  | Cell 2Note1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| BWchannel | Config 1,2 | MHz | 3: NPRB,c = 15 |  |  | 3: NPRB,c = 15 |
| BWP BW |  | MHz | 3: NPRB,c = 15 |  |  | 3: NPRB,c = 15 |
| PDSCH Reference measurement channel |  |  | SR.1.2 FDD |  |  | SR.1.2 FDD |
| CORESET Reference Channel |  |  | CR.1.3 FDD |  |  | CR.1.3 FDD |
| SSB Configuration |  |  | SSB.13 FR1 |  |  | SSB.13 FR1 |

A.14.2.2.X.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 52.5 ms from the beginning of time period T2.

The rate of correct satellite switch observed during repeated tests shall be at least 90 %.

NOTE:	The hard satellite switch with re-sync delay Dswitch_unchangedPCI can be expressed as: Tinterrupt, where:

Tinterrupt is defined in clause 6.1C.3.2.2.

Dswitch_unchangedPCI = Tinterrupt = Tsearch + Tprocessing  + T∆ + Tmargin ms

Here: Tsearch = Tfirst_SSB = 0.5ms; Tprocessing = 10ms; T∆ = 20ms; Tmargin = 2ms.

Besides, interruption uncertainty TIU = 20ms in acquiring the first PRACH transmission resource is needed.

This gives a total of 52.5 ms.

<End of Change 1>