3GPP TSG-RAN WG4 Meeting #116	R4-2509292

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on (e)RedCap RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.27.5.1

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_NTN_Ph3_Part1 was approved and many agreements were reached, but there still are a few issues left [1]. This document will discuss (e)RedCap RRM requirements for Rel-19 NTN phase 3 and give our understanding and proposals.

- Discussion

  - General consideration

- SMTC adjustment in idle/inactive

In the previous meeting, RAN4 had different understandings about the legacy capability of SMTC adjustment in idle/inactive. The Way Forward for SMTC adjustment in idle/inactive in the last meeting is copied as below [1]:

<Way Forward>: - Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN. - Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode. - Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

In R17 NTN, for SMTC adjustment in RRC_IDLE and RRC_INACTIVE, a CR of this capability was agreed in RAN2 last meeting and the feature was added in TS 38.306 [2], and the related content is copied as follows:

SMTC adjustment for RRC_IDLE/RRC_INACTIVE It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC adjustment based on propagation delay difference between serving and neighbour cells.

In our view, RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN, and the legacy principle can be reused, that is, it is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC adjustment based on propagation delay difference between serving and neighbour cells.

Observation 1: RAN2 has agreed that it is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC adjustment based on propagation delay difference between serving and neighbour cells.

Proposal 1: RAN4 to reuse legacy principle to define the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

- It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC adjustment based on propagation delay difference between serving and neighbour cells.

  - The impact of HD-FDD

- The impact on NR Handover due to HD-FDD

In RAN4#114-bis meeting, RAN4 discussed the impact on NR Handover due to HD-FDD and there were two options listed. Option 1 was to use the same principle as HD-FDD RedCap requirements, and option 2 is no spec impact. The agreement and Way Forward are copied as below [1]:

RAN4#114-bis: Agreement: - Option 1: agree on the principle below and further discuss the wording in the CR: - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU - Option 2: No spec impact RAN4#115: <Way Forward>: - Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'. - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

For option 2, we understand that defining applicability rules may be mis-interpreted as SSB may be dropped due to UL, which is not the intention. However, we slightly prefer option 1 to reuse legacy principle, as there is no difference on handover for HD-FDD between RedCap UE and RedCap with NTN.

Proposal 2: For the impact on NR Handover due to HD-FDD, we slightly prefer option 1 to reuse legacy principle, as there is no difference on handover for HD-FDD between RedCap UE and RedCap with NTN.

  - UE capability

- General principle for designing the UE capabilities for (e)RedCap UE with NTN

In the last meeting, RAN4 discussed the principle for designing the UE capabilities for (e)RedCap UE with NTN, and the WF on UE capabilities for (e)RedCap UE with FR1-NTN bands is copied as below [1]:

<Way Forward>: - Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

Firstly, both option 1 and option 2 are workable and what we are discussing here is which method is better.

In our view, the advantage of option 1 is simplicity and rationality, as most of UE features defined for NTN and RedCap can be reused for (e)RedCap UE with NTN. From the perspective of RAN2’s signaling, it may not be necessary to define new capabilities for (e)RedCap UE with FR1-NTN, as there may be duplication and waste, it will also increase the workload of RAN2.

However, RAN4 need to clarify that whether Rel-19 requirements defined for (e)RedCap UE with NTN will apply for R17/R18 (e)RedCap UE that supports RedCap feature and NTN feature respectively. If it applies, then Rel-19 requirements can be release independent from Rel-17, and the LS to RAN2 and the corresponding CR to TS 38.307 are needed. If it does not apply, then a new capability is needed to declare support for RedCap over NTN.

Observation 2: No new features have been introduced for (e)RedCap UE with FR1-NTN from RAN4 perspective.

Proposal 3: RAN4 need to clarify that whether Rel-19 requirements defined for (e)RedCap UE with NTN will apply for R17/R18 (e)RedCap UE that supports RedCap feature and NTN feature respectively.

Proposal 4: If it applies, RAN4 to declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- The Rel-19 requirement can be release independent from Rel-17.

- The LS to RAN2 for necessary UE capability description update to enable (e)RedCap UE with FR1-NTN and the corresponding CR to TS 38.307 are needed.

Proposal 5: If it does not apply, then a new capability is needed to declare support for RedCap over NTN.

- Conclusion

This document discussed the issues for (e)RedCap RRM requirements for R19 NR NTN and presented following observations and proposals:

Observation 1: RAN2 has agreed that it is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC adjustment based on propagation delay difference between serving and neighbour cells.

Proposal 1: RAN4 to reuse legacy principle to define the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN.

- It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC adjustment based on propagation delay difference between serving and neighbour cells.

Proposal 2: For the impact on NR Handover due to HD-FDD, we slightly prefer option 1 to reuse legacy principle, as there is no difference on handover for HD-FDD between RedCap UE and RedCap with NTN.

Observation 2: No new features have been introduced for (e)RedCap UE with FR1-NTN from RAN4 perspective.

Proposal 3: RAN4 need to clarify that whether Rel-19 requirements defined for (e)RedCap UE with NTN will apply for R17/R18 (e)RedCap UE that supports RedCap feature and NTN feature respectively.

Proposal 4: If it applies, RAN4 to declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- The Rel-19 requirement can be release independent from Rel-17.

- The LS to RAN2 for necessary UE capability description update to enable (e)RedCap UE with FR1-NTN and the corresponding CR to TS 38.307 are needed.

Proposal 5: If it does not apply, then a new capability is needed to declare support for RedCap over NTN.

- Reference

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- R2-2504769, Capability for SMTC adjustment in RRC_IDLE and RRC_INACTIVE, RAN2 #130

# 5 Annex

3GPP TSG-RAN WG4 Meeting # 116                                                             R4-22XXXXX

# Bengaluru , IN, 25th – 29th Aug, 2025

Title:	LS on UE capability signalling for (e)RedCap UE with FR1-NTN

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG 4

To:	RAN WG 2

Cc:

Contact person:

Name:	Lingyu Gao

E-mail Address:	gaolingyu@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1	Overall description

RAN4 is defining the requirements for (e)RedCap UE with FR1-NTN in Rel-19. RAN4 has agreed to declare the capabilities for (e)RedCap UE with FR1-NTN through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17), and the requirements defined for (e)RedCap UE with FR1-NTN in Rel-19 can be supported from Rel-17 in release independent manner. Therefore, the description of existing UE capabilities signalling for RedCap and NTN may need to be updated.

RAN4 would like to ask RAN2 to introduce necessary UE capability description update to Rel-17/Rel-18 specifications to enable (e)RedCap UE to operate in FR1-NTN bands.

2	Actions

To RAN WG2 group

ACTION: 	RAN4 respectfully requests RAN2 to take the provided information into account and introduce necessary UE capability description update to enable (e)RedCap UE to operate in FR1-NTN bands.

3	Dates of Next RAN WG4 Meetings

TSG-RAN WG4 Meeting #116-bis	13 Oct  – 17 Oct 2025	     Prague, CZ

TSG-RAN WG4 Meeting #117	17 Nov – 21 Nov 2025	     Dallas , US