3GPP RAN WG4 Meeting #116	R4-2509304

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on testability and interoperability issues for AIML mobility

Source: 	CATT

Agenda item:	7.19.4

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 continued to discuss the AI/ML mobility SI and the agreements were captured in [1]. In this contribution, we will further discuss the open issues summarized in [2] and analyse the impacts of testability and interoperability of AI/ML mobility.

Discussion

Testing goals

In RAN4#115 meeting, some proposals were brought up to refine the testing goal but no agreement was achieved. The way forward is as follows:

RAN4#115: <Way forward >: - Discuss whether to further clarify the testing goal for measurement prediction use case, e.g., - The testing goal for measurement prediction is to verify: - The reported measured results meet certain requirements under certain conditions - The reported predicted results meet certain requirements under certain conditions - Note: the requirements include core and performance requirements - FFS the relative requirements for the difference between reported measured results and reported predicted results

In the TR 38.744 [3], RAN2 clarified study goals for all use cases and the evaluation metrics are also provided for the simulation campaign:

TR 38.744 The use cases in this study focus on RRC_CONNECTED mode and cover RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure in standalone NR scenario. The study of the use cases is driven mainly by two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption). … Measurement reduction rate for intra-frequency scenario is defined in the temporal domain (called MRRT) by assuming same length of measurement time instances and spatial domain respectively(called MRRS): MRRT = skipped measurement time instances / total measurement time instances MRRS = skipped beams to be measured/ total beams to be measured

According to the TR, the motivation of this feature is to save measurement efforts or to improve HO performance which requires high measurement accuracy. Therefore the aspects that matter are the measurement efforts that UE reduces and the measurement quality that UE can provide. Given that the MRRT or MRRS is a fixed value after the corresponding measurement configuration signalling is transmitted to UEs, RAN4 can test whether the prediction results satisfy the existing measurement accuracy requirements under certain side conditions, e.g., MRRT/MRRS and SINR. However, considering that August meeting is the last meeting for this study item and RAN4 will start WI work from October meeting, we suggest focusing on specifying RRM core and performance requirements directly instead of spending time on a high-level testing goal discussion.

Proposal 1: RAN4 to focus on specifying RRM core and performance requirements directly.

Testing generalization

Regarding the testing generalization, its purpose is mainly to evaluate the generalization of AI/ML models, i.e., the performance robustness of AI/ML models in different scenarios. This issue was barely discussed in this study item due to limited time and therefore no conclusions have been achieved so far.

- RAN4 #115 Agreements: - Issue 1-1-4: Testing of Generalization aspects No conclusions on generalization aspects have been achieved so far. Note: No conclusion for this issue will not stop the completion of this study item.

Considering that this issue has been discussing in	AI/ML for NR Air Interface WI, we suggest postponing the detailed discussions on this issue. Even in WI phase, this work item can still wait for progress of generalization issues in the NR_AIML_Air work item and then leverage the methodology of test framework to improve the discussion efficiency. In August meeting, RAN4 can choose a typical scenario in this feature to initiate a high-level discussion on the test generalization framework which can be then expanded to other scenarios when the test framework is determined.

Proposal 2: RAN4 to wait for and leverage the methodology in NR_AIML_Air work item.

Proposal 3: RAN4 to choose a typical scenario in this feature to initiate a high-level discussion on the test generalization framework.

Testing configuration

In RAN#112bis meeting, it was agreed to adopt conducted testing method for AI/ML mobility test in FR1 and consider at least the following aspects for this SI:

	-	Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3)

	-	Testing cell-level L3-RSRP measurements

	-	Consideration of serving and neighbor cells

Firstly, we would like to discuss how to let DUT obtain cell-level L3-RSRPs at model input in FR1 conducted test. In general, there are two methods: (1) configure only one SSB resource for each serving/neighbor cell and DUT performs RSRP measurements on each cell, and the results are treated as cell-level RSRPs which then can be used as model inputs for further predictions. (2) configure multiple SSB resources for each serving/neighbor cell and the RSRP results are regarded as beam-level RSRPs, and then DUT calculates cell-level RSRPs for all cells based on these beam-level RSRPs. A figure illustrating these two methods is provided in Figure 1. Considering that the test will be carried out in a conducted manner and RAN4 agree to prioritize scenario 2, 3 and 4 which focus on saving measurement efforts in temporal or frequency domain, method (1), in our opinion, is sufficient to test DUT’s capability of AI/ML-enabled RRM measurement prediction.

Regarding channel model, since the prioritized scenarios in RAN4 do not involve predictions in spatial domain, AWGN or TDL channel can serve the purpose, which does not require upgrade on existing test equipment.

Figure 1. Setting up cell-level RSRP in FR1 conducted test

Proposal 4: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

Proposal 5: AWGN or TDL channel can be used in FR1 conducted test.

For number of cells configured in FR1 test, RAN4 discussed this issue before and reached the following agreements:

RAN4#114bis Agreement: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction. RAN4#115 Agreement: - Issue 1-2-3: Testing Setup for intra-frequency prediction < Agreement>: - Clarify the agreement in last meeting: - For intra-frequency intra-cell RRM measurement prediction/event prediction, RAN4 to consider two cells in the test: serving cell and another/target cell. - Note: The measurement and prediction are performed on the same cell.

There is one open issue that needs further discussion. For inter-cell RRM measurement prediction/event prediction, two approaches are considered by RAN2, i.e., single-cell approach and cluster approach. If single-cell method is adopted, configuring 2 cells in tests are enough, and more cells may be needed if cluster approach is adopted. Therefore we suggest RAN4 to decide the exact number of cells for inter-cell RRM measurement prediction/event prediction in WI phase.

Proposal 6: 2 cells are enough if single-cell approach is adopted and more cells are needed if cluster approach is used. RAN4 can decide the exact number in WI phase.

Testability of inter-frequency scenario

Inter-frequency prediction is a slightly special case since UE predicts the RSRP of carrier 1 based on the measurement result of carrier 2. Some conditions need to be met if a good performance of this mechanism is expected. One of the key aspects is these two cells needs to be co-located. How to reflect their colocation in test is a main problem. Since test up in FR2 is preferred to be deprioritized, the following discussion focuses on FR1 only.

First, both carriers belong to the same frequency range, which means they experience a similar propagation condition over the air, e.g., they probably have a similar pathloss. If reference signals carried by these two carriers are transmitted with the same power, the RSRP at the DUT may be similar as well. Hence, a straightforward method is to reflect the co-location relationship by a minor RSRP difference between reference signals carried by co-located carriers.

Proposal 7: Colocation can be reflected by a minor RSRP difference between two carriers.

Conclusions

This contribution discussed the testability and interoperability issues in AI/ML mobility and the following proposals are provided:

Proposal 1: RAN4 to focus on specifying RRM core and performance requirements directly.

Proposal 2: RAN4 to wait for and leverage the methodology in NR_AIML_Air work item.

Proposal 3: RAN4 to choose a typical scenario in this feature to initiate a high-level discussion on the test generalization framework.

Proposal 4: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

Proposal 5: AWGN or TDL channel can be used in FR1 conducted test.

Proposal 6: 2 cells are enough if single-cell approach is adopted and more cells are needed if cluster approach is used. RAN4 can decide the exact number in WI phase.

Proposal 7: Colocation can be reflected by a minor RSRP difference between two carriers.

References

[1] R4-2508269, WF on FS_NR_AIML_Mob_Part2, OPPO, RAN4#115.

[2] R4-2505565, Topic summary for [115][216] FS_NR_AIML_Mob_Part2, OPPO, RAN4#115.

[3] 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, V1.0.0.