3GPP TSG-RAN WG4 Meeting #116                                                                                           R4-2509322

Bengaluru, India, August 25th – 29th, 2025

Source: 	CATT

Title: 	Discussion on SBFD general issues for SBFD operation including BS conformance testing

Agenda item:	7.21.2.1

Document for:	Discussion

- Introduction

The WF of RAN4#115 [1] concluded that it is necessary to study open issues for SBFD parameters, and analysis of BS conformance testing is also required. In this contribution, we provide our analysis of them.

- Discussion

  - DU/UL subband declaration

Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For all FR2 BS classes, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested.

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

    - X value will be selected in next meeting.

To fully test the performance of high-power BSs, 33dBm is a reasonable power level threshold for BSs supporting FR1 MR SBFD function. If the BS output power is higher than this threshold, all configurations shall be tested. When the BS output power is lower than this threshold, the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

Proposal 1: About declaration and flexibility for SBFD operation, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

  - DL/UL subband configurations

According to WF [1] of RAN4#115, some issues about DL/UL subband configurations need to be further studied.

2.2.1 Restriction on the number of UL subband sizes supported

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

Agreement:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 subband configurations allowed for FR1 WA BS

    - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

    - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

  - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

    - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

To fully test the performance of high-power BSs, 33dBm is a reasonable power level threshold for BSs supporting FR1 MR SBFD function. If the BS output power is higher than this threshold, the maximum number of specific UL subband sizes is same as that of FR1 WA BS, i.e.3. When the BS output power is lower than this threshold, the restriction on the number of UL subband sizes supported is not applicable.

Proposal 2: About restriction on the number of UL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

2.2.2 Restriction on the number of DL subband sizes supported

Issue 1-2-2: Restriction on the number of DL subband sizes supported

Agreement:

- For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

  - This restriction shall be applied to FR1 WA BS

  - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

    - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

To fully test the performance of high-power BSs, 33dBm is a reasonable power level threshold for BSs supporting FR1 MR SBFD function. If the BS output power is higher than this threshold, the restriction on the number of DL subband sizes supported is same as that of FR1 WA BS. When the BS output power is lower than this threshold, the restriction on the number of DL subband sizes supported is not applicable.

Proposal 3: About restriction on the number of DL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

2.2.3 Restriction on DL subband size

Issue 1-2-4: Restriction on DL subband size

Agreement:

- If the SSB symbol is overlapped with SBFD symbol,

  - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration

  - FFS the test configuration during performance part discussion.

For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required, as the SSB can occupy only one “D” within the DUD configuration.

Proposal 4: For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required.

- System parameters of conformance testing

SBFD capable BS doesn’t support type 1-C, and this general principle needs to be captured in TS 38.141-1.

Proposal 5: The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1.

The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2.

Proposal 6: The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2.

The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to both TS 38.141-1 and TS 38.141-2.

Proposal 7: The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2.

- RF channels to be tested for SBFD operation

When testing SBFD modes, the SBFD carrier can be configured at both the bottom and top positions within the operating frequency band, as these locations are the positions with the strongest out-of-band interference.

Proposal 8: When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

- Conclusion

This contribution provides our further analysis for SBFD general issues. We have the following proposals.

Proposal 1: About declaration and flexibility for SBFD operation, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

Proposal 2: About restriction on the number of UL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

Proposal 3: About restriction on the number of DL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm.

Proposal 4: For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required.

Proposal 5: The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1.

Proposal 6: The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2.

Proposal 7: The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2.

Proposal 8: When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

- References

[1] R4-2508707, “Way Forward for [115][306] NR_duplex_evo_General”, Samsung, RAN4#115