3GPP TSG-RAN WG4 Meeting # 116                                                              R4-2509325

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on modification of existing Rx requirements for SBFD operation including BS conformance testing

Source: 	CATT

Agenda item:	7.21.3.3

Document for:	Discussion

- Background

WF [1] of RAN4#115 concluded that further studies are needed for some Rx RF requirements and BS conformance testing. This contribution provides our further analysis of the requirements.

- Impact on existing BS Rx RF requirement

  - ACS requirement

Issue 2-1: ACS requirement

- Way forward

  - FFS on whether the assumptions that will be applied for in-band blocking requirements derivation can be applied for ACS requirement.

NR BS ACS is dominated by UE operating in adjacent channel, but for SBFD-capable BS, when it is configured as SBFD slot, the interference signal level in adjacent channel is dominated by BS operating in adjacent channel, it is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

Proposal 1: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

The assumptions that applied for in-band blocking requirements derivation include the ACS interference signal frequency range, so it can be applied for ACS requirement.

Proposal 2: The assumptions that applied for in-band blocking requirements derivation can be applied for ACS requirement.

  - In-band blocking

In WF [1], in-band blocking requirements need to be further studied.

  - Issue 2-2: In-band blocking for FR1 WA BS

- Way forward

- For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point:

  - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift.

  - For requirements assumption as deployment guidance, FFS on the following options

    - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive

    - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values

    - Option 3: One note can be added to generally describe the deployment restriction.

  - For requirement definition, FFS on the following aspects

  - Interference source: interference from BS operating at adjacent channel(s)

    - Interference signal CBW: TBD

    - Interference signal mean power: TBD

    - Interference signal frequency:

      - For DU configuration:  [TBD] offset from the lower Base Station RF Bandwidth edge

      - For UD configuration:  [TBD]  offset from the upper Base Station RF Bandwidth edge

      - For DUD configuration: [TBD]  offset from the lower/upper Base Station RF Bandwidth edge

  - For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered

      - Issue 2-3: In-band blocking for FR2-1 WA BS

- Way forward

  - Encourage companies to have offline discussion for the simulation assumptions before next meeting.

      - Issue 2-4: In-band blocking for FR1 Micro BS

- Way forward

  - For simulation RAN4 to use the same grid shift assumption as FR1 Macro BS as starting point

  - Companies to check the blocking level in the range of [-20,  -32] dBm

In the last meeting, the in-band blocking signals power level for FR1 WA BS with 100% grid shift ranged from -28dBm to -16.7dBm, different companies supplied different values, and we propose to take -25dBm as the reference.

Proposal 3: For the in-band blocking signals power level for FR1 WA BS, we propose to take -25dBm as the reference for 100% grid shift.

We can assume a coupling loss between aggressor BS and victim BS based on the distance and angles with respect to antenna apertures for different frequencies and the assumption of coupling loss can be added as a standard note.

Proposal 4: The assumption of coupling loss between aggressor BS and victim BS can be added as a standard note.

For in-band blocking requirement, interference signals from BS operating at adjacent channel(s) need to be considered. The BS channel bandwidth of the lowest/highest carrier received, type of interfering signal and interfering signal centre frequency minimum offset from the lower/upper base station RF bandwidth edge can reuse the value specified in table 7.4.2.2-1 in TS 38.104, but the mean power of interference signals can be defined as the final concusions for different class BS.

Proposal 5:  For in-band blocking requirement, the BS channel bandwidth, 	interference signal frequency offset for three configurations and interfering signal type can reuse the value specified for NR BS.

Proposal 6: The mean power of interference signals can be defined as the final conclusions for different class BS.

In the last meeting, the in-band blocking signals power level for FR1 MR BS with 100% grid shift ranged from -32dBm to -20dBm, different companies supplied different values, and we propose to take -32dBm as the reference.

Proposal 7: For the in-band blocking signals power level for FR1 MR BS, we propose to take -32dBm as the reference for 100% grid shift.

According to [2], the simulation results of in-band blocking for FR2-1 are shown in table 1, and the interference are from TDD DL to SBFD UL.

Table 1: SBFD OTA in-band blocking simulation results (FR2-1, TDD DL to SBFD UL sub-band)

| Scenario | Frequency | Aggressor -> Victim | in-band blocking simulation results (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 99% CDF (100% grid-shift) | 99% CDF (10% grid-shift) |
| 6 | FR2-1 | Urban Macro -> Urban Macro | -51.7 | -39.8 |

For the FR2-1 urban macro scenario, the OTA in-band blocking signals power level of SBFD capable BSs of 100% grid shift is 6.3 dB higher than the corresponding value specified for NR BS, if the difference is acceptable, the existing requirements can be reused.

Observation 1: For the FR2-1 urban macro scenario, the OTA IBB signals power level of SBFD capable BSs of 100% grid shift is 6.3 dB higher than the corresponding value specified for NR BS.

  - Rx intermodulation

In WF [1], the necessity of new RX intermodulation requirement with one interfering signal needs to be further studied.

      - Issue 2-5: Necessity of New RX intermodulation requirement with 1 or 2 interfering signal

- Way forward

  - Down-select to one of the below two options in the next meeting

    - Option 1: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case

    - Option 2: New IMD requirement is needed, the following scenario with 2 input signals can be regarded as the worst case

      - Interfering signal 1: CW

      - Interfering signal 2:  TBD MHz NR

      - Power level: the final agreed in-band blocking level -9 dB

For the issue of if single interfering signal scenario is covered by IBB requirement, it depends on the frequency position of the single interfering signal. For some SBFD DL and UL spectrum configurations, the single interference signal scenario is implicitly captured by the SBFD RX blocking requirement, even if the frequency range of the single interference signal is from out-of-band to in-band rather than only in-band.

Observation 2: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

For the power level of the receiver intermodulation interference signal, whether BS2BS CLI needs to be considered depends on the power level values of the interference, and the power level can be simulated. The in-band blocking of the SBFD slot has been simulated by RAN4, and the interference signals are from BS of frequency adjacent system, these interference signals can be considered by legacy Rx intermodulation, and new RX intermodulation requirement with two interfering signals is not necessary.

Observation 3: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 8:  New RX intermodulation requirement is not necessary.

  - ICS

      - Issue 2-6: In-channel selectivity

- Way forward

  - Check whether existing ICS requirements can be reused

Table 2:  SBFD dynamic range simulation results

| Scenario | Frequency | Aggressor -> Victim | Accuracy | IoT (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 100% grid-shift |  |  | 10% grid-shift |  |  |
|  |  |  |  | 95% CDF | 99% CDF | 100% CDF | 95% CDF | 99% CDF | 100% CDF |
| 1 | FR1 | Urban Macro -> Urban Macro | 0dB | 7.9 | 15.5 | 21.0 | 10.6 | 16.8 | 24.0 |
|  |  |  | ±9dB | 9.3 | 15.9 | 22.8 | 11.9 | 17.4 | 23.5 |
| 3 | FR1 | Indoor -> Indoor | 0dB | 13.3 | 15.3 | 16.7 | —— | —— | —— |
|  |  |  | ±9dB | 15.0 | 18.2 | 21.0 | —— | —— | —— |
| 5 | FR1 | Micro -> Micro | 0dB | 3.5 | 10.0 | 15.5 | 6.5 | 13.0 | 18.5 |
|  |  |  | ±9dB | 5.1 | 12.0 | 21.5 | 8.1 | 15.1 | 24.6 |
| 6 | FR2-1 | Urban Macro -> Urban Macro | 0dB | 4.0 | 8.9 | 8.9 | 6.3 | 11.5 | 12.2 |
|  |  |  | ±9dB | 5.2 | 17.6 | 19.6 | 7.5 | 21.5 | 21.8 |
| 9 | FR2-1 | Indoor -> Indoor | 0dB | -1.4 | 0.5 | 1.6 | —— | —— | —— |
|  |  |  | ±9dB | 0.2 | 2.6 | 7.6 | —— | —— | —— |

For FR1 NR BS, ICS is agreed as 25dBc for all BS type which is the same as legacy E-UTRA ICS, and the ICS value corresponds to 16dB interference-over-thermal and a 9 dB C/I assumption. According to the simulation results of dynamic range of FR1 SBFD capable BSs, when grid shift is 100% and CDF is 95%, the IoTs of all scenarios are less than 16dB, and dynamic range simulation considered both uplink signal and gNB-to-gNB CLI signal, but the ICS only consider uplink signal, so IoTs for ICS for FR1 scenarios are less than 16dB, so the existing ICS requirements of FR1 NR BS can be reused.

For FR2 NR BS, ICS is agreed as 14dBc for all BS type, and the ICS value corresponds to 5dB interference-over-thermal and a 9 dB C/I assumption. Similar as FR1, IoTs for ICS for FR2 scenarios are less than or close to 5dB, so the existing ICS requirements of FR2 NR BS can be reused.

Proposal 9:  The existing ICS requirements of FR1 NR BS can be reused for FR1 SBFD capable BS.

Proposal 10:  The existing ICS requirements of FR2 NR BS can be reused for FR2 SBFD capable BS.

- Conformance testing for Rx requirements

  - Additional manufacture declarations and updates

In TS 38.104, the Rx requirements for SBFD capable BS and NR BS are specified in separate clauses. In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS.

Proposal 11: In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS.

According to existing TS 38.141-2, OTA test requirements specific and OTA measurement chamber specific calibration (and measurement) procedures were captured in TR 37.941 for the following requirements sets:

-	TX and Rx directional requirements

-	In-band and out-of-band TRP requirements

-	Co-location requirements

-	In-band and out-of-band blocking requirements

The definition of OTA sensitivity for SBFD capable BS takes self-interference into account. Therefore, when testing Rx requirements, all the Tx transmitters must be turned on. Thus, the OTA measurement chamber setup and calibration (and measurement) procedures for SBFD capable BS are different from those of NR systems. Therefore, the differences must be added to TS 38.141-2.

Proposal 12: The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2.

  - Beam configurations

The definition of OTA sensitivity accounts for self-interference, thus the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them. The SBFD capable BS implements separated Rx and Tx antenna planes, when testing Rx OTA requirements, the antenna beam configurations of both the interference signal input antenna and the test output antenna must also be considered.

Proposal 13: When test OTA Rx requirements of SBFD capable BS, the transmitters must be turned on.

Proposal 14: For OTA Rx requirements testing of SBFD capable BS, the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them.

For OTA Rx requirements, in addition to the beam configurations of the Tx and Rx antennas, it is also necessary to consider the directional correspondence that needs to be satisfied between the test input antenna beam and the Rx antenna beam.

Proposal 15: For OTA Rx requirements testing of SBFD capable BS, it is necessary to consider the directional correspondence between the test input antenna beam and the Rx antenna beam.

For OTA co-location blocking requirements, in addition to the beam configurations of the Tx and Rx antennas, it is also necessary to consider the isolation requirement that needs to be satisfied between the beam of the co-location interference signal input antenna and the Rx antenna beam.

Proposal 16: For OTA co-location blocking requirement testing of SBFD capable BS, it is necessary to consider the isolation requirement between the beam of the co-location interference signal input antenna and the Rx antenna beam.

  - Additional test models for SBFD-capable BS

Since the definition of OTA sensitivity takes self-interference into account, both the transmitter and the receiver operate simultaneously, and a new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS.

Proposal 17: A new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS.

In existing TS 38.141-1 and TS 38.141-2, there are test requirements for each Rx requirement of NR BS. These test requirements can serve as the baseline for SBFD capable BS, but the NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS. Additionally, any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

Proposal 18: Test requirements for each Rx requirement of NR BS in TS 38.141-1 and TS 38.141-2 can serve as the baseline for SBFD capable BS.

Proposal 19: In test requirements, NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS.

Proposal 20: Any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level.

Proposal 2: The assumptions that applied for in-band blocking requirements derivation can be applied for ACS requirement.

Proposal 3: For the in-band blocking signals power level for FR1 WA BS, we propose to take -25dBm as the reference for 100% grid shift.

Proposal 4: The assumption of coupling loss between aggressor BS and victim BS can be added as a standard note.

Proposal 5:  For in-band blocking requirement, the BS channel bandwidth, 	interference signal frequency offset for three configurations and interfering signal type can reuse the value specified for NR BS.

Proposal 6: The mean power of interference signals can be defined as the final conclusions for different class BS.

Proposal 7: For the in-band blocking signals power level for FR1 MR BS, we propose to take -32dBm as the reference for 100% grid shift.

Observation 1: For the FR2-1 urban macro scenario, the OTA IBB signals power level of SBFD capable BSs of 100% grid shift is 6.3 dB higher than the corresponding value specified for NR BS.

Observation 2: The single interference signal scenario is implicitly captured by the SBFD RX blocking requirement.

Observation 3: Interference signals from BS of frequency adjacent system can be considered by legacy Rx intermodulation requirement.

Proposal 8:  New RX intermodulation requirement is not necessary.

Proposal 9:  The existing ICS requirements of FR1 NR BS can be reused for FR1 SBFD capable BS.

Proposal 10:  The existing ICS requirements of FR2 NR BS can be reused for FR2 SBFD capable BS.

Proposal 11: In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS.

Proposal 12: The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2.

Proposal 13: When test OTA Rx requirements of SBFD capable BS, the transmitters must be turned on.

Proposal 14: For OTA Rx requirements testing of SBFD capable BS, the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them.

Proposal 15: For OTA Rx requirements testing of SBFD capable BS, it is necessary to consider the directional correspondence between the test input antenna beam and the Rx antenna beam.

Proposal 16: For OTA co-location blocking requirement testing of SBFD capable BS, it is necessary to consider the isolation requirement between the beam of the co-location interference signal input antenna and the Rx antenna beam.

Proposal 17: A new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS.

Proposal 18: Test requirements for each Rx requirement of NR BS in TS 38.141-1 and TS 38.141-2 can serve as the baseline for SBFD capable BS.

Proposal 19: In test requirements, NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS.

Proposal 20: Any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

- Reference

[1] R4-2508713 “Way Forward for [115][307] NR_duplex_evo_BSRF”, Huawei, RAN4#115

[2] R4-2415106, “SBFD in-band blocking and dynamic range simulation results”, CATT, RAN4#112bis