3GPP TSG-RAN WG4 Meeting # 116                                                              R4-2509328

Bengaluru, India, August 25th – 29th, 2025

Title: 	                 Discussion on RF requirements for CW for D1T1

Source: 	CATT

Agenda item:	7.22.3.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for RF Requirements for CW for scenario D1T1-B according to the WF [1].

- RF Requirement for CW for D1T1

2.1 Phase noise

### Issue 3-4: Phase noise

- Agreement:

  - For BS with phase noise cancellation capability, no phase noise requirement is specified.

  - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS

A-IoT BS receiver reference sensitivity degradation needs to consider the impact of CW interference signals. A-IoT BS requires the cancellation of received CW interference signals, and the cancellation capabilities vary among different companies, so the impaction of CW phase noise should be considered by BS companies, and the phase noise requirement of CW signal needn’t to be introduced in Rel-19 WI.

Proposal 1: In Rel-19 WI, phase noise requirement of CW signals needn’t to be introduced.

2.2 Unwanted emission

- Issue 3-5: Unwanted emission

- Agreement:

  - FFS on unwanted emission boundary and corresponding values:

  - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth

Channel bandwidth for CW signal needs to be defined, and both the definition of the requirement and the CW interference cancellation capability requirement are related to it. The channel bandwidth for CW can be defined as 15kHz.

  - Option 2: FFS whether or not to define channel bandwidth for CW

  - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

  - Other options are not precluded.

Proposal 2: Channel bandwidth for CW signal needs to be defined.

Proposal 3: The channel bandwidth for CW signal can be defined as 15kHz.

Table 1: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

The spectrum emission mask for CW node can be defined as table 1. The Δf is defined from the center of the CW transmission. The frequency range of Δf from  0 to 90kHz can be covered by phase noise and can be left to implementation.

Proposal 4: The frequency range of Δf from 0 to 90kHz can be covered by phase noise and left to implementation.

Proposal 5: The spectrum emission mask for CW node can be defined as table 1.

- Summary

This contribution provides our analysis for RF Requirements for CW for scenario D1T1-B according to the WF [1], and our proposals can be summarized in the following:

Proposal 1: In Rel-19 WI, phase noise requirement of CW signals needn’t to be introduced.

Proposal 2: Channel bandwidth for CW signal needs to be defined.

Proposal 3: The channel bandwidth for CW signal can be defined as 15kHz.

Proposal 4: The frequency range of Δf from 0 to 90kHz can be covered by phase noise and left to implementation.

Proposal 5: The spectrum emission mask for CW node can be defined as table 1.

Reference

[1] R4-2508101, “WF on A-IoT BS and CW requirements”, Huawei, RAN4#115