3GPP TSG-RAN WG4 Meeting # 116                                                              R4-2509329

# Bengaluru, India, August 25th – 29th, 2025

Title: 	                 Discussion on system parameters for A-IoT

Source: 	CATT

Agenda item:	7.22.2

Document for:	Discussion

- Introduction

This contribution provides our further analysis for R2D and D2R system parameters according to the WF [1].

- Discussion

2.1 System parameter for R2D

- 2.1.2 R2D channel raster

The transmission bandwidth configuration of R2D is 1PRB, 2PRB, 3PRB and 4PRB with 15 kHz SCS, and the channel raster is 10kHz. For both standalone and in-band modes, if the mapping between the A-IoT RF reference frequency on the channel raster is same as NR system, there is no channel raster offset between them.

Proposal 1: We propose that the mapping between the A-IoT RF reference frequency and the channel raster is the same as that of the NR system, and there is no channel raster offset between them.

2.2 System parameter for D2R

In WF[1], D2R bandwidth needs to be further studied.

Issue 2-2-1: D2R bandwidth

Agreement in RAN4#115:

- Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1.

  - Assumed 10% SFO for device

  - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered.

- The value of transmission BW and channel BW will be explicitly listed.

- Using following equation for BS as starting point:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

  - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

- Using following equation for device as starting point:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

- The following are for information:

- Figure 1-1 Small frequency shift diagram of FDMA without SFO

- Figure 1-2. SFO impact of transmission bandwidth and small frequency shift of FDMA

- As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scales the small frequency shift value .

- The 2SB transmission BW and small frequency shift _both measured in kHz——with and without SFO are as follows:

- Note : BW and  Small frequency shift  are in kHz;Tc and Tb are in μs

| 2SB Transmission BW (kHz) |  | Small frequency shift (kHz) |  |
| --- | --- | --- | --- |
| Without SFO | With SFO | Without SFO | With SFO |
| Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc) | Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣) | SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc) | SFSwith SFO =SFSwithout SFO×(1±∣SFO∣) |

Table 1 Final Tb, Tchip, and R

|  | Tchip (μs) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  |
| 133.33 |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  |
| 66.67 |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  |
| 33.33 |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  |
| 16.67 |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  |
| 8.33 |  |  |  |  |  | R=1 | R=2 | R=4 |  |
| 4.17 |  |  |  |  |  |  | R=1 | R=2 |  |
| 1.39 |  |  |  |  |  |  |  |  | R=1 |

Table 2: D2R channel bandwidth for device with 10% SFO

| D2R channel bandwidth for device with 10% SFO (kHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) | Tchip (μs) |  |  |  |  |  |  |  |  | 2SB BW=4/Tb (kHz) |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |  |
| 266.67 | 16.50 | 24.75 | 41.25 | 74.25 | 140.25 | 272.25 | 536.25 | 1064.25 |  | 15 |
| 133.33 |  | 33.00 | 49.50 | 82.50 | 148.50 | 280.50 | 544.50 | 1072.50 |  | 30 |
| 66.67 |  |  | 66.00 | 99.00 | 165.00 | 297.00 | 561.00 | 1089.00 |  | 60 |
| 33.33 |  |  |  | 132.00 | 198.00 | 330.00 | 594.00 | 1122.00 |  | 120 |
| 16.67 |  |  |  |  | 264.00 | 396.00 | 660.00 | 1188.00 |  | 240 |
| 8.33 |  |  |  |  |  | 528.00 | 792.00 | 1320.00 |  | 480 |
| 4.17 |  |  |  |  |  |  | 1056.00 | 1584.00 |  | 960 |
| 1.39 |  |  |  |  |  |  |  |  | 3168.00 | 2880 |
| SFS =R/Tb (kHz) | 3.75 | 7.50 | 15.00 | 30.00 | 60.00 | 120.00 | 240.00 | 480.00 | 720.00 |  |

Table 3: D2R channel bandwidth for reader with 10% SFO and 0.9 filter spectrum utility

| D2R channel bandwidth for reader with 10% SFO and 0.9 filter spectrum utility (kHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) | Tchip (μs) |  |  |  |  |  |  |  |  | 2SB BW=4/Tb (kHz) |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 | 2.08 | 1.04 | 0.69 |  |
| 266.67 | 18.33 | 27.50 | 45.84 | 82.50 | 155.83 | 302.50 | 595.83 | 1182.50 |  | 15 |
| 133.33 |  | 36.67 | 55.00 | 91.67 | 165.00 | 311.67 | 605.00 | 1191.67 |  | 30 |
| 66.67 |  |  | 73.34 | 110.00 | 183.33 | 330.00 | 623.33 | 1210.00 |  | 60 |
| 33.33 |  |  |  | 146.67 | 220.00 | 366.67 | 660.00 | 1246.67 |  | 120 |
| 16.67 |  |  |  |  | 293.33 | 440.00 | 733.33 | 1320.00 |  | 240 |
| 8.33 |  |  |  |  |  | 586.67 | 880.00 | 1466.67 |  | 480 |
| 4.17 |  |  |  |  |  |  | 1173.33 | 1760.00 |  | 960 |
| 1.39 |  |  |  |  |  |  |  |  | 3520.00 | 2880 |
| SFS =R/Tb (kHz) | 3.75 | 7.50 | 15.00 | 30.00 | 60.00 | 120.00 | 240.00 | 480.00 | 720.00 |  |

The final Tb, Tchip, and R which have been agreed in RAN 1 are shown in table 1, the 2SB bandwidth and SFS can be calculated bases on R and Tb, then the D2R channel bandwidth for device with 10% SFO and D2R channel bandwidth for reader with 10% SFO and 0.9 filter spectrum utility can be calculated using the formulators shown in WF[1], the calculate results are shown in table 2 and table 3.

Proposal 2: The D2R channel bandwidth for device with 10% SFO is shown in table 2.

Proposal 3: The D2R channel bandwidth for reader with 10% SFO and 0.9 filter spectrum utility is shown in table 3.

2.3 Other issue

The maximum number of devices allowed to communicate simultaneously with one A-IoT BS needs to be clarified.

Proposal 4: The maximum number of devices allowed to communicate simultaneously with one A-IoT BS needs to be clarified.

- Summary

This contribution provides our further analysis for A-IoT system parameters. We have the following observation and proposals.

Proposal 1: We propose that the mapping between the A-IoT RF reference frequency and the channel raster is the same as that of the NR system, and there is no channel raster offset between them.

Proposal 2: The D2R channel bandwidth for device with 10% SFO is shown in table 2.

Proposal 3: The D2R channel bandwidth for reader with 10% SFO and 0.9 filter spectrum utility is shown in table 3.

Proposal 4: The maximum number of devices allowed to communicate simultaneously with one A-IoT BS needs to be clarified.

Reference

[1] R4-2508116, “WF on A-IoT_device”, CMCC, RAN4#115

[2] 3GPP TSG RAN WG1 Meeting RAN1#121 Chairman’s Notes, May 19th – 23rd, 2025