3GPP TSG-RAN WG4 Meeting #116                                                                                                         R4-2509332

Bengaluru, India, 25th –29th August, 2025

Source: 	CATT

Title: 	Further discussion on BS demodulation performance for MMSE-IRC

Agenda item:	7.13.4

Document for:	Approval

- Introduction

In RAN4#115 meeting, a Way Forward [1] for [115][322] NR_demod_Ph5_Part1_General_BS was agreed, and the remaining issues on BS demodulation performance for MMSE-IRC in WF require further discussion.

In this contribution, we provide our views on BS demodulation performance for MMSE-IRC.

- Discussion

Applicability for different BS classes and test scenarios

As per the WF [1], the agreement and remaining open issue concerning applicability for difference BS classes and test scenarios is shown as below:

Whether to consider additional applicability rule on how the test is performed - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

From agreement in WFs [1, 2], the TDLC300-100 is defined as propagation condition for HomNet, and TDLA30-10 is defined as propagation condition for HetNet. The delay of TDLC is larger than TDLA, and 300ns delay spread of TDLC300-100 is larger than 30ns delay spread of TDLA30-10, at same time, 100Hz Doppler of TDLC300-100 is larger than 10Hz Doppler of TDLA30-10, so from perspective of propagation, the TDLC300-100 is stringent than TDLA30-10, then, propagation condition of HomNet is worse than that of HetNet, so, in this case, if a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be passed,

From agreement in WF [2] in RAN4#112bis, the INR set1: (5.35.N/A) and (7.28.-2.52) are defined for HomNet, and INR set2: (9.82. NA/) and (12.38. -0.97) are defined for HetNet. The above INR sets show that I+N value for HomNet is less than that for HetNet, which seems to indicate that option 1 is not valid. But the interference level just impacts on the SNR @ 70% of maximum throughput, and the TDLC300-100 is defined as propagation condition for interferers for HomNet and HetNet, so we think that the INR sets can’t be considered as factor for choosing Option1 and Option 2, propagation condition for serving UE as factor for choosing Option1 and Option 2 is sufficient.

Based on above analysis, since propagation condition of HomNet is worse than that of HetNet, so, it is reasonable to define that if a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be passed. So we propose,

Proposal 1: Adopt Option 1 as additional applicability rule for BS MMSE-IRC receiver as below:

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

Based on above proposal, a draft CR for 38.141-1 on applicability rule for BS requirement with MMSE_IRC receiver is provided for approval.

- Conclusion

This contribution provides analysis on additional applicability rule for NR BS MMSE-IRC demodulation. The following proposal is concluded as follows:

Proposal 1: Adopt Option 1 as additional applicability rule for BS MMSE-IRC receiver as below:

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

Based on above proposal, a draft CR [4] for 38.141-1 on applicability rule for BS requirement with MMSE_IRC receiver is provided for approval.

- References

- R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#115

- R4-2504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#114bis

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telcom, RAN4#112bis

- R4-2509334, Draft CR for 38.141-1, Introduction of applicability rule for BS requirement with MMSE_IRC receiver, CATT, RAN4#116.

- R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#114

- R4-2503453, Simulation results for BS requirement with MMSE_IRC receiver, CATT, RAN4#114bis