3GPP TSG-RAN WG4 Meeting #116                                                                                                            R4-2509341

Bengaluru, India, 25th –29th August, 2025

Source: 	CATT

Title: 	Further discussion on framework simplification for co-location/co-existence requirements

Agenda item:	10.1

Document for:	Approval

- Introduction

In RAN4#115 meeting, A Way forward for [115][340] BSRF_Simplify_CoLo_Coex [1] was agreed and remaining issues in WF need further discussion.

In this contribution, we provide our views on framework simplification for co-location/co-existence requirements.

- Discussion

In RAN4#115 meeting, we analyse that notes in note column of Table 6.6.5.2.3-1 of TS 38.104 for co-existence requirement. Based on above analysis, we propose to use the following Table 6.6.5.2.3-1 to simplify the co-existence requirement in TS 38.104. And same approach can be reused for other specs, but this approach method for notes need more works.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of co-existence downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100 kHz | For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence downlink operating band is less than or equal to [5MHz], or BS downlink operating band full covers frequency range of co-existence downlink operating band, or BS downlink operating band partially covers frequency range of co-existence downlink operating band (including only one frequency), this requirement does not apply. Note 1 |
|  | DCS1800 or PCS1900 | -47 | 100 kHz |  |
|  | Other co-existence system type | -52 | 1 MHz |  |
| Frequency range of co-existence uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100 kHz | For NR operating band, if BS uplink operating band or BS downlink operating band full covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band except n28, n67, n68, n1, n2, and n5, if BS downlink operating band or BS uplink operating band partially covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band except n29 and n28, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence uplink operating band is less than or equal to [4MHz], this requirement does not apply. Note 1, Note 2, Note 3, Note 4, Note 5, Note 6, Note 7,Note 8. |
|  | Other co-existence system type | -49 | 1 MHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. NOTE 5: For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz. NOTE 6: For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz. NOTE 7: For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz. NOTE 8: For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz. |  |  |  |  |

Observation 1: The following table can be used to simplify the co-existence requirement in TS 38.104.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of co-existence downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100 kHz | For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence downlink operating band is less than or equal to [5MHz], or BS downlink operating band full covers frequency range of co-existence downlink operating band, or BS downlink operating band partially covers frequency range of co-existence downlink operating band (including only one frequency), this requirement does not apply. Note 1 |
|  | DCS1800 or PCS1900 | -47 | 100 kHz |  |
|  | Other co-existence system type | -52 | 1 MHz |  |
| Frequency range of co-existence uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100 kHz | For NR operating band, if BS uplink operating band or BS downlink operating band full covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band except n28, n67, n68, n1, n2, and n5, if BS downlink operating band or BS uplink operating band partially covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band except n29 and n28, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence uplink operating band is less than or equal to [4MHz], this requirement does not apply. Note 1, Note 2, Note 3, Note 4, Note 5, Note 6, Note 7,Note 8. |
|  | Other co-existence system type | -49 | 1 MHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. NOTE 5: For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz. NOTE 6: For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz. NOTE 7: For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz. NOTE 8: For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz. |  |  |  |  |

As above analysis, the new format table can simplify the co-existence/co-location requirement, but for other readers, e.g., product design engineers, it is not very straightforward and convenient. For other readers, they first need to distinguish between frequency gap, full overlapping, partial overlapping, and then check special notes at end of table, this process is also a bit complicated.

If we don’t remove the existing co-existence/co-location requirement, and adding co-existence/co-location requirement for Other co-existence system type not listed above in end of co-existence requirement table can reduce significant number of CRs when new frequency band is introduced. For this approach to simplify co-existence/co-location requirement, we do not need to delete existing notes in the co-existence and co-location requirement clauses. For example is shown as below:

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
|  | 921 – 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to BS operating in band n8 |
| GSM900 | 876 – 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. |
| … | … | … | … | … |
|  | … | … | … | … |
| E-UTRA Band 111 | 1820 – 1830 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
|  | 1800 – 1810 MHz | -49 dBm | 1 MHz |  |
| Other co-existence system type not listed above | Frequency range of co-existence downlink operating band | -52dBm | 1 MHz | For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence downlink operating band is less than or equal to [5MHz], or BS downlink operating band full covers frequency range of co-existence downlink operating band, or BS downlink operating band partially covers frequency range of co-existence downlink operating band (including only one frequency), this requirement does not apply. |
|  | Frequency range of co-existence uplink operating band | -49dBm | 1MHz | For NR operating band, if BS uplink operating band or BS downlink operating band full covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band, if BS downlink operating band or BS uplink operating band partially covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence uplink operating band is less than or equal to [4MHz], this requirement does not apply. |

Observation 2: Don’t remove the existing co-existence/co-location requirement, and adding co-existence/co-location requirement for Other co-existence system type not listed above in end of co-existence requirement table can reduce significant number of CRs when new frequency band is introduced. For this approach to simplify co-existence/co-location requirement, we do not need to delete existing notes in the co-existence and co-location requirement clauses. A example is shown as below:

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
|  | 921 – 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to BS operating in band n8 |
| GSM900 | 876 – 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. |
| … | … | … | … | … |
|  | … | … | … | … |
| E-UTRA Band 111 | 1820 – 1830 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
|  | 1800 – 1810 MHz | -49 dBm | 1 MHz |  |
| Other co-existence system type not listed above | Frequency range of co-existence downlink operating band | -52dBm | 1 MHz | For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence downlink operating band is less than or equal to [5MHz], or BS downlink operating band full covers frequency range of co-existence downlink operating band, or BS downlink operating band partially covers frequency range of co-existence downlink operating band (including only one frequency), this requirement does not apply. |
|  | Frequency range of co-existence uplink operating band | -49dBm | 1MHz | For NR operating band, if BS uplink operating band or BS downlink operating band full covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band, if BS downlink operating band or BS uplink operating band partially covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence uplink operating band is less than or equal to [4MHz], this requirement does not apply. |

- Conclusion

This contribution provides analysis on framework simplification for co-location/co-existence requirements, and the following proposal is concluded as follows:

Observation 1: The following table can be used to simplify the co-existence requirement in TS 38.104.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Frequency range of co-existence downlink operating band | GSM850, CDMA850 or GSM900 | -57 | 100 kHz | For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence downlink operating band is less than or equal to [5MHz], or BS downlink operating band full covers frequency range of co-existence downlink operating band, or BS downlink operating band partially covers frequency range of co-existence downlink operating band (including only one frequency), this requirement does not apply. Note 1 |
|  | DCS1800 or PCS1900 | -47 | 100 kHz |  |
|  | Other co-existence system type | -52 | 1 MHz |  |
| Frequency range of co-existence uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | 100 kHz | For NR operating band, if BS uplink operating band or BS downlink operating band full covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band except n28, n67, n68, n1, n2, and n5, if BS downlink operating band or BS uplink operating band partially covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band except n29 and n28, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence uplink operating band is less than or equal to [4MHz], this requirement does not apply. Note 1, Note 2, Note 3, Note 4, Note 5, Note 6, Note 7,Note 8. |
|  | Other co-existence system type | -49 | 1 MHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 or NR Band n68 UL operating band, where requirement applies for between 698 MHz and 703 MHz NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. NOTE 5: For BS operating in Band n68, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n109 UL operating band, where requirement applies for 728 MHz to 733 MHz. NOTE 6: For BS operating in Band n2, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXV/25/n25 UL operating band, where requirement applies for 1910 MHz to 1915 MHz. NOTE 7: For BS operating in Band n5, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band XXVI/26/n26 UL operating band, where requirement applies for 814 MHz to 824 MHz. NOTE 8: For BS operating in Band n1, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 65/n65 UL operating band, where requirement applies for 1980 MHz to 2010 MHz. |  |  |  |  |

Observation 2: Don’t remove the existing co-existence/co-location requirement, and adding co-existence requirement for Other co-existence system type not listed above in end of co-existence requirement table can reduce significant number of CRs when new frequency band is introduced. For this approach to simplify co-existence/co-location requirement, we do not need to delete existing notes in the co-existence and co-location requirement clauses. A example is shown as below:

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
|  | 921 – 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to BS operating in band n8 |
| GSM900 | 876 – 915 MHz | -61 dBm | 100 kHz | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band n8, since it is already covered by the requirement in clause 6.6.5.2.2. |
| … | … | … | … | … |
|  | … | … | … | … |
| E-UTRA Band 111 | 1820 – 1830 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
|  | 1800 – 1810 MHz | -49 dBm | 1 MHz |  |
| Other co-existence system type not listed above | Frequency range of co-existence downlink operating band | -52dBm | 1 MHz | For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence downlink operating band is less than or equal to [5MHz], or BS downlink operating band full covers frequency range of co-existence downlink operating band, or BS downlink operating band partially covers frequency range of co-existence downlink operating band (including only one frequency), this requirement does not apply. |
|  | Frequency range of co-existence uplink operating band | -49dBm | 1MHz | For NR operating band, if BS uplink operating band or BS downlink operating band full covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band, if BS downlink operating band or BS uplink operating band partially covers frequency range of co-existence uplink operating band, this requirement does not apply. For NR operating band, if the adjacent frequency gap between BS downlink operating band and frequency range of co-existence uplink operating band is less than or equal to [4MHz], this requirement does not apply. |

- References

- R4-2508664, Way forward for [115][340] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#115

- R4-2504741, Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#114-bis

- R4-2502306, Way forward for [114][331] BSRF_Simplify_CoLo_Coex, Moderator (Ericsson), RAN4#114

- RP-243288, Moderator's summary of discussion about R19-RAN4-BSRF, RAN4 Chair (Huawei), RAN#106

- R4-2500102, On RAN task to work on framework simplification for co-location/co-existence requirements, Nokia

- R4-2500880, On the topic of BS RF specification structure improvements, Ericsson

- RP-242767, On new objective under NR_BS_RF_req_evo, Nokia, Ericsson, ZTE, Huawei, HiSilicon, RAN#106