3GPP TSG-RAN WG4 Meeting #116                                                                                                         R4-2509347

Bengaluru, India, 25th –29th August, 2025

Source: 	CATT

Title: 	Discussion on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band

Agenda item:	5.29.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band.

- Discussion

The Table 7.2.2-1 of TS 38.108 is copied as below:

Table 7.2.2-1: SAN GEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -99.3 |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -99.3 (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -99.4 |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -96.5 |
| 20 | 15 | G-FR1-NTN-A1-4 | -92.9 |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -92.3 (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 | -93.2 |
| 20 | 60 | G-FR1-NTN-A1-6 | -93.3 |
| Note 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. Note 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. Note 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. Note 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

As above table, the PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) is -92.3 (Note 2), which is reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band,  PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-4 is -92.9, they are different in TS 38.108.

The test requirement for above core requirement in Table 7.5.2-1 of TS 38.181 is copied as below:

Table 7.2.5-1: SAN GEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -98.6 |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -98.6 (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -98.7 |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -95.8 |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -92.2 |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -92.2 (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -92.5 |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -92.6 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

As above, in TS 38.181, the PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) is -92.2 (Note 2), which is reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band,  PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-4 is -92.2,  they are same in TS 38.181.

So, the -92.3 (Note 2) for PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) in TS 38.108 is incorrect, and it should be -92.9.

Observation 1: The PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) in TS 38.108 should be -92.9 instead of -92.3.

As above observation, we propose,

Proposal 1: Update PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) in TS 38.108 from -92.3 to -92.9.

Based on above proposal, two CR for TS38.108 on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band are provided for approval.

- Conclusion

This contribution provides analysis on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band. The following observation and proposal is concluded as follows:

Observation 1: The PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) in TS 38.108 should be -92.9 instead of -92.3.

Proposal 1: Update PREFSENS for 20MHz CBW and 15kHz SCS with G-FR1-NTN-A1-11 (Note 4) in TS 38.108 from -92.3 to -92.9.

Based on above proposal, two CR for TS38.108 on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band are provided for approval.

- References

- R4-2502348, (TEI18) CR to 38.108 NB-IoT NTN inband operation with NR NTN [NTNNBIoT_inbandNTNNR], Inmarsat, Viasat, EchoStar, Thales, RAN4#114

- R4-2509348, (TEI18)CR for TS38.108, Correction on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band[NTNNBIoT_inbandNTNNR],CATT, RAN4#116

- R4-2509349, (TEI18)CR for TS38.108, Correction on reference sensitivity levels requirement apply to SAN that supports NB-IoT operation in NTN NR in-band[NTNNBIoT_inbandNTNNR],CATT, RAN4#116