3GPP TSG-RAN WG4 Meeting #116                                                                                             R4-2509359

Bengaluru, IN, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on RF requirements for NTN RedCap UE

Agenda item:	7.27.3.1

Document for:	Approval

- Introduction

In RAN4 #115 meeting, A WF [1] on NR NTN phase 3 UE RF requirements was agreed. However, there are still some remaining issues about NR NTN UE requirements need to be discussed in the next meeting.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE requirements.

- Discussion

The agreement on NTN RedCap UE RF requirements was achieved during the previous meetings, as shown below:

## Topic 1: Output power enhancement for NR NTN PC3 Agreements: - Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting - FFS for details, capabilities will be needed for feature list - When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations] - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost - Boost is 0.5 dB - FFS on duplex modes - For information only: HD-FDD/FD-FDD support is indicated per band - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting - Rel-18 and Rel-19 power boosting schemes can be simultaneously supported - FFS for details - PC3 ACLR requirements apply for power boosted transmissions - Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5 ## Topic 3: UL capacity enhancements Agreements: - As a starting point, re-use DMRS-bundling requirement values and side conditions - Add “No UE specific and common TA updates” to side conditions - FFS for further updates based on inputs in next meeting - UE needs to be able to indicate OCC2/4 feature and/or OCC duration support for GSO deployment scenario only, for NGSO deployment scenario only, or for both GSO and NGSO deployment scenario - Further check and optimize alignment with RAN1 capabilities # Topic 4: DL coverage enhancements Agreements: - Discuss updates to sync raster due to impact of cell search delay with 160ms SSB periodicity in next meeting - Encourage companies to provide detailed proposal on sync raster update - Note: also 3 MHz is being introduced in rel-19

At the last meeting, RAN4 agreed that the Rel-18 power boosting feature could be applicable for NR NTN RedCap UE. Meanwhile, new UE capabilities will be introduced for NTN RedCap UE to indicate support for power boosting. Considering that for CP-OFDM and DFT-s-OFDM 16/64 QAM, the power boosting is implementation based, then for the corresponding case, there is no need to introduce extra UE capabilities.

Observation 1: For NTN (e) RedCap UE, the implementation based modulation does not require the introduction of extra UE capabilities.

The applicable duplex mode of NTN RedCap UE power boosting was discussed at last meeting as well. Since support for HD-FDD/FD-FDD is indicated per band, power boosting could be applied to both HD-FDD and FD-FDD, i.e. the network can indicate whether each operating band is boosted as needed.

Observation 2: For NTN (e) RedCap UE, the applicability of power boosting is not restricted by duplex mode.

In addition, according to the power boosting scheme discussed at the last meeting, the Rel-18 and Rel-19 power boosting scheme could be simultaneously supported. Considering that NTN RedCap UE's Rel-18/19 power boosting scheme is slightly different (the Rel-19 power boosting benefits from IL reduction), the UE capabilities and the corresponding UE features may also different. Therefore, for Rel-19 scheme power boosting, different UE capabilities should be defined, and the UE feature should be introduced separately.

Observation 3: For NTN (e) RedCap UE that support Rel-19 scheme (additional) power boosting, new UE capabilities and UE features should be introduced.

Therefore, for NTN (e) RedCap UE, the power boosting related UE capabilities should be introduced as follow:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [NTN-additionalPowerBoosting-pi2BPSK-QPSK -r19] Indicates whether NTN (e) RedCap UE supports additional power boosting for DFT-s-OFDM pi/2 BPSK and QPSK by reducing insertion loss, when applicable as defined in 6.2 of TS 38.101-5 | Band | NO | NO | FR1 only |

Proposal 1: For NTN (e) RedCap UE, the UE capabilities definition should be introduced as described above for additional power boosting.

The DL coverage enhancement was also discussed during the previous meeting, the agreements are shown as below:

# Topic 4: DL coverage enhancements Agreements: - Discuss updates to sync raster due to impact of cell search delay with 160ms SSB periodicity in next meeting - Encourage companies to provide detailed proposal on sync raster update - Note: also 3 MHz is being introduced in rel-19

Generally speaking, when existing spec content cannot support the operation of related features, the spec needs to be updated. However, considering that the function of the sync raster is to place SSB to ensure that devices could synchronise correctly for initial access, based on the sync raster currently defined in TS 38.108 and TS 38.101-5, no potential issues have been observed with devices operating under 160ms SSB periodicity.

In addition, as discussions of Rel-19 are nearing completion, the update to the sync raster should be made with caution and may require extensive discussion. According to TU schedule of the WID, the NTN enhancement ph3 will be closed in September, so we believe that updating the sync raster at this point may be unnecessary.

Proposal 2: There is no need to update sync raster for NTN.

- Conclusion

This contribution provides our views on NR NTN (e) RedCap UE requirements. The following proposals are concluded as follows:

Observation 1: For NTN (e) RedCap UE, the implementation based modulation does not require the introduction of extra UE capabilities.

Observation 2: For NTN (e) RedCap UE, the applicability of power boosting is not restricted by duplex mode.

Observation 3: For NTN (e) RedCap UE that support Rel-19 scheme (additional) power boosting, new UE capabilities and UE features should be introduced.

Proposal 1: For NTN (e) RedCap UE, the UE capabilities definition should be introduced as described above for additional power boosting.

Proposal 2: There is no need to update sync raster for NTN.

- References

- R4-2508764, Way Forward for [115][309] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4# 115.