3GPP TSG RAN WG4 Meeting #116		R4-2509374

Bengaluru, India, Aug 25th – 29th, 2025

Title: 	Finalized consideration on testability for UE RF requirements for LP-WUS

Agenda Item:	7.24.3.4

Source: 	CATT

Document for:	Discussion

1 Introduction

In RAN4#115, discussions continued on testability for UE RF requirements for LP-WUS, and WF [1] was approved with the following agreements on testability:

Issue 4-1-3: skipping rule for LP-WUR different mode Agreements: - For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly. - FFS For LP-WUR support both.

For UE supporting both idle-mode and connected-mode LP-WUR, the testing skipping rule is still FFS. In this contribution, we share our views on the testing skipping rules for UE supporting both modes.

2 Discussion

As agreed in previous meetings and captured in TR 38.774 [2] clause 8.1.1, it has been agreed that same LP-WUS RF requirements will apply to all RRC states, but only one RRC state needs to be tested. For the UE supporting both idle-mode and connected-mode LP-WUR, testing under one mode is sufficiently to verify the UE’s RF capability. In this way, only testing the RRC_CONNECTED state provides following advantages over testing in RRC_IDLE or RRC_INACTIVE state:

- Conformance with legacy test frameworks: basically all the UE RF requirements are tested in RRC_CONNECTED state. Verifying the connected-mode LP-WUR performance aligns with legacy RF test methodologies.

- No need to introduce a new test mode: testing in RRC_CONNECTED state using ordinary test configurations and signalling procedures is able to verify UE’s native behaviour.

- Reduction of testing time cost: avoiding waking up MR frequently, statistics such as MDR and FAR can be stored during the test and sent via MR to TE by waking up MR at the end of test.

Observation 1: Testing LP-WUR in RRC_CONNECTED state provides advantages over in RRC_IDLE/INACTICE state such as conformance with legacy test frameworks, no need for a new test mode and reduced testing efforts etc.

Based on the above observation, we propose only to test LP-WUS performance in the RRC_CONNECTED state for UE supporting both modes.

Proposal 1: For UE supporting both idle-mode and connected-mode LP-WUR, the RF requirements for LP-WUS is tested only in RRC_CONNECTED state.

Furthermore, the main purpose of measuring LPWUS RF metrics is to verify UE LR performance. To avoid excessive testing time, the NR MR should stay in sleep mode throughout the test, with all testing statistics such as MDR and FAR buffered locally until the measurement is complete, then the MR is woken up to transmit the packed testing data to the TE.

Proposal 2: UE MR stays in sleep mode and the detection rate for LP-WUS is counted without waking up MR during the test.

Proposal 3: UE MR is woken up to send the test statistics to TE at the end of the test.

3 Conclusion

In this contribution, we submit following observation and proposals on the testability issue for UE RF requirements for LP-WUS:

Observation 1:	Testing LP-WUR in RRC_CONNECTED state provides advantages over in RRC_IDLE/INACTICE state such as conformance with legacy test frameworks, no need for a new test mode and reduced testing efforts etc.

Proposal 1:	For UE supporting both idle-mode and connected-mode LP-WUR, the RF requirements for LP-WUS is tested only in RRC_CONNECTED state.

Proposal 2:	UE MR stays in sleep mode and the detection rate for LP-WUS is counted without waking up MR during the test.

Proposal 3:	UE MR is awakened to send the test statistics to TE at the end of the test.

Reference

[1]	R4-2507936, “WF for [115][136] NR_LPWUS_UERF”, vivo

[2]	TR 38.744 v0.4.0, Technical Specification Group Radio Access Networks;Radio Frequency (RF) requirements for Low-Power Wake-up Signal and Receiver (Release 19)