3GPP TSG RAN WG4 Meeting #116		R4-2509380

Bangalore, India, Aug 25th – 29th , 2025

Title: 	Finalized consideration on REFSENS, ACS and ASCS for LP-WUS

Agenda Item:	7.24.3.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#115, the rest issues related to determine REFSENS requirement have successfully been solved, and also UE capabilities needs further discussions [1]:

Issue 2-1-1: SNR for LP-WUR two types Agreements: - There is no repetition of information bits to derive OFDM-based receiver SNR. - SNR=-4.5dB for envelope detection and OFDM-based receiver. Issue 2-1-2: Whether update the equation for REFSENS Agreements: - Use legacy REFSENS approach to derive requirements in spec. For TR, some statement of this issue may be presented. Issue 2-1-3: RMC for LP-WUR requirements Agreements: - In the FR1 spec, there should be two tables for 15kHz and 30kHz; - The following parameter is agreed. Further adding other parameter (if necessary) is not precluded. - Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements Parameter; Unit; Value. MR Channel bandwidth; MHz; All CBW. LP-WUS bandwidth; RB; 11. Subcarrier spacing; kHz; 15/30kHz. RM coding; bits; 16bits. CRC; No CRC. Chip rate; M=4 (4 chips in an OFDM symbol). overlaid OFDM sequence; Length 33: generated by 31-length ZC sequence with extension. Number of overlaid OFDM sequence per chip to carry information; 4. WUS duration for OOK; 8 OFDM symbols. WUS duration for OFDM; 2 OFDM symbols. Manchester coding for OOK; 1/2. Number of information bits; 5bits. Issue 2-1-6: UE feature for LP-WUS WF: - FFS if RAN4 will specify capability (or capabilities) for the two different types of LP-WUR.

In this contribution, we share the final REFSENS values based on the previous agreements, and our views on whether or not to specify capabilities for the two different types of LP-WUR.

# 2 Discussion

According to the previous agreements, there are two values for NF+IM: Type 1 – 18 dB, Type 2 – 13.5dB, and furthermore, SNR for both types is -4.5dB where one-shot demodulation is assumed for OFDM-based receiver. Furthermore, the following REFSENS equation illustrates how REFSENS can be derived:

REFSENS dBm= -174 dBm+10*log10NRBLP*12*SCS+NF+IM+SNR

where is the number of PRBs for LP-WUS.

Hence, we can have REFSENS as the following table for Type 1 and Type 2 receiver respectively.

|  | SCS(kHz) | REFSENS (dBm) |
| --- | --- | --- |
| Type 1 | 15 | -97.5 |
|  | 30 | -94.5 |
| Type 2 | 15 | -102.0 |
|  | 30 | -99.0 |

Proposal 1: RAN4 to specify LP-WUR REFSENS as follows: Type 1 receiver — −97.5 dBm at 15 kHz SCS and −94.5 dBm at 30 kHz SCS; Type 2 receiver — −102.0 dBm at 15 kHz and −99.0 dBm at 30 kHz.

Regarding UE capabilities, RAN1 has already introduced such capabilities:

| FG | RAN1 UE FG for LP-WUS |
| --- | --- |
| 62-1 | 62-1 LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62-1a | 62-1a LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence |
| 62-2 | 62-2 LP-WUS operation in CONNECTED mode based on OOK signal |
| 62-2a | 62-2a LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

However, these capabilities are related to base-band processing, and RAN4 needs to take into account both baseband and RF capabilities. For example, when RAN4 defines REFSENS, there are different components contributed to this requirement:

- SNR: reflect base-band processing. Coincidently, no matter operation based on OOK signal or OFDM overlaid sequence, SNR is the same as -4.5dB and it does not make any difference in both ways. Hence RAN4 cannot directly reuse these RAN1 feature groups.

- The other terms -174dBm+10*log10NRBLP*12*SCS+NF+IM reflect RF processing, and RAN4 has agreed two set of NF+IM associated with two different implementations from RF perspective. And these two types of RF hardware do not need to have 1-to-1 mapping for two base-band processing ways. Hence RAN4 needs its own corresponding features for the two types of receivers.

Observation 1: RAN1 UE feature groups for LP-WUS reflect different ways of base band processing, and cannot reflect different RF implementations.

Observation 2: RAN4 has agreed that SNR for deriving REFSENS are -4.5dB for both OOK-based and OFDM-based receivers, thus makes no difference from RF perspective.

Observation 3: From RAN4 perspective, there is no need to mandate 1-to-1 mapping between baseband and RF capabilities for LP-WUR.

Proposal 2: RAN4 to introduce UE feature groups to reflect two types of receivers for LP-WUS.

# 3 Conclusion

In this contribution, we have the following observations and proposals for REFSENS for LP-WUS:

Observation 1: RAN1 UE feature groups for LP-WUS reflect different ways of base band processing, and cannot reflect different RF implementations.

Observation 2: RAN4 has agreed that SNR for deriving REFSENS are -4.5dB for both OOK-based and OFDM-based receivers, thus makes no difference from RF perspective.

Observation 3: From RAN4 perspective, there is no need to mandate 1-to-1 mapping between baseband and RF capabilities for LP-WUR.

Proposal 1: RAN4 to specify LP-WUR REFSENS as follows: Type 1 receiver — −97.5 dBm at 15 kHz SCS and −94.5 dBm at 30 kHz SCS; Type 2 receiver — −102.0 dBm at 15 kHz and −99.0 dBm at 30 kHz.

Proposal 2: RAN4 to introduce UE feature groups to reflect two types of receivers for LP-WUS.

# Reference

[1] R4-2507936, “WF for [115][136] NR_LPWUS_UERF”, vivo