3GPP TSG-RAN WG4 Meeting #116           	R4-2509397

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.28.6

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for IoT NTN

Document for:	Discussion

- Introduction

In the RAN#107 meeting, the revised WI was endorsed to enhance the NTN for IoT with the following objectives， which can be categorized as following

- Objective 1: Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload

- Objective 2: Support of Capacity enhancements for uplink

- Objective 3: Support broadcase of PWS messages of NB-IoT re-using the LTE mechanisms

In the last meeting,   the test scope and test setup for IoT NTN phase 3 was discussed, the related agreement was captured into WF in [1] as following

- SAN demodulation requirements - NPUSCH format 1 demodulation requirements - Whether to define NPUSCH format 1 demodulation requirements with OCC for Rel-19 IoT NTN - Introduce NPUSCH format1 demodulation requirement with OCC for Rel-19 IoT-NTN - SCS - Both 15KHz and 3.75KHz SCS with OCC - Number of allocated subcarriers - Prioritize the single tone for specifying NPUSCH format 1 requirement with OCC - OCC length - OCC length = 2 - DMRS pattern - CDM DMRS is assumed for 15kHz SCS OCC and TDM work assumption is assumed for 3.75kHz SCS OCC - RAN4 needs to follow RAN1’s conclusion of DMRS sequence mapping for TDM DMRS slots. - Antenna configuration - 1Rx and 2Rx - Number of repetitions - 2 for single tone with 3.75kHz and 8 for single tone with 15kHz - MCS - BPSK MCS 0 in Table 16.5.1.2-1 of TS 36.213 - Propagation conditions - Only consider NTN TDLA100-1 - Frequency offset - 128Hz for 3.75kHz SCS and 15kHz SCS. - Tx Duration - 256 for 3.75kHz SCS and 128 for 15kHz - Test metric - 70% maximum throughput - NPRACH demodulation requirements - Whether to define NPRACH demodulation requirements for Rel-19 IoT-NTN - No need to define NPRACH demodulation requirements for Rel-19 IoT-NTN.

In this contribution, the view on  remainig issue of test setup on  NPUSCH1 demodualtion requirement is provided. Meanwhile, the initial simulation results are provided for alignment purpose.

# 2	Discussion

In this section, the view on remaining issue of test setup for NPUSCH format 1 demodulation requirement

DMRS pattern for OCC

Regarding the DMRS pattern for OCC, based on RAN1 agreement, as for 15KHz SCS, it was agreed that DMRS symbols are spread before OCC is applied. according to the formula:

ru,OCCMn+m=runqm, 0≤n<X/M, m=0, …, M-1

As for 3.75KHz, the TDM pattern was agreed for 3.75KHz SCS in RAN1, RAN4 needs to follow RAN1’s conclusion of DMRS sequence mapping for TDM DMRS slots.

Agreement Confirm the following working assumption from RAN1#120 Athens: “For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.” Agreement For 3.75kHz SCS OCC for NPUSCH format 1, the following mappings between DMRS sequence samples and active TDM DMRS slots is applied: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots

Proposal 1: RAN4 to define NPUSCH 3.75KHz SCS with TDM DMRS configuration based on RAN1 agreement

Number of Repetition

Regarding the number of repetitions, Due to the OCC operation, the actual number of transmissions will be extended. To maintain the same length compared to existing NPUSCH requirement s without OCC feature, RAN4 has agreed the number of repetitions for both 3.75KHz and 15KHz single tone scenarios for inter-symbol and inter-slot OCC feature

Agreement - 2 for single tone with 3.75kHz and 8 for single tone with 15kHz

Meanwhile, as agreed in RAN1, the total number of slots in NPUSCH transmission after OCC is following

Agreement The total number of slots in the NPUSCH transmission after OCC is applied is MrepNPUSCHNslotsULNRU , where the parameters have the legacy definitions in TS36.211 and MrepNPUSCH≥2.

The number of repetitions agreed in RAN4 should be the number of transmissions before OCC operation. Since the parameters of MrepNPUSCH has the legacy definition for OCC feature, it has already included the number of slots extension with OCC processing. Therefore, to align existing specification related with the configuration of repetitions, we suggest to configure 4 for single with 3.75KHz and 16 for single tone with 15KHz in the specification, although the actual repetition transmission before OCC is 2 and 8, respectively. Some additional clarification or note is needed.

Observation 1: The total number of slots in NPUSCH transmission is defined in RAN based on after OCC operation, while the number of repetitions agreed in RAN4 should be the number of transmissions before OCC operation

Proposal 2: RAN4 should discuss how to capture the number of repetitions into specification when CR drafting stage.

Power difference between two UEs

Since the purpose of inter-slots OCC is to support multi-UE transmission, according to RAN1 agreement, the length of OCC is 2, which means two UEs are modeling when defining the corresponding results. For the power control, RAN1 has assumed the same transmit power from different UEs, which simulates the similar receiving power at gNB. Therefore, for simplicity, we assume that there is no power difference between two UEs for specifying the NPUSCH requirement with OCC feature.

Proposal 3: Assuming the same transmission power for two UEs when defining the corresponding NPUSCH requirement with supporting inter-slots OCC.

Test metric

In the last meeting, the SNR with 70% of maximum throughput was agreed as the test metric, which is widely applied for specifying the requirement. For IoT NTN with inter-slot OCC feature, two UEs will be configured for transmission. The achievable maximum throughput should be double compared with single UE scenario. When calculating the SNR value, RAN4 should clarify whether it is based on maximum throughput for each UE or the total maximum throughput of total UEs.

Proposal 4: RAN4 should clarify whether the SNR value is calculated based on 70% of maximum throughput for each UE or the total maximum throughput of total UEs.

FRC

In the last meeting, MCS 0 was agreed for specifying the requirement of NPUSCH with OCC feature. And the FRC table used for 3.75KHz and 15KHz was agreed as following

Table 1: FRC parameters for NB-IoT NPUSCH format 1 in TS 36.108

| Reference channel | A7-1 |
| --- | --- |
| Subcarrier spacing (kHz) | 3.75 |
| Number of allocated subcarriers | 1 |
| Diversity | No |
| Modulation | BPSK |
| ITBS / IRU | 0 / 1 |
| Payload size (bits) | 32 |
| Allocated resource unit | 2 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.29 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Channel estimation length (ms) Note 1 | 16 |

Table 2: FRC parameters for NB-IoT NPUSCH format 1 in TS 36.104

| Reference channel | A16-1 | A16-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 1 |
| Diversity | No | No |
| Modulation | BPSK | BPSK |
| ITBS / IRU | 0 / 1 | 0 / 1 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Channel estimation length (ms) Note 1 | 16 | 4 |

For 15KHz single tone case, since we have agreed the number of Tx duration is 128ms, considering the agreed number of repetitions as 8, and the configured IRU as 1 indicated in A16-2, where the number of RU is 2. Then, the Tx duration is 8*2*2*16*0.5=256ms

Observation 2: Based on agreed FRC table and number of repetitions for 15KHz SCS single tone scenario, the Tx duration should be 256ms instead of 128ms.

Therefore, in case RAN4 agree to define the Tx duration is 128ms for 15KHz SCS single scenario, then, RAN4 should define the new FRC table f 15KHz, with updating the number IRU as 0 for meet the number of Tx duration as 128m. The updated FRC table is proposed as following table 3

Table 3: FRC parameters for NB-IoT NPUSCH format 1 15KHz, with single tone

| Reference channel | A16-x |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 1 |
| Diversity | No |
| Modulation | BPSK |
| ITBS / IRU | 0 / 0 |
| Payload size (bits) | 16 |
| Allocated resource unit | 1 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.42 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Channel estimation length (ms) Note 1 | 4 |

Another solution is to update the Tx duration as 256ms with reusing the current FRC table as (A16-2) and assumption of number of repetitions.

Proposal 5: RAN4 should define  the new FRC table for 15KHz, with updating the number I_RU as 0 to  meet the number of Tx duration as 128ms,

| Reference channel | A16-x |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 1 |
| Diversity | No |
| Modulation | BPSK |
| ITBS / IRU | 0 / 0 |
| Payload size (bits) | 16 |
| Allocated resource unit | 1 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.42 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 96 |
| Total symbols per resource unit | 96 |
| Channel estimation length (ms) Note 1 | 4 |

or update the Tx duration as 256ms with reusing the current FRC table and assumption of number of repetitions.

Frequency offset

Regarding the frequency offset, in the last meeting, the value was agreed as 128Hz for both 3.75KHz and 15KHz.

Regarding the frequency offset setting, during the Rel-18 discussion, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of Nsegmentprecompensation time units, where the same value pre-compensation will be applied at the UE side within one segment. Due to the moving of satellite, the additional frequency doppler shift offset will be introduced, which may make the dropper shift at the end of segment different with the 1st repetition. For requirement definition, RAN4 agreed to apply the largest accumulate offset with considering the Tx duration, where 128Hz was setting for 3.75KHz with total Tx duration as 256ms, and 8Hz for 15KHz with total Tx duration as 16ms.

Observation 3: RAN4 has specified the NPUSCH requirement with assuming the largest accumulate offset with 128Hz for 3.75KHz with total Tx duration as 256ms, and 8Hz for 15KHz 12 tones with total Tx duration as 16ms.

Considering the Tx duration, RAN4 has agreed 256ms for 3.75kHz and 128ms for 15KHz.  As discussed, the issue in FRC, we may need to refine the previous agreement for 15KHz. If considering the Tx duration of 15KHz as 128ms, then, the frequency offset for 15KHz should be updated as 64Hz. Otherwise, the Tx duration of 15KHz should be updated as 256ms assuming the frequency offset is 128Hz.

Proposal 6: RAN4 should further align the Tx duration and frequency offset assumption for 15KHz SCS. If considering the Tx duration of 15khZ SCS as 128ms, the frequency offset for 15KHz should be updated as 64Hz. Otherwise, the Tx duration of 15KHz should be updated as 256ms assuming the frequency offset is 128Hz.

# 3	Simulation Results

In this section, based on agreed test setup, the initial simulation results are provided for alignment purpose.

Table 4: Initial simulation results for 3.75KHz with supporting inter-symbol OCC, single tone

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 4 without OCC | 70% | -3.77 |
|  | 0 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 4 without OCC | 70% | -7.11 |
| 1 | 0 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -2.6 |
|  | 1 | 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -2.58 |
| 2 | 0 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -5.62 |
|  | 1 | 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 128 Hz | 256 | 2 before OCC | 70% | -5.58 |

Table 5: Initial simulation results for 15KHz with supporting inter-slot OCC, single tone

| Case | UE | Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 64Hz | 128 | 16 without OCC | 70% | -6.33 |
|  | 0 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 64Hz | 128 | 16 without OCC | 70% | -10.06 |
| 1 | 0 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128Hz | 256 | 16 without OCC | 70% | -8.21 |
|  | 0 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128Hz | 256 | 16 without OCC | 70% | -11.36 |
| 2 | 0 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 64H | 128 | 8 before OCC | 70% | -6.66 |
|  | 1 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 64HZ | 128 | 8 before OCC | 70% | -6.57 |
| 3 | 0 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 64HZ | 128 | 8 before OCC | 70% | -9.73 |
|  | 1 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-x | 64HZ | 128 | 8 before OCC | 70% | -9.9 |
| 4 | 0 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128H | 256 | 8 before OCC | 70% | -8.48 |
|  | 1 | 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128H | 256 | 8 before OCC | 70% | -8.52 |
| 5 | 0 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128H | 256 | 8 before OCC | 70% | -11.53 |
|  | 1 | 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 128H | 256 | 8 before OCC | 70% | -11.54 |

For 3.75KHz SCS, there is around 1-2dB performance degradation compared with single UE without OCC operation, considering the DMRS for each UE is TDM allocation, it results in the performance degradation of channel estimation.

For 15KHz SCS, the performance degradation due to OCC operation is minor compared with single UE without OCC operation.

Meanwhile, compared with 2Rx configuration, around 3dB performance degradation was observed for 1Rx configuration.

Compared with the existing FRC, where 2RU is configured, there is about 2dB performance degradation for the updated FRC table in 15KHz SCS.  Since only 1 RU is configured and the coding rate is high compared with 2RU configuration, therefore, less coding gain is achieved for 1RU configuration.

Observation 4: For 3.75KHz SCS, with OCC operation, around 1-2dB performance degradation compared with single UE without OCC operation.

Observation 5: For 15KHz SCS, with OCC operation, minor performance degradation compared with single UE without OCC operation

Observation 6: Compared with 2Rx configuration, around 3dB performance degradation was observed for 1Rx configuration

Observation 7: Compared with the existing FRC table, where 2RU is configured, there is about 2dB performance degradation for the updated FRC table in 15KHz SCS

# 4	Conclusion

In this contribution, the view on remaining issue of test setup for NPUSCH demodulation requirement with supporting inter-slot OCC. Meanwhile, the initial simulation results are provided for alignment purpose.

Proposal 1: RAN4 to define NPUSCH 3.75KHz SCS with TDM DMRS configuration based on RAN1 agreement

Number of Repetition

Observation 1: The total number of slots in NPUSCH transmission is defined in RAN based on after OCC operation, while the number of repetitions agreed in RAN4 should be the number of transmissions before OCC operation

Proposal 2: RAN4 should discuss how to capture the number of repetitions into specification when CR drafting stage.

Power difference between two UEs

Proposal 3: Assuming the same transmission power for two UEs when defining the corresponding NPUSCH requirement with supporting inter-slots OCC.

Proposal 4: RAN4 should clarify whether the SNR value is calculated based on 70% of maximum throughput for each UE.

Observation 2: Based on agreed FRC table and number of repetitions for 15KHz SCS single tone scenario, the Tx duration should be 256ms instead of 128ms.

Proposal 5: RAN4 should create the new FRC table for 15KHz, with updating the number I_RU as 0 for meet the number of Tx duration as 128ms, or update the Tx duration as 256ms with reusing the current FRC table and assumption of number of repetitions.

Observation3: RAN4 has specified the NPUSCH requirement with assuming the largest accumulate offset with 128Hz for 3.75KHz with total Tx duration as 256ms, and 8Hz for 15KHz 12 tones with total Tx duration as 16ms.

Proposal 6: RAN4 should further align the Tx duration and frequency offset assumption for 15KHz SCS. If considering the Tx duration of 15khZ SCS as 128ms, the frequency offset for 15KHz should be updated as 64Hz. Otherwise, the Tx duration of 15KHz should be updated as 256ms assuming the frequency offset is 128Hz.

Observation 4: For 3.75KHz SCS, with OCC operation, around 3dB performance degradation compared with single UE without OCC operation.

Observation 5: For 15KHz SCS, with OCC operation, minor performance degradation compared with single UE without OCC operation

Observation 6: Compared with 2Rx configuration, around 3dB performance degradation was observed for 1Rx configuration

Observation 7: Compared with the existing FRC table, where 2RU is configured, there is about 2dB performance degradation for the updated FRC table in 15KHz SC

# References

RP-250472, Revised WID on NTN for internet of Things (IoT) phase 3, MTK

R4-2508756, Way Forward for [115][334] IoT_NTN_Ph3