3GPP TSG-RAN WG4 Meeting #116           	 R4-2509398

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.27.7

Source:	Samsung

Title:	Discussion and simulation results on demodulation requirement for NR NTN

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WI was endorsed to enhance the NTN for NR with the following objectives， which can be categorized as following [1]

- Objective 1: Downlink coverage enhancements

- Objective 2: Uplink Capacity/Cell Throughput Enhancement

- Objective 3: Signalling of intended service area of a broadcast service

- Objective 4: Support of regenerative payload

- Objective 5: Support RedCap UEs with NR NTN operating in FR1-NTN bands

In the las meeting, the test scope and test setup for NR NTN phase 3 was discussed, the related agreement was captured into the WF in [2] as following

- SAN demodulation requirements - RedCap/eRedCap - No SAN demodulation requirements for the support of (e)RedCap UEs - Uplink capacity/cell throughput enhancements - PUSCH requirement for UL coverage enhancements - Define SAN PUSCH demodulation requirements for DFT-s-OFDM with orthogonal cover codes - OCC parameters - Apply inter-slot OCC length 2 - Apply PUSCH type A repetitions - FFS whether to define PUSCH requirements with inter-slot OCC length 4 - Detailed PUSCH parameters - Assume to set the following parameters - FDD SCS=15KHz with 5MHz CBW and FDD SCS=30KHz with 10MHz CBW - Both 1Rx and 2Rx, with test applicability rule for different Rx number - Same MCS for two UEs for PUSCH with OCC - Same channel model for two UEs for PUSCH with OCC - Frequency offset between two UEs - Baseline assumption: same frequency offset for two UEs in one OCC group. FFS what frequency offset value is assumed. - FFS whether to consider different frequency offsets values for two UEs additionally.

In this contribution, the view on remaining issue of test setup on NR NTN PUSCH demodulation requirement is provided. Meanwhile, the initial simulation results are provided for alignment purpose.

# 2	Test Setup

In this section, the view on the remaining issue of test setup for NR PUSCH requirement was provided.

OCC length

Both OCC length 2 and OCC length 4 are supported for multiple UE multiplexing. In the last meeting, inter-slot OCC length with 2 was agreed, and RAN4 will further discuss whether to define PUSCH requirements with inter-slot OCC length 4. As agreed in RAN1, there would be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Given OCC length 4 is the additional UE capability, and there is no reference performance with repetition 4, we think considering OCC length as 2 should be enough to verify the baseband processing of supporting inter-slot OCC feature.

Meanwhile, based on current NTN channel model, where 200Hz doppler was considered, the channel within 4 OCC length may be varying, it is not proper to support 4 UEs transmission with OCC length 4, since the channel coherence time is small. With combining the copies from neighbor slots, the resulting PUSCH BLER performance will be degraded.

Therefore, to simply the test and align with the repetition requirement in existing NR NTN scenario, we prefer to only consider OCC length as 2 for specifying PUSCH requirement with OCC.

Proposal 1:  Only consider OCC length as 2 for specifying PUSCH requirement with OCC

Number of OCC groups and Number of repetitions

According to RAN1 agreement as following

118bis agreement - For RV cycling for OCC with PUSCH - For inter-slot OCC for OCC length 2 and for inter-slot OCC length 4 in option 1 - Same RV value is used in one OCC group 119 agreements - For RV cycling for OCC with DG-PUSCH, the following are considered - Option 1: RV cycling is used across OCC groups - Note 1: RV cycling is applied when the number of repetitions is greater than OCC length

According to RAN1 agreement, if the OCC length is same as number repetitions, only one OCC group is available with same RV value. If the number of repetitions is greater than OCC length, RV cycling is applied. The number of OCC groups depends on the number of repetitions.

Regarding the number of repetitions, up to 32 number of repetitions can be supported. In current NTN, 2 is configured to specify the requirement for PUSCH repetition for both 15KHz and 30KHz SCS. And 4 and 8 slots are configured for requirements for PUSCH with joint channel estimation, in 15KHz and 30KHz, separately.   As staring point, we prefer to configure number of repetitions as 2.

For different RV across OCC group, the RV combination has been verified with repetition transmission. While we are also open whether other values will be considered, in order to verify the RV cycling across OCC groups as following

Figure 1:  RV group to support inter-slot OCC

Proposal 2: Consider one OCC group, and number of repetitions with 2 as starting point, the initial RV can be configured as 0. FFS on considering two OCC groups with number of repetitions as 4 to verify the RV cycling function across OCC groups, the initial RV value for two OCC group 0, 2

BW

According to RAN1 evaluation, the small number of RB was selected, such as 1 PRB, 2 PRBs, considering the main targeting scenario is VoIP and low data rate service.  In the last meeting, RAN4 agreed that only the minimum number of channel bandwidth for each SCS is considered.  Although large RB was used, there is no different from baseband processing perspective. Therefore, we think the minimum number of channel bandwidth can still meet the test purpose.

Proposal 3: Only consider the minimum number of channel bandwidth for each SCS for specifying PUSCH requirement with OCC

- 15KHz, 5MHz

- 30KHz, 10MHz

Frequency offset Assumption

One of key factor to enable inter-slots OCC is that UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH. Based on RAN1 evaluation, the value of frequency offset is uniform selected from [-0.1 ppm, +0.1ppm] for modeling impairments for link level evaluation as following

Agreement Adopt the table below for assumptions for modelling impairments for link level evaluation in NR NTN UL capacity and throughput enhancements FO; Reported by companies - Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. - Optional: with lower maximum residual FO, to be reported by companies.

For NTN scenario, the baseline assumption is that UE can do the pre-compensation. Then, the residual frequency offset is around 0.1ppm. Based on this value, the doppler spread with 200Hz was considered during Rel-17 NTN channel discussion. Therefore, the channel model has already included the impact of frequency offset. Given that, we would like to configure the frequency offset is 0Hz for two UEs as baseline. We also check the performance with the non-zero frequency offset and different frequency offset for two UEs, the performance will be degraded, since the cross-slot OCC will be very sensitive to Doppler and frequency error, it will impact the orthogonality across slots. We are also open to further discuss the frequency offset assumption.

Proposal 4: Configure the same frequency offset as 0Hz for two UEs.

Channel Model for two UEs

Regarding the channel model, as agreed, the same channel model for two UEs for PUSCH with OCC was considered. For NR NTN PUSCH requirement with DFT-s-OFDM waveform, both NTN-TDLA100-200 and NTN-TDLC5-200 was considered. While, the NR NTN PUSCH requirement with repetition Type A, only NTN-TDLA100-200 was considered.

Based on RAN1 evaluation, the channel model with NTN-TDLC rural, was selected. Therefore, we would like to use the NTN-TDLC5-200 for specifying the requirement of PUSCH with inter-slot OCC.

Proposal 5: Select the channel model with NTN-TDLC5-200 for specifying PUSCH requirement with OCC feature.

MCS selection and MCS table for two UEs

As agreed in the last meeting, the same MCS for two UEs will be applied for specifying the PUSCH requirement. Based on RAN1 evaluation, low data rate and small number of RB was assumed targeting VoIP and low data rate services. Meanwhile, the MCS table in Table 6.1.4.1-2 was selected. Therefore, we would like to choose the small MCS without configuring pi2BPSK and 26QAM, the existing MCS (MCS 5 with coding rate 99/1024) in low 'qam64LowSE' MCS table for repetition type A transmission requirement can be used as starting point.

Table 6.1.4.1-2: MCS index table 2 for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 60/q | 0.0586 |
| 1 | q | 80/q | 0.0781 |
| 2 | q | 100/q | 0.0977 |
| 3 | q | 128/q | 0.1250 |
| 4 | q | 156/q | 0.1523 |
| 5 | q | 198/q | 0.1934 |
| 6 | 2 | 120 | 0.2344 |

Proposal 6: Configure the MCS 5 in MCS index table 2 (Table 6.1.4.1-2) without considering pi2BSK to specify the PUSCH requirement with inter-slot OCC

DMRS port configuration for two UEs

Regarding the DMRS port configuration for two UEs, since only two UEs are modeling for inter-slot OCC feature, for simplicity, we would like to configure the DMRS port {0} for UE 0, and port {1} for UE 1 as baseline.

Regarding the frequency hopping and sequence hopping, similar as existing requirement for DFT-s-OFDM waveform, they are disabled.

Regarding the base sequences for DMRS sequence generation, the group number u and the base sequence number v within the group can be set as 0

Proposal 7: Configure the DMRS port {0} for UE 0, and port {1} for UE 1, Disable frequency and sequence hopping and configure the group number u and sequence number v as 0 for DMRS sequence generation.

Test Metric

Regarding the test metric, both the SNR with 70% of maximum throughput and 1% BLER were applied in NR BS demodulation requirement and RAN1 evaluation for OCC feature.  Since the UL capacity enhancement is mainly based on repetition transmission scheme, where 1% of BLER as test metric was usually used to specify the requirements with repetition transmission. Therefore, we would like to apply the 1% of BLER as the test metric, where the BLER is defined as residual BLER. For NR NTN with inter-slot OCC feature, two UEs will be configured for transmission. RAN4 should apply the 1% of BLER as test metric for specifying PUSCH requirement with inter-slot OCC feature per each UE.

Proposal 8: RAN4 apply the 1% of BLER as test metric for specifying PUSCH requirement with inter-slot OCC feature per each UE.

Mapping type

For NR NTN PUSCH, both mapping type A and mapping type B are supported, and the related requirements has been specified in Rel-17 NR NTN. Meanwhile, to reduce the test efforts, RAN4 has introduced the test applicability rule to test the requirement of mapping type A and type B. Similarly, we prefer to introduce NR NTN PUSCH requirement with inter-slot OCC feature for both mapping type A and type B, and apply the same test applicability rule of existing NTN PUSCH requirements

Proposal 9: Introduce NR NTN PUSCH requirement with inter-slot OCC feature for both mapping type A and type B, and apply the same test applicability rule of existing NTN PUSCH requirements to test.

Power difference between two UEs

Since the purpose of inter-slots OCC is to support multi-UE transmission, according to RAN1 agreement, the length of OCC is 2, which means two UEs are modeling when defining the corresponding results. For the power control, RAN1 has assumed the same transmit power from different UEs, which simulates the similar receiving power at gNB. Therefore, for simplicity, we assume that there is no power difference between two UEs for specifying the PUSCH requirement with inter-slot OCC feature.

Proposal 10: Assuming the similar receiving power for two UEs at gNB side for specifying the PUSCH requirement with inter-slot OCC feature.

Others

For other simulation, the existing test parameters for PUSCH requirement with DFT-s-OFDM waveform can be considered as reference.

Proposal 11: Considering the existing test parameters for PUSCH with DFT-s-OFDM waveform requirement as reference.

# 3	Simulation Results

In this section, based on preferred simulation assumption, the initial simulation results are provided for proper test setup feasibility checking

Table 1: Simulation Assumption for PUSCH requirement with supporting inter-slot OCC

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port | UE0 | UE1 |
|  |  | 0 | 1 |
|  | DM-RS sequence generation | U, V=0, group hopping and sequence hopping are disabled |  |
| Time domain resource assignment | PUSCH mapping type | A, B |  |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |

Table 2: Initial simulation results for 15KHz with supporting inter-slot OCC, mapping A

| CASE | UE | Tx/Rx | SCS | BW | Channel | Target BLER per UE | MCS | FO | Rep | OCC length | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1/2 | 15KhZ | 5MHz | NTN-TDLC5-200 Low | 1% | MCS5 | 0Hz | 2 | N/A | -13.5 |
| 1 | 0 | 1/2 | 15KhZ | 5MHz | NTN-TDLC5-200 Low | 1% | MCS5 | 0Hz | 2 | 2 | -13.32 |
|  | 1 |  |  |  |  |  |  | 0Hz |  |  | -13.4 |
| 2 | 0 | 1/2 | 15KhZ | 5MHz | NTN-TDLC5-200 Low | 1% | MCS5 | 50Hz | 2 | 2 | -13.38 |
|  | 1 |  |  |  |  |  |  | 50Hz |  |  | -13.36 |
| 3 | 0 | 1/2 | 15KhZ | 5MHz | NTN-TDLC5-200 Low | 1% | MCS5 | 50Hz | 2 | 2 | -12.5 |
|  | 1 |  |  |  |  |  |  | 150Hz | 2 | 2 | -12.74 |

Compared with the single UE without OCC, assuming the same frequency offset for two UEs, there is a minor performance degradation with OCC operation. While assuming different frequency offset for two UEs, we can observe about 1 dB performance degradation.

Observation 1: Compared with single UE without OCC, there is a minor performance degradation with OCC operation, assuming the same frequency offset for two UEs

Observation 2: Assuming different frequency offset for two UEs,  about 1 dB  performance degradation can be observed.

# 4	Conclusion

In this contribution, the view on the test setup for NR NTN PUSCH demodulation requirement was provided. Meanwhile, the initial simulation results are provided for proper test up feasibility checking.

Proposal 1:  Only consider OCC length as 2 for specifying PUSCH requirement with OCC

Proposal 2: Consider one OCC group, and number of repetitions with 2 as starting point, the initial RV can be configured as 0. FFS on considering two OCC groups with number of repetitions as 4 to verify the RV cycling function across OCC groups, the initial RV value for two OCC group 0, 2

Proposal 3: Only consider the minimum number of channel bandwidth for each SCS for specifying PUSCH requirement with OCC

- 15KHz, 5MHz

- 30KHz, 10MHz

Proposal 4: Configure the same frequency offset as 0Hz for two UEs.

Proposal 5: Select the channel model with NTN-TDLC5-200 for specifying PUSCH requirement with OCC feature.

Proposal 6: Configure the MCS 5 in MCS index table 2 (Table 6.1.4.1-2) without considering pi2BSK to specify the PUSCH requirement with inter-slot OCC

Proposal 7: Configure the DMRS port {0} for UE 0, and port {1} for UE 1, Disable frequency and sequence hopping and configure the group number u and sequence number v as 0 for DMRS sequence generation.

Proposal 8: RAN4 apply the 1% of BLER as test metric for specifying PUSCH requirement with inter-slot OCC feature per each UE.

Proposal 9: Introduce NR NTN PUSCH requirement with inter-slot OCC feature for both mapping type A and type B, and apply the same test applicability rule of existing NTN PUSCH requirements to test.

Proposal 10: Assuming the similar receiving power for two UEs at gNB side for specifying the PUSCH requirement with inter-slot OCC feature.

Proposal 11: Considering the existing test parameters for PUSCH with DFT-s-OFDM waveform requirement as reference.

Observation 1: Compared with single UE without OCC, there is a minor performance degradation with OCC operation, assuming the same frequency offset for two UEs

Observation 2: Assuming different frequency offset for two UEs,  about 1 dB  performance degradation can be observed.

# References

RP-240775, Non-Terrestrial Networks (NTN) for NR Phase 3,

R4-2508755, Way Forward for [115][333] NR_NTN_Ph3_demod