3GPP TSG-RAN WG4 Meeting #116            	R4-2509410

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.6.4.6

Source:	Samsung

Title:	Simulation results on NTN demodulation requirements for Less than 5MHz

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the remaining issue on the test scope for performance part of NTN for less than 5MHz was under discussion. The related agreement was captured into WF in [1] as following

SAN Demodulation performance requirements - Test Scope - No new PUSCH requirements - Define PUCCH format 2 cases for SAN - Simulation assumption for SAN requirements - The following simulation assumptions can be considered for evaluation SAN PUCCH performance with less than 5MHz - A note can be added into the table to capture the value of ‘First PRB after frequency hopping’, which is 3 Parameter; Value. Modulation order; QSPK. First PRB prior to frequency hopping; 0. Intra-slot frequency hopping; enabled. Frist PRB after frequency hopping; The largest PRB index – (Number of PRBs – 1). Number of PRBs; 9. Number of symbols; 2. The number of UCI information bits; 22. First symbol; 12. DM-RS sequence generation; NID0=0. - Applicability rules for PUCCH requirements - Introduce the following test applicability rule in TS 38.181: - For a SAN declaring to support less than 5MHz (see D.XX in table 4.6-1) a UCI BLER performance test with PUCCH Format 2 shall be tested in 3 MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 can be skipped if applicable. - Applicability rules for PRACH requirements - Option 1: Introduce applicability rules for PRACH - For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply - Option 2: No applicability rule is needed for PRACH

In this contribution, the view on remaining issue of NR NTN demodulation performance requirements for less than 5MHz was provided.  And the initial simulation result is provided for alignment purpose.

# 2	Test Scope

## 2.1	Test Scope for SAN demodulation

Test applicability for PRACH requirements

For PRACH, it was agreed that no new PRACH requirement for NR with Less than 5MHz, and adding an applicability rule that for BS supporting less than 5MHz carrier bandwidth, only test requirements relating to PRACH preamble format3 with 15KHz SCS with sequence length LRA=139, and PRACH format with 1.25kHz SCS with sequence length LRA=839 shall apply.

For SAN PRACH, similar as NR with less than 5MHz, there is no new PRACH requirement for NR NTN with less than 5MHz, to keep the consistence of NR TN and NR NTN specification, our preference is to add the similar test applicability rule for NR NTN with supporting less than 5MHz.

- For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

Proposal 1:  Adding an applicability rule for PRACH requirement targeting NR NTN SAN supporting less than 5MHz carrier bandwidth

- For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

# 3	Simulation Results

In this section, the initial simulation result is provided for PUCCH format2, based on agreed simulation assumption as following.

Table 1: Simulation assumption for PUCCH format 2 with NTN less than 5MHz

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Note: The first PRB after frequency hopping is 3 |  |

Table 2: Minimum requirement for PUCCH format 2, 15KHz SCS and 3MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex D) | SNR (dB) (BLER%1) |
| --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 7.39 |
|  | 2 | Normal | NTN-TDLA100-200 Low | 0.53 |

# 4	Conclusion

In this contribution, the view on the remaining issue on test scope for performance part of NTN for less than 5MHz was under discussion. Meanwhile, the initial simulation result is provided for alignment purpose.

Proposal 1:  Adding an applicability rule for PRACH requirement targeting NR NTN SAN supporting less than 5MHz carrier bandwidth

- For SAN supporting less than 5MHz carrier bandwidth, only test requirements relating to short RACH preamble formats with 15kHz SCS with sequence length LRA=139, and long PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply

# 5	 References

R4-2508725, Way Forward for [115][325] NR_IoT_NTN_less_than_5MHz_demod, Ericsson