3GPP TSG-RAN WG4 Meeting #116	R4-2509431

Bengaluru, India, August 25th – 29th, 2025

Source:	Apple

Title:	General aspects on AI/ML for mobility in NR

Agenda item:	7.19.2

Document for:	Discussion

Introduction

During the RAN4#112bis, the discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) will start in RAN4. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this discussion paper we overview the main objectives and general aspects of AI/ML for mobility in NR.

Discussion

Main objectives and RAN2 status

The FS_NR_AIML_Mob focuses on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made at the network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

In addition to the RAN4 objectives already presented in the Introduction section above, the following RAN2 objectives and use-cases are listed in the SID [1]:

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2]

In the SID [1], TSG RAN identified the following use-cases to be considered that are further clarified during the RAN2 meetings and reflected in the TP to the TR based on the achieved agreements [3]:

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model)

At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- RLF/HOF prediction (UE sided model)

  - Both direct and indirect approaches for RLF prediction are being discussed in RAN2.

    - Direct RLF prediction: is a method that uses AI/ML to predict whether a Radio Link Failure  is happening or the likelihood of it occurring within a specific future time window. This prediction is based on current and historical radio measurements and other relevant inputs. The output of the prediction is the probability of RLF within the given time window

    - Indirect RLF prediction: The output of the indirect prediction is the predicted SINR. Using this predicted SINR, it is possible to identify the moment when the RLF occurs without needing additional AI/ML models.

  - HOF prediction is down prioritized.

- Measurement events prediction (UE sided model).

  - Measurement event A3 is taken as starting point, other events are FFS. We also make a proposal for a beam change event prediction

  - At least indirect measurement event prediction will be studied. And direct measurement event prediction is also allowed.

The study of the use cases is driven mainly by two study goals [3]:

- The 1st study goal is to reduce measurement efforts in temporal, spatial (cell level and beam level) or frequency domain by using predicted measurements from a set of subsampled measurements (same concept with set B of beams).
The following metrics are considered:

  - MRRS	Measurement reduction rate in spatial domain

  - MRRT	Measurement reduction rate in temporal domain

- The 2nd study goal is to improve the mobility and handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

The use-cases are considered in the following dimensions: [3]

- Frequency (intra- and inter-frequency predictions):

  - FR1 to FR1 intra-frequency

  - FR2 to FR2 intra-frequency

    - Both measurements (i.e., model inputs) and temporal predictions belong to the same frequency.

  - FR1 to FR1 inter-frequency

    - Serving frequency measurements can be taken as an input/reference but the predictions are done for another frequency.

- Cell (intra- and inter-cell predictions):

  - Inter-cell case

    - It is considered that serving cell measurements can be taken as an input/reference, but the predictions are done for another cell.

      - Intra-frequency prediction based on inter-cell measurements were discussed in RAN2 but was deprioritized.

    - Cluster-based approach is also considered when measurement results from multiple cells (e.g., including serving and/or neighbouring cells) can be used as an input to the model.

      - Cluster-based approach evaluation is optional and lower priority for now.

  - Intra-cell covers only intra-frequency case

    - Both measurements (i.e., model inputs) and predictions belong to the same cell.

- Temporal domain predictions:

  - Case A: predicting measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell for both FR1 and FR2 scenario without reducing any measurement in observation window. This case is evaluated for the 2nd study goal.

  - Case B: predicting a sub set of measurement instances in temporal domain of the same cell for both FR1 and FR2 scenario evaluated for 1st study goal.

- Spatial domain predictions:

  - Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a sub set of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell

We give below some examples of possible measurement reduction configurations:

- Spatial prediction of a cell: Use cell 1’s partial Tx beam measurement results as set B to predict the cell 1’s all Tx beam results.

- Cell prediction: use cell 1’s measurement results (cell level measurement results can be based on Tx beam averaging or based on best Tx beam) as set B:

    - to predict temporal cell 1’s RSRP in future prediction window,  (temporal prediction)

    - to predict cell 2 on F2 (inter-freq cell, collocated ), (spatial/frequency prediction)

    - to predict cell 2 on F1 (intra-freq non-collocated cell),   (spatial prediction )

    - to predict cell 2 on F2 (inter-freq non-collocated cell)    (spatial/frequency prediction )

- Use cells 1,2,3 (F1) Tx beam measurement results (set B) to predict 1,2,3 ’s future Tx beams. —> cell spatial and temporal prediction,

- Use cells 1,2,3 cell measurement results to predict 1,2,3 ’s future cell RSRPs —> cell temporal prediction,

- Use cells 1,2,3 Tx beam measurement results (Set B) to predict 4,5,6 ’s Tx beams. —> Intra-freq non-collocated cell spatial prediction, (spatial prediction)

- Use cells 1,2,3 cell measurement results to predict 4,5,6 ’s cell RSRPs —>  Intra-freq non-collocated cell prediction,

- Use cells 1,2,3 Tx beam measurement results (set B) to predict  1,2,3 (F2) Tx beams —> inter-freq collocated cell spatial prediction,

- Use cells 1,2,3 cell measurement results to predict 1,2,3 ’s cell RSRPs —>  inter-freq collocated cell prediction

- Use cells 1,2,3 Tx beam measurement results (set B) to predict  4,5,6 ’s Tx beams —> inter-freq non-collocated cell spatial prediction,

The following scenarios for RRM measurement prediction use case were captured and prioritized for RAN4 evaluation in RAN4#112bis (highlighted in yellow)

Table 1: Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

From RAN4 perspective the potential benefits of measurement reductions across multiple domains are elaborated below:

- 1. Inter-Frequency Layer RRM Prediction

  - Reduced Measurement Gaps:

      - FR1: 6ms MG per 40ms => 15% DL and UL resources => Tput impact is more than 20%

      - FR2: 5ms MG per 20ms => 25% DL and UL resources => Tput impact is more than 30%

      - Measurement delay is 20ms x 24 x # of frequency layer

    - Observation: MG occupies tremendous DL/UL resources and has significant Tput impacts and measurement delay.

    - MG is an important RRM configuration and hard to be avoided in traditional way.

    - Goal: inter-frequency measurement be predicted by AI/ML based on intra-frequency measurement

  - Enhanced Handover Efficiency/Mobility: optimize mobility procedures

- 2. Temporal RRM Prediction

  - Lower Measurement Overhead: reduce the number of real-time measurements required, saving UE battery life and reducing

    - Reducing measuring samples can help to enlarge the SMTC periodicity (or measurement interval) from 20ms/40ms (typical configuration in RRM spec) to 160ms

    - Reducing measurement samples can even further relax the RRM measurement interval to KxSMTC (change in spec) periodicity (K is RRM scaling factor) , e.g enlarge measurement interval from 160ms (in current RRM spec) to 3*160ms = 480ms (K=3)

- 3. Spatial RRM prediction

  - Across Tx/Rx beams

    - Reduced Beam Search Overhead: reduce the need for frequent beam searches, especially in mmWave systems.

    - Reduce the measurement overhead for FR2: The overhead of RS used for RRM (i.e., SSB and CSI-RS) is quite high in the current NR design, e.g., when SMTC periodicity is 20ms, the related SMTC overhead in FR2 is 25%, especially for FR2 (with 5ms SSB burst). Reducing the measurement overhead will lead to:

    - Reduced scheduling restrictions. (it will result in throughput increase)

      - Reduced measurement restrictions

      - L3/L1 measurements delay reduction by minimizing the Rx beam sweeping set

      - Potential decrease of MGL, it will lead to throughput increase

RRM measurement predictions provide several advantages, including reducing measurement gaps, increasing the periodicity of SMTC windows, minimizing scheduling and measurement restrictions, decreasing measurement delays, and shortening measurement gap lengths

Differences between Spatial Domain Prediction in Mobility and BM

The AI/ML use cases for mobility in NR serve distinct purposes, especially when comparing Case 1 - FR2 Beam Prediction for BM and Case 6 - Spatial Domain Prediction for Mobility. While both use AI/ML to predict sptial aspects of the radio environment, their objectives, inputs, and deployment scopes differ significantly.

1 Purpose and Outcome of Prediction

- Case 1 (FR2 Beam Prediction) focuses on optimizing beam selection within the serving cell, typically for initial access, beam tracking, or beam refinement in FR2 (mmWave). The AI/ML model predicts the best beam (or beam set) based on immediate link conditions, with the goal of improving DL/UL link quality.

- Case 6 (Mobility Spatial Domain Prediction) aims at predicting neighbor cell KPIs, such as expected RSRP, to support mobility events like handover preparation, neighbor ranking, or measurement gap reduction. The purpose is to proactively assist mobility management by forecasting neighbor behavior, even when direct measurements are not available.

- In BM prediction, we are considering index prediction and/or L1-RSRP prediction, but for L3 spatial prediction the RSRP prediction may always be needed to compare different cells for HO decision.

2  Source of Input Features

- Case 1 uses serving cell beam-level measurements (like SSB RSRP, CSI-RS). The focus is on exploiting real-time serving-cell beam decisions (beam ID or RSRP).

- Case 6 relies on serving cell measurement reports and spatial fingerprints, leveraging multi-beam KPIs, past neighbor reports, and spatially correlated radio patterns. The AI/ML model uses these as fingerprints to infer neighbor performance, capturing spatial correlations over time or cells.

3  Model Location and Training Focus

- In Case 1, AI/ML models are typically co-located with the UE or gNB beam management functions, trained to predict the best serving beam using local measurements.

- In Case 6, the AI/ML model is expected to reside at the network side (gNB or centralized network function), where it learns cross-cell spatial correlations using aggregated multi-UE and multi-cell data. The focus is on predicting neighbor cell or beam KPIs from serving-side fingerprints.

4  SNR condition, beam widths

- SINR condition is different for spatial domain prediction for RRM, e.g., neighbor cell SINR can be -6dB

- There could be codebook differences between serving and neighbor cells.

- Rx beam difference for serving and neighbor cells. UE can use narrow beam for BM input within a rough beam, but neighbor cell beam inputs shall be a fixed pattern, e.g., 8 beams in 360 degrees.

It is our viewpoint the Case 1 for BM conclusions cannot be reused for Case 6 spatial prediction for mobility for the following reasons:

- Different Prediction Tasks:

  - Case 1 predicts the optimal beam for the serving cell (link adaptation).

  - Case 6 predicts neighbor KPIs for proactive mobility, often without real-time measurements.

- Different Spatial Scope:

  - Case 1 works within a single TRP or cell.

  - Case 6 requires understanding cross-cell relationships and spatial patterns.

- Different KPIs and Decision Points:

  - Case 1 targets beam ID or beam set for link maintenance.

  - Case 6 targets neighbor RSRP, beam ranking, or handover candidate selection.

- Different Deployment Strategies:

  - Case 1 models operate close to beam control functions.

  - Case 6 models run at the network level, aggregating fingerprints across UEs and cells.

The two spatial prediction cases serve fundamentally different roles in the NR system. Case 1 (AI/ML BM spatial prediction) is designed for intra-cell beam management using immediate link feedback, while Case 6 focuses on neighbour prediction and proactive mobility support using cross cell measurements and AI/ML model at the network side.

The conclusions, model designs, and deployment approaches validated for Case 1 (AI/ML for BM) cannot be assumed valid for Case 6 AI/ML for mobility. Case 6 demands a broader spatial awareness, cross-cell learning, and prediction mechanisms that are unique to mobility scenarios beyond the scope of beam management addressed in Case 1

Spatial-domain prediction for mobility (Case 6) is fundamentally more demanding than intra-cell FR2 beam-management prediction (Case 1) because it must capture inter-cell correlations and cross-frequency transfer functions that vary widely with geography, and deployment topology. Whereas Case 1 can rely on a tight feedback loop between the NW and the UE, Case 6 has to model how dozens of neighbour cells, often operating on different bands, antenna codebooks and beam-widths. Achieving acceptable inference accuracy could therefore require many more training data, and many graph- or geo-spatial models able to generalize across different vendors and deployments. The risk of negative transfer (a model trained in one site degrading mobility KPIs in another) is high, and the cost of errors is high: a mis-ranked neighbour can translate directly into RLF, ping-pong or extended gap scheduling. These challenging uncertainties are unlikely to be resolved within the R-20 schedule, so forcing the topic into the WID would jeopardize timely standardization.

Spatial prediction for mobility (Case 6) is more challenging than beam management (BM Case 1) because it must estimate KPIs for multiple neighbor cells across varied bands, antenna configurations, and SINR conditions without access to real-time neighbor beam measurements. Instead, it relies on limited serving-cell  measurements and potentially outdated neighbor reports, forcing the model to infer performance from incomplete information. This demands larger, more diverse training datasets and strong generalization across deployments, while any misprediction can directly harm handover quality.

Based on the above observation, we have the following proposal:

Spatial-domain prediction for mobility requires complex cross-cell modeling across diverse antenna configurations, frequency bands, and SINR regimes, and is highly sensitive to both measurement errors and generalization errors. Addressing these challenges demands large multi-cell training datasets and robust error-tolerant model designs. We propose postponing this study to the 6G timeframe to allow sufficient time for developing scalable solutions, with good generalization, and defining comprehensive evaluation methods.

Measurement Event Prediction

At the first two RAN2 meetings, the focus was mostly on RRM measurement prediction use-case and common evaluation assumptions for it. At the RAN2#127 meeting, the discussion of the RLF use-case has also started. However, no major contribution on RLF is planned at RAN2#127bis and simulation results evaluation is expected to take place only in 2025. On the other hand, at RAN2#127bis, the measurement even prediction use-case will be discussed, as it assumed by the agenda below:

8.3.3 Measurement event predictions Contributions should focus on measurement event prediction use cases/scenarios to focus during the study, simulation assumptions and relevant performance metrics/KPIs to evaluate 8.3.4 RLF/HO failure prediction No contributions expected for this meeting Contributions should focus on discussing RLF specific methodology and simulation assumptions (addressing the differences or additional aspects from RRM prediction assumptions). Relevant metrics and assumptions not covered by email discussion No evaluations/simulation results expected for this meeting

Further discussion is needed in RAN4 regarding future requirements for RLF design:

- Should the indication be reported from the UE to the network, or only from the UE's lower layers to higher layers?

- If a direct or indirect indication is sent from the UE to the network:

  - To be determined (FFS): How should such an RLF indication be triggered to the network?

    - For a direct indication, should the UE report a predicted RLF to the network when all measured and/or predicted downlink radio link quality values on all configured RLM-RS resources fall below a threshold (Qout)?

    - For an indirect indication, should the UE report all predicted SINR values with corresponding RS indices to the network when the measured and/or predicted downlink radio link quality on all configured RLM-RS resources is worse than Qout? Alternatively, should the UE indicate the predicted SINR with the relevant RS indices as long as at least one configured RLM-RS resource shows measured and/or predicted downlink radio link quality?

In dense beamforming and massive MIMO systems, accurately maintaining RRM measurements for {TX, RX} beam pairs is increasingly difficult due to the high dimensionality of the beamspace and frequent channel variations. Conventional RRM configurations often require periodic or continuous measurement of numerous beams to ensure timely adaptation, leading to significant measurement overhead, increased UE power consumption, and reduced DL/UL throughput from scheduling conflicts with reference signal bursts. This burden becomes especially unsustainable in FR2 deployments, where measurement gaps and beam sweeps are more frequent.

AI/ML-based temporal prediction provides a promising alternative by forecasting when a {TX, RX} beam pair change is likely to occur, using historical beam-switch patterns, mobility traces, and spatial context (e.g., UE location or past beam sequences). This enables a transition from fixed-period measurement to on-demand, prediction-triggered measurement, where RRM measurements are only activated during predicted change windows. Such an approach conserves resources and maintains high link reliability, particularly for slow-moving or stationary UEs where beam changes are infrequent and predictable.

RAN4 is invited to study the use of AI/ML for temporal prediction of {TX, RX} beam pair changes in dense beamforming and MIMO systems. The objective is to reduce RRM measurement overhead by predicting when beam changes are likely to occur, enabling on-demand measurement activation and avoiding unnecessary monitoring. The study should define performance requirements, prediction accuracy metrics, and error handling procedures to ensure robust operation.

RAN4 should consider a new use case within the AI/ML-based RRM prediction framework: “Temporal Prediction of Beam Pair Changes for RRM Optimization.” This use case focuses on predicting the timing of beam transitions, rather than the best beam index, differentiating it from existing beam management and spatial prediction cases. The goal is to reduce RRM measurement efforts in the temporal domain by enabling on-demand measurement activation. The study should define performance requirements, accuracy metrics, and error-handling procedures to ensure robust operation, while aligning with the first study goal of minimizing measurement overhead

Conclusion

In this paper we listed the objectives and the RAN2 progress on the FS_NR_AIML_Mob [3]. We have additionally outlined some directions for the future working plan. The following Observations and Proposals were made:

- RRM measurement predictions provide several advantages, including reducing measurement gaps, increasing the periodicity of SMTC windows, minimizing scheduling and measurement restrictions, decreasing measurement delays, and shortening measurement gap lengths

The two spatial prediction cases serve fundamentally different roles in the NR system. Case 1 (AI/ML BM spatial prediction) is designed for intra-cell beam management using immediate link feedback, while Case 6 focuses on neighbour prediction and proactive mobility support using cross cell measurements and AI/ML model at the network side.

Spatial prediction for mobility (Case 6) is more challenging than beam management (BM Case 1) because it must estimate KPIs for multiple neighbor cells across varied bands, antenna configurations, and SINR conditions without access to real-time neighbor beam measurements. Instead, it relies on limited serving-cell  measurements and potentially outdated neighbor reports, forcing the model to infer performance from incomplete information. This demands larger, more diverse training datasets and strong generalization across deployments, while any misprediction can directly harm handover quality.

Further discussion is needed in RAN4 regarding future requirements for RLF design:

- Should the indication be reported from the UE to the network, or only from the UE's lower layers to higher layers?

- If a direct or indirect indication is sent from the UE to the network:

  - To be determined (FFS): How should such an RLF indication be triggered to the network?

    - For a direct indication, should the UE report a predicted RLF to the network when all measured and/or predicted downlink radio link quality values on all configured RLM-RS resources fall below a threshold (Qout)?

    - For an indirect indication, should the UE report all predicted SINR values with corresponding RS indices to the network when the measured and/or predicted downlink radio link quality on all configured RLM-RS resources is worse than Qout? Alternatively, should the UE indicate the predicted SINR with the relevant RS indices as long as at least one configured RLM-RS resource shows measured and/or predicted downlink radio link quality?

- The conclusions, model designs, and deployment approaches validated for Case 1 (AI/ML for BM) cannot be assumed valid for Case 6 AI/ML for mobility. Case 6 demands a broader spatial awareness, cross-cell learning, and prediction mechanisms that are unique to mobility scenarios beyond the scope of beam management addressed in Case 1

- Spatial-domain prediction for mobility requires complex cross-cell modeling across diverse antenna configurations, frequency bands, and SINR regimes, and is highly sensitive to both measurement errors and generalization errors. Addressing these challenges demands large multi-cell training datasets and robust error-tolerant model designs. We propose postponing this study to the 6G timeframe to allow sufficient time for developing scalable solutions, with good generalization, and defining comprehensive evaluation methods.

- RAN4 should consider a new use case within the AI/ML-based RRM prediction framework: “Temporal Prediction of Beam Pair Changes for RRM Optimization.” This use case focuses on predicting the timing of beam transitions, rather than the best beam index, differentiating it from existing beam management and spatial prediction cases. The goal is to reduce RRM measurement efforts in the temporal domain by enabling on-demand measurement activation. The study should define performance requirements, accuracy metrics, and error-handling procedures to ensure robust operation, while aligning with the first study goal of minimizing measurement overhead

- References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.

- R2-24xxxxx, Discussion on work plan of AI mobility, RAN2#127bis, Hefei, China, Oct 14th – 18th, 2024.

- R4-2415968, Work Plan for RAN4 Discussions on AI/ML for Mobility in NR, Hefei, China, Oct 14th – 18th, 2024.