3GPP TSG-RAN WG4 Meeting #116	R4-2509434

Bengaluru, India, August 25th – 29th, 2025

Source:	Apple

Title:	Testability and Interoperability Issues for AIML Mobility

Agenda item:	7.19.4

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114bis in the scope of Rel-19 SI ‎[1]. In RAN4#115 meeting, following topics are to be discussed.

- RAN4 impacts for RRM measurement prediction

- RAN4 impacts for measurement event prediction

- RAN4 General aspects

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution, we provide our views and discuss different aspects of testability and interoperability issues in RAN4 for AI/ML Mobility.

Discussion

This document addresses the Testability and interoperability issues related to AI/ML mobility SI in RAN4 based on the agreements and the recommended way forward of the issues from the RAN4#114bis meeting. Specifically, we cover following topics:

- Testing setup

- Multiple cells in testing setup, and

- Prediction consistency in time domain.

Testing setup

Following is the RAN4#114 agreement for AIML Beam management use case:

Agreement: - Use CDL-based channel model as starting point

Legacy RRM performance testing defined in 3GPP TS 38.133 relies primarily on AWGN channel models and static configurations, which do not adequately reflect the dynamic variations present in real-world mobility scenarios. For AI/ML-based mobility and beam management use cases, prediction accuracy is strongly influenced by time-varying fading characteristics and spatial beam dynamics. RAN4 has already agreed, in the context of FR2 beam management, that CDL-based fading channel models are a more realistic baseline. However, this agreement is not yet formally extended to AI/ML mobility use cases in either FR1 or FR2, leaving a gap in aligning test environments with the operating conditions that AI/ML models will face in deployment.

Most existing test cases are designed without L3 filtering (filter coefficient set to zero) and use a minimal number of beams to simplify setup. While suitable for deterministic AWGN evaluation, this does not capture the multi-beam and filtering effects relevant to AI/ML mobility prediction. In many mobility sub-use cases, such as predicting L3 cell-level RSRP from L1 beam-level measurements, a multi-beam configuration is inherently required..

Figure 1: 3GPP TS 38.300 Measurement Model

As illustrated in Figure 1, when L3 filtering is absent, the L1 and L3 beam measurement levels at point A1 align. Furthermore, for L3 cell-level RSRP predictions at Point C, the predicted values may correspond to the L1 and L3 beam-level predictions when a single beam is employed for prediction purposes. Furthermore, in some of the sub-use cases (for instance Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)) cell level measurements are predicted based on beam level measurements. Therefore, even if RAN4 agrees to consider only cell level measurements, a multi beam test setup would still be required to test such a use case.

RAN4 is invited to incorporate test configurations that include both L3 filtering and multi-beam scenarios when developing AI/ML mobility use case test cases. This approach ensures that prediction accuracy is evaluated under conditions representative of real-world deployments, where L3 filtering impacts measurement dynamics and multiple beams contribute to cell-level RSRP

To get ground truth (GT) for FR1 at the test equipment (TE), following agreement has been made in RAN4#114 meeting:

- Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Furthermore, following agreement was reached in RAN4#114bis meeting;

Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Alternative1: Test Equipment–Derived GT from Known Parameters
In Alt 1, the ground truth is calculated at the Test Equipment (TE) side based on parameters that are fully known and controlled, for example, transmitted or received power, calibrated channel conditions, and known propagation models. This approach is most naturally suited to conducted testing, where the TE–DUT connection is stable and calibrated, allowing the TE to compute the “ideal” RSRP without relying on the UE’s internal measurement process. Its strengths are high reproducibility, independence from UE implementation details, and immunity to “UE cheating,” since the UE’s reported predictions are not used to establish GT. However, in realistic AI/ML mobility contexts, uncertainties such as fast fading, misalignment between prediction and GT timestamps, and filtering effects (L1/L3) make exact alignment challenging. To be viable, Alt 1 requires defining a tolerance range those accounts for these uncertainties, too wide and the test loses discrimination, too narrow and it becomes unreasonably strict.

Alternative 2 – UE-Reported Measurement as GT under Controlled Conditions
In Alt 2, the ground truth is taken directly from the UE’s own measurement reports, provided they are obtained under specific high-quality conditions (e.g., sufficiently high SNR). This is closer to how the UE measures in the field and can naturally incorporate real-time channel variation, making it potentially more representative for handling fast fading and realistic propagation effects. However, this approach depends heavily on trust in the UE’s reporting, without safeguards, the UE could report a previously predicted value instead of an actual measurement (“cheating”). Detecting such behavior requires additional test mechanisms, such as comparing RSRP-vs-distance profiles, cross-checking multiple collocated UEs, or running duplicate tests (prediction run vs measurement run) under identical channel conditions. Moreover, Alt 2 typically requires at least two test runs or carefully emulated parallel channels, which is challenging in fast-varying environments because channel conditions may differ between runs.

Below we describe the anti-cheating strategies the testing procedure could employ to mitigate the cheating risks:

### 1) “Two parallel channels in one run” (twin-cell single-run)

Concept:  The TE generates two cells with matched configs and the same instantaneous channel realization with some power offset only known to TE. In the same run, the UE (a) predicts RSRP for Cell-A over PW, and (b) measures RSRP for Cell-B over the same PW; Cell-B’s stream is the GT. Because prediction and GT are simultaneous on different cells, the UE can’t simply replay predictions as “measurements,” and there’s no run-to-run fading mismatch. This mirrors the “two neighbor cells” safeguard discussed for Alt-2, but does it concurrently instead of in two separate runs.

Strengths: Removes run-to-run channel drift; closes the “replay my predictions as GT” loophole.
Caveats. The rig must synchronize channels tightly; true identity of instantaneous fading across two RF paths can be challenging OTA.

In the twin-cell single-run method, the UE must output its prediction for time t at time t – Lookahead, before the actual measurement at t exists. This timing constraint means the UE cannot simply copy a future measured value into the prediction field , the measurement is not yet available when the prediction is due. The test bench timestamps predictions and measurements, so any attempt to “back-fill” predictions with later measurements would be immediately detectable.

### Conversely, copying the prediction into the measurement field would fail because live L1 measurements for the reference cell must include the real-time effects of channel fading, receiver hardware imperfections, and L1 filtering, all captured at the moment of measurement. Replaying a prediction as a measurement strips out these dynamics and would be detected by the test bench’s liveness and plausibility checks. Introducing a known, fixed (or even time-varying) power offset between Cell A and Cell B provides an additional safeguard: if the offset is only known to the TE and applied consistently during the prediction/measurement window, the UE cannot simply reuse its Cell A predictions as Cell B measurements without failing the KPI. To pass, it must perform a genuine L1 measurement on Cell B’s signal  including the offset, which preserves test integrity while keeping both cells’ fading profiles otherwise matched for fair comparison.

### 2) Profile-comparison checks (RSRP vs distance / attenuation / Tx power)

Concept : The TE sweeps a deterministic knob (e.g., distance/attenuation/Tx power) and draws a reference RSRP profile for the emulated path. The UE’s reported “GT” profile is overlaid. Significant deviation signals cheating or misbehavior. This can be done conducted (vary attenuation) or OTA (vary probe Tx power).

Strengths:  Simple to reason about; works with a single UE; good sanity test even when channel identity can’t be guaranteed.
Caveats. A clever UE might mimic a plausible curve unless the sweep is rich enough (multi-segment, randomized order). Also, residual fading/noise can mask small cheats; so we may still need tolerances.

### 3) Multi-UE consistency (collocated UEs)

Concept:  Put two or more collocated UEs under the same conditions. Compare their reported values; if one consistently violates RSRP1 ≈ RSRP2 ± δ (noise/prediction tolerance), it’s suspicious. This would scale to >2 UEs.

Strengths:  Harder to fake consistently across different implementations; it is great for spotting outliers.
Caveats. Requires multiple DUTs and true collocation; different RF chains/calibrations create legitimate offsets, so δ must be well-defined per band/setup.

Based on the discussion above we have the following proposal:

RAN4 to adopt a layered anti-cheating framework for Alt-2 GT in FR1, with the primary method as a single-run twin-cell setup where the TE drives two synchronized cells , the UE predicts Cell-A and measures Cell-B simultaneously, and the setup includes documented channel-correlation methods plus a tolerance model for residual mismatch. At least one secondary check is required: (i) profile-comparison sweep with predefined design and pass/fail tolerance, or (ii) multi-UE collocated consistency with a specified δ per band/BW/test mode.

Impacts on Testability aspects for Time domain Case A

An example of Case A is depicted in Figure 2. Briefly, in this case, there is no measurement reduction, and the focus is instead to optimize mobility performance based on predictions.

Figure 2: Time domain case A RRM measurement prediction.

In the context of Time Domain Case A for AI/ML-based RRM measurement prediction, δ is the delay between the prediction time reference and the ground truth (GT) measurement time reference as observed by the test equipment (TE).

More specifically:

- If the UE operates sequentially, δ represents the time gap between when the UE finishes a prediction for the prediction window (PW) and when it subsequently performs and reports the GT measurement for that same PW.

- If the UE operates concurrently, δ can be much smaller, as prediction and GT measurement are made in overlapping timeframes, but processing delays within the UE can still contribute to δ.

This observation matters because δ directly impacts how the TE aligns predicted and measured RSRP values for accuracy evaluation, if not properly accounted for, mismatched timing can lead to erroneous pass/fail results.

Figure 3: Definition of delta in Time Domain Case A

RAN4 to consider the delay parameter δ in Time Domain Case A for both Alt 1 and Alt 2, defined as the time gap between prediction and GT measurement references. δ shall be measured or configured by the test equipment to align results for both sequential (larger δ) and concurrent (smaller δ with processing delays) UE modes, with tolerances to prevent false failures from timing offsets.

Impacts on Testability aspects for Time domain Case B

- In Time Domain Case B, the configuration of the skipping pattern defines exactly which measurement opportunities (slots/subframes) within the Observation Window (OW) the UE should perform actual measurements, and which ones it should skip, for a given Measurement Reduction Rate in Temporal Domain (MRRT)

In temporal domain case B, a measurement reduction is realized following the parameter MRRT (Measurement reduction rate in temporal domain). However, for a given selected MRRT value (e.g. 50%), different possible skipping patterns can be applied as shown in the example below.

Figure 4.  Example of intra-frequency temporal domain case B with MRRT of 50%

Testability is complicated by the lack of agreement in RAN2 on how the measurement skipping pattern is conveyed to the UE for a given measurement reduction rate in the temporal domain (MRRT). Even when MRRT is fixed (e.g., 50%), multiple valid skipping patterns exist, and without a standardized indication mechanism, the UE may either receive the pattern as part of the ML inference configuration or autonomously choose it. This ambiguity creates challenges for ensuring reproducible and comparable test results across vendors. Additionally, the need to evaluate prediction accuracy for potentially multiple metrics (e.g., absolute and relative RSRP accuracy, with separate thresholds) across multiple iterations, combined with numerous possible configurations of MRRT, OW, and PW, can significantly increase test complexity and execution time. There should be a clear agreement on pattern signaling and test optimization methods such as parallel processing.

We believe that multiple skipping pattern configurations should be tested because different patterns stress the AI/ML model in different ways, uniformly spaced patterns (e.g., measure–skip–measure–skip) give the model regularly spaced “anchor points,” making interpolation easier as channel changes between consecutive measurements are smaller and more predictable, while bursty patterns (e.g., measure–measure–skip–skip) create longer gaps with no measurements, forcing the model to predict over extended periods where channel conditions may vary more significantly due to mobility, fading, or interference, thus providing a more challenging scenario.

Testing only one fixed pattern risks vendors optimizing models for that specific case rather than achieving robust, generalizable performance. Real-world deployments may apply varying MRRT patterns based on mobility or scheduling, so testing across a small, standardized set of representative patterns ensures compliance, prevents overfitting, and validates that UE performance remains within requirements regardless of the specific pattern used.

RAN4 to define a standardized set of representative skipping patterns for each MRRT in Time Domain Case B, including at least one uniformly spaced (e.g., measure–skip–measure–skip) and one bursty (e.g., measure–measure–skip–skip) pattern, to cover both easier and more challenging prediction scenarios. Uniform patterns give regularly spaced anchor points that ease interpolation, while bursty patterns create longer gaps that test robustness under greater channel variation. Testing across this set will prevent overfitting, reflect real deployment variability, and ensure compliance under diverse conditions.

Multiple cells in the testing setup

In RAN2#128 meeting, following were the agreements related to cluster approach:

Agreements/Observations to be captured in TR - The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases. - For temporal domain case B, with the same MRRT, different skipping pattern can provide different prediction performance. Companies may report the adopted skipping pattern when providing simulation results. Companies are not required to run new simulations but can clarify in the spreadsheet. We can capture in the TR how the skipping patter affects the performance. - For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations. - For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. - In cluster approach the model takes measurements from more than one cell as inputs

In RAN4#114bis meeting, following were the agreements for the multiple cells for the test setup:

Issue 4-1-3: Multiple cells in the testing setup Agreement: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction.

For inter-frequency inter-cell scenario (i.e., scenario 3), RAN2 agreements indicate that the measurements from more than one cell are required as input to the ML model/ functionality for the cluster approach.

In the cluster approach, the AI/ML model doesn’t rely solely on measurements from the serving cell. Instead, it takes measurements from multiple cells (often geographically or topologically related, hence "cluster") as inputs, because the combined information can better capture the propagation environment and improve prediction accuracy. For example, in inter-frequency prediction, knowing the RSRP (or other metrics) from both the serving cell and a neighboring cell at one frequency layer gives the model more spatial and channel condition context to predict the measurement on another frequency layer. If we had only provided one cell’s input, we lose the diversity of spatial/beam perspectives that helps the model generalize better, especially when correlation between frequency layers is not perfect.

Figure 5.  Example of 2 input and 1 target cell output

RAN4 to adopt a cluster-based testing setup for inter-frequency inter-cell prediction scenarios, requiring at least two input cells (e.g., serving and neighbor) and at least one target cell as output. This ensures the AI/ML model leverages multi-cell measurement context to improve prediction accuracy, particularly when frequency-layer correlations are imperfect

Prediction consistency in time domain

Issue 4-1-2: Prediction consistency in time domain Agreement: - To ensure prediction consistency in time domain, RAN4 to discuss how to model different time-varying characteristics per cell/site due to moving UE trajectories. - Discuss how to incorporate controlled randomness and the extent of time-domain variation and correlation RAN4 to prioritize discussion on FR1

During RAN4#115 the following issue was discussed:

### Issue 1-1-1: Prediction consistency in time domain

<Way forward >:

- Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1.

- Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1.

  - Option 1: Change Tx power.

    - FFS: whether it is possible to change Tx power every symbol at TE using conductive test

  - Option 2: Change channel condition with fixed Tx power.

  - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model.

  - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

We provide below an assessment for the above-mentioned methods per our understanding:

Option 1 – Symbol-wise Tx-power stepping
This method creates time variation by adjusting the cell’s transmit power in discrete steps, potentially at OFDM-symbol granularity. The UE observes these changes as variations in RSRP, which are intended to mimic large-scale fading effects along a UE trajectory. In a conducted setup, this is implemented directly in the test equipment (TE) without any fading hardware, the commanded Tx power profile defines the exact “ground truth,” making pass/fail checks straightforward.

Pros/Cons/Conclusion: It is very simple to implement in FR1 conducted testing and requires no fading emulator. Ground truth equals the commanded power, so verification is trivial. However, TE vendors are uncertain about their ability to switch power cleanly at every OFDM symbol, and the method only emulates large-scale fading , there is no Doppler, delay spread, or realistic inter-cell correlation. This risks producing over-optimistic lab results. Suitable as an entry-level sanity check, but insufficient on its own for Rel-19 conformance.

Option 2  Fixed EPRE + time-varying fading channel
Here the transmit power (EPRE) remains fixed, but the TE drives a channel emulator to vary amplitude, Doppler, and delay in real time according to a chosen fading model (e.g., TDL for FR1, CDL if spatial features are critical). This captures multipath evolution realistically and can be standardized by selecting a reference UE trajectory from TR 38.843 , for example, urban-macro radial, urban-micro loop, or highway straight-line. All cells follow the same timestamped trajectory, so their large-scale fading evolves coherently, imposing realistic temporal correlation between serving and neighbour cells without OTA testing.

In this approach, the channel is generated in real time by the test equipment’s fading HW. The tap coefficients that define the channel response are recalculated continuously, every symbol or subframe, based on the configured scenario parameters. As a result, the signal arriving at the RF front end is truly “live,” with variations in power levels, peak-to-average power ratio (PAPR), Doppler shifts, and other fading effects occurring dynamically as the test runs. This live interaction means that the hardware experiences the same type of variability it would encounter in the field, including how front-end impairments respond to instantaneous signal changes. The main advantage is realism, but it comes with less-than-perfect repeatability because the hardware and channel emulator are interacting in real time, and small differences in timing or state can slightly alter the results from run to run.

In Option 2 (dynamic channel emulation), the ground truth is generated in real time by the fading emulator, and the TE must infer when the UE actually took its measurement in order to retrieve the correct reference value from the continuously evolving channel state. Since UE implementations can have variable internal processing delays (δ), and the measurement–report pipeline is not always strictly deterministic, the TE often relies on the report arrival time as a proxy for the measurement instant. This introduces uncertainty: if the UE measured slightly earlier or later than assumed, the TE may compare the report against the wrong point in the channel timeline. In high-Doppler or fast-fading scenarios, even a small misalignment in the assumed measurement time can cause several dB of error in the ground-truth match, directly affecting pass/fail decisions.

Pros/Cons/Conclusion: This option reproduces realistic multipath and Doppler, exercises the same RF impairments seen in the field (e.g., EVM widening, phase-noise skirts), and re-uses existing base-station emulators plus fading boxes in a conducted setup. It does require tight timestamp alignment between UE reports and TE channel state, and very high Doppler seeds can stress TE memory bandwidth. Overall, it offers the best balance of realism, feasibility, and repeatability for FR1 baseline testing.

Option 3 – Pre-generated multi-cell waveforms (“simulation replay”)
In this approach, multi-cell signals are fully simulated offline , including power ramps, fading, and event triggers  and stored as IQ files with embedded metadata for exact ground truth. The TE replays these waveforms from sample 0 with a fixed clock, so UE reporting is inherently aligned to the ground-truth timeline. No real-time channel emulation is performed, eliminating drift or timing-alignment uncertainties.

In this approach, the entire channel response is computed offline ahead of time and stored as a complete IQ waveform file. This file already contains all the desired variations, fading, Doppler, power ramps, baked into the sample sequence. During the test, the equipment simply streams this waveform to the RF front end without recalculating anything. This means the RF hardware sees the exact same voltage sequence every run, producing perfectly repeatable results. However, because the signal is fixed, there is no live adaptation, and dynamic hardware responses to varying instantaneous conditions do not change from run to run.

In Option 3 (waveform playback), the channel evolution is precomputed offline, stored as an IQ sequence, and replayed with a fixed and known subframe clock. Because the measurement slots are predetermined and embedded in the playback timeline, the TE inherently knows which ground-truth value corresponds to each slot index  regardless of when the UE’s report actually arrives. If the UE is expected to measure at subframe 120, the TE already has the exact reference value for that subframe in the waveform metadata. Even if the UE sends the result 3 ms later due to processing or signaling delays, the TE can still unambiguously align the reported value to the correct ground-truth point. This eliminates the guesswork in mapping report arrival time to measurement instant, thereby removing a major source of timing-alignment error present in Option 2.

Pros/Cons/Conclusion: This gives unlimited modelling freedom, perfect run-to-run repeatability, and inherently solves the timing-alignment problem. It is excellent for debugging and detailed profiling. However, it omits dynamic RF front-end impairments, so there is potential mismatch to field behaviour. File sizes can be large, and without a common public waveform library, vendors might use incompatible proprietary scripts. It should be optional until such a library exists, rather than a mandatory baseline.

Option 4 – Reference UE trajectory from TR 38.843 (applied to any of the above)
This is not a standalone variation method but a way to provide a common mobility profile for Options 2 or 3. The trajectory defines UE speed, heading, and position over time, which can be mapped to channel parameters in the fading emulator (Option 2) or in the waveform generator (Option 3).

Pros/Cons/Conclusion: It strengthens consistency across tests and links directly to RAN2 simulation assumptions. Mapping the trajectory to channel parameters is straightforward in Option 2 and purely offline in Option 3. FR2 applications may still require OTA or probe weighting, which RAN4 has deferred. As such, this is a complementary measure and should be paired with Option 2 as the first priority.

Considering the above pros and cons, and to give RAN4 a solid basis for verifying Release-19 AI/ML mobility features with unambiguous pass/fail criteria, we propose adopting Option 2 (fixed Tx power + live fading) as the mandatory FR1 baseline, using TDL fading as the simplest model that still captures the time-domain randomness an AI predictor must handle, with conducted cabling to keep cost low and re-use legacy RF paths. Option 1 (symbol-rate power stepping) should be retained only as an entry-level sanity check, allowing EPRE variation each symbol subject to test-equipment feasibility, but passing Option 1 alone shall not be sufficient for conformance. Every run should be bound to a single reference UE trajectory from TR 38.843, using an agreed coordinates-versus-time file to standardize temporal correlation across cells without requiring an OTA chamber. Option 3 (pre-generated multi-cell waveforms) should be offered as an optional reproducibility tool for debugging but must not replace the fading-based baseline.

RAN4 to adopt Option 2 (fixed EPRE + time-varying fader) as the FR1 baseline for prediction-consistency testing, conducted with TDL fading, and a reference UE trajectory from TR 38.843 to standardize temporal evolution, accompanied by clear definitions for channel seeds, channel update rates, δ bounds, and pass/fail tolerances. Option 3 (precomputed playback) should be approved as a supplementary mode for debugging, regression, and cross-lab reproducibility. Option 1 (symbol-wise Tx-power stepping) should remain a basic sanity check only. Pass/fail tolerances for timing alignment and explicit rules for mapping UE reports to ground truth shall be documented to ensure interoperability and comparability across test equipment’s

Below we propose a concrete action plan for RAN4 to ensure time-domain prediction consistency in the test setup:

Figure 5.  Testing procedure for temporal prediction

RAN4 to adopt the RAN4 AI/ML Mobility Test Procedure  With MRRT & Skipping Patterns as outlined in the flowchart, ensuring each step is followed in sequence. This includes freezing the baseline configuration, defining the ground-truth derivation, locking the mandatory test matrix with standardized MRRT values and skipping patterns (uniform and bursty) alongside OW/PW and δ handling rules, verifying equipment headroom, optionally providing a shared IQ-file library for reproducibility, and planning FR2 migration using the same methodology with extensions as required.

Generalization

During RAN4#115 meeting the following issue was recorded:

### Issue 1-1-4: Testing of Generalization aspects

No conclusions on generalization aspects have been achieved so far.

Note: No conclusion for this issue will not stop the completion of this study item.

We provide an analysis and corresponding proposal on generalization in the accompanying paper addressing RRM requirements for mobility.

Testing goals

### During RAN4#115 the following issue was discussed:

### Issue 1-1-7: Testing goal

<Way forward >:

- Discuss whether to further clarify the testing goal for measurement prediction use case, e.g,,

  - The testing goal for measurement prediction is to verify:

    - The reported measured results meet certain requirements under certain conditions

    - The reported predicted results meet certain requirements under certain conditions

  - Note: the requirements include core and performance requirements

  - FFS the relative requirements for the difference between reported measured results and reported predicted results

AI-based mobility allows the UE to predict future RSRP (or other RRM metrics) so that some real measurements can be skipped. Legacy conformance tests only checked the measured value against an absolute‐ or relative-accuracy mask. Release-19 must now verify two things under the same radio scene:

- the measured report in the observation window (OW) still satisfies the legacy (or updated) accuracy mask;

- the predicted report in the prediction window (PW) also stays within a tolerance, and does not diverge significantly from what the UE would have reported had it actually measured.

With AI-based mobility in Release-19, UEs can predict future RSRP (or other RRM metrics) to reduce measurement activity, expanding the testing scope beyond legacy conformance. Legacy tests focused solely on verifying measured values against absolute or relative accuracy masks. Now, RAN4 must ensure under the same radio scene that: (i) measured reports in the Observation Window (OW) meet the legacy (or updated) accuracy masks, and (ii) predicted reports in the Prediction Window (PW) remain within a defined tolerance and do not deviate significantly from what the UE would have reported if it had actually measured.

The main challenge is setting the allowable difference between predicted and measured results, whether to retain the legacy relative-accuracy limit (Option 1) or relax it by a margin X dB to account for longer prediction horizons and the power-saving benefits of higher MRRT values (Option 2). This tolerance may also need to scale with factors such as MRRT, PW length, or channel dynamics. Several contributions stress the importance of distinguishing core requirements (no degradation in mobility reliability) from performance requirements (e.g., power saving, gap reduction) when defining these limits.

It is our understanding that three complementary checks are needed to cover all failure modes:

- Measured-to-GT mask
Existing ±4 dB requirement.
Ensures the UE’s conventional measurement path remains Release-15/18 compliant. If the measured value is wrong, prediction quality is irrelevant.

- Predicted-to-GT mask
Stricter core (±4 dB) or core + Δoffset for performance rows.
Verifies that the AI/ML predictor on its own is accurate enough for link-budget calculations and network optimization.

- Predicted-to-Measured mask
Relative-delta constraint (e.g. |Pred – Meas| ≤ Δrel).
Catches pair-wise drift or step changes between the two streams so the network can switch seamlessly between measured and predicted reports without upsetting event triggers.

For example, a UE could satisfy (1) and (2) yet still let measured and predicted values drift in parallel (or diverge in opposite directions), breaking time-series continuity. Parallel drift: both values float upward by roughly the same amount, so each is still within its own band; yet a 4 dB jump appears when the network switches from “measured” to “predicted. ”Opposite drift :  one value sits at the upper edge of its band, the other at the lower edge, creating an even larger discontinuity (8 dB here) while individually staying compliant. Conversely, a small relative error (3) does not guarantee either value is close to ground truth. Using the trio keeps legacy accuracy intact, validates prediction quality, and enforces smooth hand-over between modes with minimal additional test burden.

In Release-19, AI-based mobility introduces a need to test both absolute and relative prediction accuracy. A practical approach is to define a dual-accuracy metric. First, keep the existing absolute RSRP mask for both measured and predicted values. Second, add a relative-delta mask |Predicted – Measured| ≤ Δ_rel, where Δ_rel can depend on side-conditions such as MRRT and PW length. An initial Δ_rel of 2 dB could be applied under nominal conditions, widening only if simulations show that longer PWs or higher MRRT values do not harm mobility KPIs. This ensures that predictions track real measurements closely, while still allowing flexibility for harder prediction scenarios.

For predicted values, an extra absolute slack Δ_abs may be needed to reflect the inherent difficulty of forecasting without fresh measurements. One example is |Predicted − Ground-Truth| ≤ 4 dB + Δ_abs, where Δ_abs might be 2 dB at MRRT = 25 % and 4 dB at MRRT = 50 %. This builds in more tolerance for aggressive measurement skipping, but still anchors the requirements to the familiar 4 dB baseline for measured RSRP. The exact Δ_abs values can be tuned once RAN2 quantifies any handover impacts.

Side-conditions such as prediction-window length, UE speed, and channel Doppler strongly influence achievable accuracy. Longer PWs increase extrapolation error because the channel decorrelates over time, and higher MRRT reduces the freshness of inputs. In Case A (no skipping, MRRT = 0 %), error growth is driven almost entirely by PW length, with a relatively gentle slope. In Case B (skipping enabled), error starts higher and grows faster as PW increases, especially beyond MRRT = 50 %. These relationships suggest that any tolerance table (PW, MRRT) → Δ should widen Δoffset in proportion to prediction difficulty.

Other test-design elements must also be addressed. The ground-truth method, whether TE-calculated (Alt 1) or UE-reported (Alt 2), can affect how relative accuracy is verified and whether “cheating” can be detected. Delay (δ) between measurement and prediction reporting must be accounted for, as misalignment can inflate apparent error. The influence of L3 filtering also matters if predictions bypass the smoothing applied to measurements.

AI/ML prediction accuracy is shaped by both the measurement-reduction ratio (MRRT) and prediction-window length (PW).
In Case A (no skipping, MRRT = 0 %), errors mainly depend on PW, staying close to the legacy ±4 dB mask for short horizons and growing steadily as PW extends beyond channel coherence time.
In Case B (measurement skipping), higher MRRT reduces fresh inputs and increases error; the rise is roughly proportional to MRRT, with skipping pattern and Doppler affecting slope. For small MRRT (≤25 %), Case B tracks Case A closely, but above ~50 % MRRT, accuracy degrades more quickly.
Longer PWs further widen errors in both cases, but the effect is sharper in Case B due to less up-to-date data.
Practically, higher MRRT offers battery savings but may require shorter PW or extra tolerance (Δoffset) to keep mobility KPIs stable, an approach under consideration for Release-19.

In practical terms, vendors can trade battery savings (higher MRRT) against tolerance budgets: to keep hand-over KPIs stable, either shorten the PW when MRRT is high, or allocate an additional Δoffset (e.g. +2 dB) in the test mask, exactly the approach being debated for Release-19 minimum-performance requirements.

We propose a dual-accuracy framework for Release-19 to differentiate between existing accuracy requirements for measured values and adjusted requirements for predicted values. Measured RSRP should continue to meet the current absolute accuracy mask, ensuring alignment with legacy conformance. Predicted RSRP should satisfy the same absolute mask with additional absolute slack Δ_abs applied only to predictions, allowing for the extra uncertainty introduced by prediction. In addition, a relative-delta requirement |Predicted – Measured| ≤ Δ_rel(MRRT, PW) should be applied, where Δ_rel may widen under side conditions such as higher MRRT, longer PW, increased UE speed, or higher Doppler. Both Δ_abs and Δ_rel should be defined in a standard parameter table, linked to side conditions, and verified against agreed ground-truth.

Any widening of Δ_abs or Δ_rel must be conditional on demonstrating that core mobility KPIs (e.g., handover success rate, RLF rate) are not degraded. This safeguard ensures that prediction flexibility does not compromise network reliability. The verification should be based on agreed test cases and methodologies, ideally leveraging RAN2 simulation results before finalizing tolerance values. Only when KPI stability is confirmed under the proposed slack should the corresponding relaxed tolerances be applied in Release-19 conformance testing.

Testing setup for RRM measurement prediction

### During RAN4#115 the following issue was discussed:

### Issue 1-2-1: Testing Setup in FR2

<Way forward >:

- For the testing setup for AI/ML mobility in FR1 and FR2, discuss the following configurations in the next meeting:

  - Number of SSBs per cell

  - Channel model

    - FFS the same or different for temporal/frequency domain prediction

  - Impact of UE trajectory

    - FFS the same or different for temporal/frequency domain prediction

  - Dynamic range of received power (consistency of channel)

  - Measurement prediction specific parameters

  - Emulation of noise and EPRE

  - L3 filter coefficient

### Below we provide some reasoning on why FR-1 can stay conducted while FR-2 requires an OTA system setup for the following reasons:

In FR-2 (24–52 GHz), both UEs and gNBs use fully integrated phased-array antennas with no direct RF connectors, making it impossible to inject a controlled channel through cables once the array is sealed. As a result, the only practical way to apply controlled test signals is through radiated transmission in an OTA chamber.

Propagation characteristics at FR-2 further reinforce the need for OTA. At these frequencies, path loss and blockage effects are heavily angle-dependent, and accurate reproduction requires spatial channel playback with realistic angle-of-arrival (AoA) and elevation profiles, something a purely conducted setup cannot emulate.

Finally, FR-2 deployments typically involve a much higher number of SSB beams per cell (8–64) compared to FR-1. Generating and controlling these beam patterns requires forming them in free space. Even with optimization techniques such as beam rotation-weighting, the correct spatial power distribution can only be achieved through OTA methods, leveraging the single-probe OTA framework already defined for beam management testing.

Designing an FR2 AI/ML mobility prediction test setup presents challenges that go beyond traditional FR1 or legacy mobility testing. At FR2 frequencies, the test environment must capture the spatial richness, high beam density, and rapid channel dynamics that AI models encounter in the field, while maintaining repeatability and cost efficiency. The key difficulty is balancing physical realism, so predictions are tested under representative multi-beam, 3D channel conditions, with the practical constraints of OTA test systems, chamber complexity, and measurement alignment. Without careful definition of parameters such as channel model, SSB configuration, trajectory, and noise/EPRE settings, test results risk being either unrealistic (over-simplified) or non-reproducible (over-complex).

Below we provide our thoughts on the issues and key gaps in an FR2 AI/ML-mobility test setup:

- SSB Density – FR2 gNBs transmit many SSB beams per cell, unlike legacy single-SSB tests. A one-probe chamber can simulate beams sequentially by re-weighting a single feed, but RAN4 must decide if true multi-beam transmission is required to realistically stress AI prediction.

- Channel Model – Some contributors favor simple TDL or AWGN for cost efficiency, while others want to reuse CDL (as in Beam-Management FR2 testing) for consistency. The group must resolve whether both temporal and frequency-domain prediction use the same model.

- UE Trajectory – FR2 OTA must include realistic 3D motion (including turntable rotation) so power trends, angular shifts, and blockage patterns evolve coherently. Without it, AI sees random fades that undermine beam/event prediction validity.

- Dynamic Range Control – The chamber must maintain a consistent received-power envelope along the trajectory to keep test difficulty within the AI model’s expected operating range.

- Prediction-Specific Parameters – OW, PW, and MRRT settings must be explicitly defined in the test script to ensure precise timestamp alignment between ground truth and predictions.

- Noise and EPRE Scaling – OTA must inject controlled noise and apply per-beam EPRE adjustments to test both core (high SNR) and performance (low SNR) conditions without exceeding probe dynamic range.

- L3 Filter Coefficient – AI models may have been trained with non-zero L3 filtering, but disclosing coefficients risks overfitting; hiding them entirely can inflate tolerances. A balance must be found.

In every FR2 AI/ML mobility test, the Observation Window (OW), Prediction Window (PW), and Measurement-Reduction Rate (MRRT) must be explicitly defined and included in the header of the test script. These three parameters tell the test equipment (TE) exactly when the UE is expected to take its measurements, how far into the future the prediction applies, and which measurement slots were intentionally skipped. With this information, the TE can correctly align the fading-seed timeline to the UE’s reports and compute the ground-truth RSRP at the right instants. If these parameters are missing, the TE risks misaligning predictions with the actual channel state, which can cause timing drift in the evaluation or allow the UE to “game” the results by shifting its reporting schedule.

The following steps outline a baseline approach that supports conformance evaluation, avoids over-complexity in the early phase, and leaves room for later enhancements once Beam-Management and FR2 OTA methods mature.

Step 1 – SSB Setup
Begin with one active SSB per cell to maintain single-probe OTA chamber feasibility. If a UE requests more beams than the TE can emulate, the extra beams should be merged in its report without causing a failure. Multi-SSB playback can be revisited once Beam-Management completes relevant studies.

Step 2 – Channel Model
Use TDL-C as the baseline fading model for both temporal- and frequency-domain prediction. Add an optional CDL-mini profile (with a limited number of clusters) later to incorporate partial spatial realism without early complexity.

Step 3 – Reference Trajectory
Adopt a single standardized UE trajectory from TR 38.901, including defined speed, heading, and rotation parameters, and replay it identically in all FR2 labs to ensure temporal consistency in channel evolution.

Step 4 – Power Envelope Standardization (large scale) 
Publish a common time-indexed table defining peak/trough RSRP levels and ramp rates so that all labs reproduce the same large-scale fading dynamics, independent of chamber geometry or emulator interpolation.

Step 5 – Prediction Parameters
Require that OW, PW, and MRRT values are explicitly embedded in every test-script header and applied by the DUT. This ensures accurate timestamp alignment for ground-truth comparison and prevents artificial timing offsets.

Step 6 – Noise and EPRE Control
For core accuracy runs, maintain a constant high SNR across the trajectory. For performance runs, introduce slow-fading SNR variations while keeping per-beam EPRE fixed, enabling evaluation of predictor robustness under realistic SNR swings. ±6 dB fading means that, in addition to the fixed transmit power and the main large-scale fading profile (path loss + shadowing), you deliberately introduce a slow variation in received signal strength that moves up to 6 dB above or below the nominal value.

Step 7 – L3 Filtering
Fix the FilterCoefficient to zero for core pass/fail evaluation. Include an optional test row with a moderate non-zero coefficient to assess AI/ML robustness under typical network filtering conditions.

Figure 6.  FR2 AI/ML mobility test setup flowchart

RAN4 should follow the FR2 AI/ML mobility testing procedure outlined in the accompanying flowchart, using a single-probe OTA chamber configured with one active SSB per cell, a TDL-C fading model, and a common reference UE trajectory from TR 38.901. The setup shall embed OW, PW, and MRRT in each test script to ensure time alignment, with high-SNR “core” runs for baseline accuracy and “performance” runs adding controlled ±6 dB slow-fading variations to stress prediction robustness.

Testing setup for Intra-Frequency Prediction

During RAN4#115 the following issue was discussed:

### Issue 1-2-3: Testing Setup for intra-frequency prediction

< Agreement>:

- Clarify the agreement in last meeting:

  - For intra-frequency intra-cell RRM measurement prediction/event prediction, RAN4 to consider two cells in the test: serving cell and another/target cell.

    - Note: The measurement and prediction are performed on the same cell.

We present out perspective on this issue below:

From a control-plane perspective, the need for two cells in intra-frequency prediction testing arises from how RRM measurements and prediction signaling are configured in 5G NR. In a real network, the serving cell is responsible for maintaining the RRC connection, configuring measurement objects, and instructing the UE which neighbour cells to monitor. For intra-frequency intra-cell prediction, the UE measures and predicts RSRP (or other RRM metrics) for a target cell that is on the same carrier as the serving cell. Even if no actual mobility event or handover is triggered, the serving cell still sends the configuration for observation window (OW), prediction window (PW), and reporting criteria for the target cell. Without a serving cell in the setup, there is no realistic RRC signalling path to configure and control the prediction task, making it impossible to test the UE’s measurement and prediction behaviour under standard-compliant procedures.

Temporal prediction is bound to a single cell: the UE must estimate the future RSRP of whichever cell it is currently reporting ,either the serving cell or a configured neighbour , not a cross-cell delta. Measurement and prediction therefore share the same PCI and ARFCN in every validation step. In this setup, the UE remains anchored to the serving cell for all control-plane operations while performing measurements and predictions exclusively for the target cell. Measurements in the OW are taken according to the serving cell’s configuration, using CSI-RS or SSB resources from the target cell, and predictions in the PW are generated based on these observations. The serving cell also determines the reporting schedule and receives both measured and predicted reports, ensuring that the prediction is exercised under realistic control-plane timing and signalling conditions while keeping the serving cell stable and enabling a clean evaluation of prediction accuracy.

RAN4 should adopt a two-cell setup for intra-frequency intra-cell prediction testing, with one cell acting as the serving (control-plane anchor) and the other as the target (prediction subject). This configuration ensures that the UE’s measurement and prediction are exercised under realistic RRC signaling, with the serving cell configuring the OW, PW, and reporting parameters for the target cell. It also reflects actual network behavior, where prediction is bound to a single cell’s PCI/ARFCN but is still managed via control-plane procedures from the serving cell.

Test environment mismatch

During RAN4#115 the following issue was raised:

### Issue 1-2-6: Test environment mismatch

<Way forward>:

- Discuss whether and how the following aspects should be considered for AI/ML mobility testing:

  - Data consistency between training and inference

  - Data consistency between serving cell configuration (e.g. TCI state update, TA updates, etc) and dynamic radio environment

The core problem in Issue 1-2-6 is the mismatch between the conditions under which an AI/ML mobility model is trained and the conditions under which it is tested. In the field, models could be trained on logs covering a wide variety of speeds, Doppler spreads, SNR ranges, MRRT levels, beam switches, and timing-advance (TA) changes. However, conformance testing is often carried out in a controlled, low-variance lab setup, which can create a distribution shift. This mismatch may make an underperforming model appear artificially strong if the test environment is too “easy,” or conversely penalize a well-trained model if the lab omits important signal features it relies on. The key challenge is ensuring that the statistical coverage of test scenarios matches the diversity of conditions in the training dataset.

Another dimension of this mismatch is the loss of coherence between the control plane and the radio environment. In a live network, serving cell parameters like TCI-state and TA updates track the UE’s motion and evolving channel. If a lab replay of the fading profile does not update TCI or TA accordingly, the UE experiences a control-plane state that does not match the physical channel. For example, keeping the same TCI state throughout a trajectory where path loss changes by 30 dB produces an unrealistically “easy” condition. Likewise, updating TCI too often may create an overly harsh scenario. A realistic cadence for TCI-state changes, aligned with UE trajectory, speed, and fading, ensures that control signaling and channel behavior remain consistent, which is critical for testing predictors trained on coherent field conditions.

In NR, the gNB signals a TCI state to indicate which transmit beam (SSB or CSI-RS index) should be treated as quasi-co-located with the PDSCH and control channels. This state changes over time as the optimal downlink beam shifts with the UE’s movement. The cadence of TCI updates is critical for realistic testing. In live networks, updates occur every few slots to a few hundred milliseconds, depending on UE speed, blockage, and scheduling. If a lab test never updates the TCI state, the UE experiences an unrealistically stable beam despite large fading variations, making the prediction task too easy. On the other hand, updating every slot would create overly harsh conditions not seen in typical deployments. A realistic cadence scripts TCI updates in sync with the UE’s trajectory and the fading profile, slower rates for pedestrian or indoor scenarios, faster rates for vehicular or highway speeds. This alignment keeps the control plane (beam index) and the radio channel (path loss, Doppler) coherent, ensuring that AI/ML models are tested in conditions that reflect their training environment. Such realistic TCI-state switching patterns should also be incorporated into conformance testing so that prediction models are evaluated under conditions representative of real deployments.

A further gap lies in the exclusion of neighbor-cell effects. In real deployments, the serving-cell signal is rarely isolated; at least one co-channel neighbor is present, affecting interference levels, and the triggering of mobility events Omitting such a neighbor in lab tests produces artificially clean measurement inputs and removes competitive dynamics in event triggers (e.g., A3). To close this gap, FR2 AI/ML mobility conformance should identify which real-world complexities, Doppler variation, SNR range, TCI/TA update cadence, and neighbor interference, must be faithfully emulated in the lab to keep test statistics representative of the field. This will ensure that model performance in testing reflects its expected behavior in deployment.

RAN4 should align AI/ML mobility conformance testing with key real-world conditions to avoid training–testing mismatch. Test scenarios must cover representative Doppler and SNR ranges, MRRT patterns, and beam/TA dynamics matching the model’s training data. Control-plane coherence should be preserved by updating TCI and TA states at realistic cadences linked to the UE trajectory, keeping beam index changes in step with the channel. Neighbor-cell effects should also be included to reproduce realistic interference. This will ensure lab tests fairly reflect expected field performance.

Testing aspects of measurement event prediction

### During RAN4#115 the following issue was discussed:

### Issue 1-3-1: Testing aspects of measurement event prediction

<Proposals and Observations>:

- Proposal 1:

  - For the testing of event prediction, RAN4 to study how to know the real event occurs.

We discuss below the problem statement and the issues we need to address:

1. Problem Statement – Event Timing is Algorithmic, Not Physical
In measurement event prediction, “ground truth” event timing is not a single RSRP sample but the output of an algorithm, typically a filtered neighbour-minus-serving RSRP compared against a threshold for the full Time-To-Trigger (TTT). This means the “true” event time depends on the fading profile, the L3 filter coefficient, and the TTT setting. Any mismatch between these parameters in training vs. testing shifts the ground-truth event moment, sometimes by as little as 20 ms, but enough to cause large swings in F1-score. This creates a challenge: how to define an unambiguous, reproducible event time that both reflects physical reality and is resistant to manipulation by the UE under test.

2. AI Model Types – Indirect vs. Direct Prediction
Two main AI/ML approaches complicate testing. Indirect prediction first predicts future RSRPs and then derives the event time using the same detection formula as legacy. Here, the test equipment (TE) can reuse its RSRP ground truth. Direct prediction outputs the probability of the event itself; RAN2 suggests evaluating this with an F1-score inside an Event-Time-Deviation (ETD) window. However, ETD width, start trigger, and alignment rules are still open. The choice of evaluation method affects how ground truth is generated and whether testing can be unified for both prediction styles.

Below we present some options for defining the ground truth of event:

Option 1 – Spec-Based Ground Truth (GT)
This approach calculates event times entirely offline from the fading script and the exact event parameters (threshold, L3 filter, TTT, hysteresis). It is cheat-proof because the UE never sees the GT during testing and cannot influence it. The advantage is perfect repeatability across labs, but the risk is parameter mismatch: if the DUT is signalled a different filter coefficient than assumed in GT generation, pass/fail results will be wrong. To avoid this, the full event configuration must be in the test script, and the DUT must echo these parameters.

Option 2 – Ideal-Channel Ground Truth (GT)
Here, the test scene is replayed once at high SNR (noise and interference removed) to capture event times from a golden reference receiver. This ensures GT reflects real receiver behavior, including filter delay, without random noise jitter. A second run replays the same fading script under realistic SNR/impairments for conformance. This method aligns well with real-world receiver timing but risks cheating if the DUT participates in the clean run, as it could cache timestamps. The solution is to derive GT exclusively from an independent reference receiver, not the DUT.

Option 3 – UE-Reported Ground Truth (GT)
In this method, the DUT itself provides both the measured event time and the predicted event time in the same noisy run. While operationally simple (single run, no offline GT), it is inherently vulnerable: the UE can simply copy its measured time into its prediction, guaranteeing near-zero error. This makes it unsuitable for official conformance, though it may be valuable for debugging during development. For standardization, the most robust approach is a spec-based GT as the primary reference, calibrated periodically against a golden-receiver high-SNR replay to ensure parameter fidelity, with predictions tested in realistic noisy runs against this cheat-proof benchmark.

A calibration step would be necessary because even though the spec-based ground truth (GT) is calculated perfectly from the fading script and UE parameters, it is still an idealized timeline, it assumes that a real receiver processes measurements with zero extra delay.

In reality, receiver hardware introduces small but consistent offsets. Examples include:

- L3 filter delays if the filter coefficient is non-zero.

- ADC sampling and processing latency in the UE’s baseband.

- Pipeline delays in how the UE applies measurement filtering before event evaluation.

If we skip calibration, these small delays can cause the “real” measured event time to drift slightly from the calculated GT, even in perfect SNR conditions. That drift could unfairly penalize a DUT (or make it look better than it is) during noisy runs.

By replaying the exact fading profile once at very high SNR and logging event times with a golden reference receiver(not the DUT), we get a clean, measurement-based view of when the event truly fires on real hardware without random noise jitter. We then check that these measured times match the spec-based GT within ±1 slot. If they don’t, it’s a sign that the theoretical calculation is missing a fixed delay factor, and we can adjust before running noisy tests.

This step bridges theory and practice so that the GT is both cheat-proof (still computed offline) and physically accurate, making it a trustworthy benchmark for all conformance testing.

Spec-Based Ground Truth as Primary Reference
RAN4 should adopt a spec-based ground truth (GT) as the primary reference for event-prediction conformance. This GT is computed entirely offline from the scripted fading profile using the exact signalled event parameters (threshold, L3 filter coefficient, TTT, hysteresis). Because it is deterministic and independent of the DUT’s measurements, it is inherently cheat-proof and repeatable across all labs. UE-reported GT should be excluded from conformance testing to prevent trivial alignment cheats, though it may remain available for vendor debugging.

Calibration and Prediction Workflow
To ensure that the spec-based GT reflects real receiver behavior, each test bench should periodically perform a one-time high-SNR calibration replay using a golden reference receiver. Measured event times must fall within ±x slot of the spec-based GT before use in testing. For prediction runs, the identical fading script is replayed under the intended noisy conditions, and the DUT’s predicted event times are compared directly to the pre-computed GT using agreed Event-Time-Deviation (ETD) accuracy masks.

Conclusion

In this paper, we have shared our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML Mobility use cases. The aspects covered in this paper include testing setup, multiple cells in the testing setup, and prediction consistency in time domain. Thus, the following Observations and Proposals were made.

- RAN4 is invited to incorporate test configurations that include both L3 filtering and multi-beam scenarios when developing AI/ML mobility use case test cases. This approach ensures that prediction accuracy is evaluated under conditions representative of real-world deployments, where L3 filtering impacts measurement dynamics and multiple beams contribute to cell-level RSRP

- RAN4 to adopt a layered anti-cheating framework for Alt-2 GT in FR1, with the primary method as a single-run twin-cell setup where the TE drives two synchronized cells, the UE predicts Cell-A and measures Cell-B simultaneously, and the setup includes documented channel-correlation methods plus a tolerance model for residual mismatch. At least one secondary check is required: (i) profile-comparison sweep with predefined design and pass/fail tolerance, or (ii) multi-UE collocated consistency with a specified δ per band/BW/test mode.

RAN4 to consider the delay parameter δ in Time Domain Case A for both Alt 1 and Alt 2, defined as the time gap between prediction and GT measurement references. δ shall be measured or configured by the test equipment to align results for both sequential (larger δ) and concurrent (smaller δ with processing delays) UE modes, with tolerances to prevent false failures from timing offsets.

RAN4 to define a standardized set of representative skipping patterns for each MRRT in Time Domain Case B, including at least one uniformly spaced (e.g., measure–skip–measure–skip) and one bursty (e.g., measure–measure–skip–skip) pattern, to cover both easier and more challenging prediction scenarios. Uniform patterns give regularly spaced anchor points that ease interpolation, while bursty patterns create longer gaps that test robustness under greater channel variation. Testing across this set will prevent overfitting, reflect real deployment variability, and ensure compliance under diverse conditions.

RAN4 to adopt a cluster-based testing setup for inter-frequency inter-cell prediction scenarios, requiring at least two input cells (e.g., serving and neighbor) and at least one target cell as output. This ensures the AI/ML model leverages multi-cell measurement context to improve prediction accuracy, particularly when frequency-layer correlations are imperfect

RAN4 to adopt Option 2 (fixed EPRE + time-varying fader) as the FR1 baseline for prediction-consistency testing, conducted with TDL fading, and a reference UE trajectory from TR 38.843 to standardize temporal evolution, accompanied by clear definitions for channel seeds, channel update rates, δ bounds, and pass/fail tolerances. Option 3 (precomputed playback) should be approved as a supplementary mode for debugging, regression, and cross-lab reproducibility. Option 1 (symbol-wise Tx-power stepping) should remain a basic sanity check only. Pass/fail tolerances for timing alignment and explicit rules for mapping UE reports to ground truth shall be documented to ensure interoperability and comparability across test equipment’s

Figure 5.  Testing procedure for temporal prediction

RAN4 to adopt the RAN4 AI/ML Mobility Test Procedure – With MRRT & Skipping Patterns as outlined in the flowchart, ensuring each step is followed in sequence. This includes freezing the baseline configuration, defining the ground-truth derivation, locking the mandatory test matrix with standardized MRRT values and skipping patterns (uniform and bursty) alongside OW/PW and δ handling rules, verifying equipment headroom, optionally providing a shared IQ-file library for reproducibility, and planning FR2 migration using the same methodology with extensions as required.

We propose a dual-accuracy framework for Release-19 to differentiate between existing accuracy requirements for measured values and adjusted requirements for predicted values. Measured RSRP should continue to meet the current absolute accuracy mask, ensuring alignment with legacy conformance. Predicted RSRP should satisfy the same absolute mask with additional absolute slack Δ_abs applied only to predictions, allowing for the extra uncertainty introduced by prediction. In addition, a relative-delta requirement |Predicted – Measured| ≤ Δ_rel(MRRT, PW) should be applied, where Δ_rel may widen under side conditions such as higher MRRT, longer PW, increased UE speed, or higher Doppler. Both Δ_abs and Δ_rel should be defined in a standard parameter table, linked to side conditions, and verified against agreed ground-truth.

Any widening of Δ_abs or Δ_rel must be conditional on demonstrating that core mobility KPIs (e.g., handover success rate, RLF rate) are not degraded. This safeguard ensures that prediction flexibility does not compromise network reliability. The verification should be based on agreed test cases and methodologies, ideally leveraging RAN2 simulation results before finalizing tolerance values. Only when KPI stability is confirmed under the proposed slack should the corresponding relaxed tolerances be applied in Release-19 conformance testing.

Figure 6.  FR2 AI/ML mobility test setup flowchart

RAN4 should follow the FR2 AI/ML mobility testing procedure outlined in the accompanying flowchart, using a single-probe OTA chamber configured with one active SSB per cell, a TDL-C fading model, and a common reference UE trajectory from TR 38.901. The setup shall embed OW, PW, and MRRT in each test script to ensure time alignment, with high-SNR “core” runs for baseline accuracy and “performance” runs adding controlled ±6 dB slow-fading variations to stress prediction robustness.

RAN4 should adopt a two-cell setup for intra-frequency intra-cell prediction testing, with one cell acting as the serving (control-plane anchor) and the other as the target (prediction subject). This configuration ensures that the UE’s measurement and prediction are exercised under realistic RRC signaling, with the serving cell configuring the OW, PW, and reporting parameters for the target cell. It also reflects actual network behavior, where prediction is bound to a single cell’s PCI/ARFCN but is still managed via control-plane procedures from the serving cell.

RAN4 should align AI/ML mobility conformance testing with key real-world conditions to avoid training–testing mismatch. Test scenarios must cover representative Doppler and SNR ranges, MRRT patterns, and beam/TA dynamics matching the model’s training data. Control-plane coherence should be preserved by updating TCI and TA states at realistic cadences linked to the UE trajectory, keeping beam index changes in step with the channel. Neighbor-cell effects should also be included to reproduce realistic interference. This will ensure lab tests fairly reflect expected field performance.

Spec-Based Ground Truth as Primary Reference
RAN4 should adopt a spec-based ground truth (GT) as the primary reference for event-prediction conformance. This GT is computed entirely offline from the scripted fading profile using the exact signalled event parameters (threshold, L3 filter coefficient, TTT, hysteresis). Because it is deterministic and independent of the DUT’s measurements, it is inherently cheat-proof and repeatable across all labs. UE-reported GT should be excluded from conformance testing to prevent trivial alignment cheats, though it may remain available for vendor debugging.

Calibration and Prediction Workflow
To ensure that the spec-based GT reflects real receiver behavior, each test bench should periodically perform a one-time high-SNR calibration replay using a golden reference receiver. Measured event times must fall within ±x slot of the spec-based GT before use in testing. For prediction runs, the identical fading script is replayed under the intended noisy conditions, and the DUT’s predicted event times are compared directly to the pre-computed GT using agreed Event-Time-Deviation (ETD) accuracy masks.

- References

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19).