3GPP TSG-RAN WG4 Meeting #116		R4-2509439

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	7.21.4.1

Source:	Apple

Title:   	On UE RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# 1	Introduction

In the latest R19 SBFD WI [1], there is RAN4 objective on UE CLI measurement and reporting, as shown below:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

Most open issues had been closed by the last RAN4 meeting. In this contribution, we continue to share our views on the open issues captured in the WF [2].

# 2	Discussion

Requirement applicability

#### Issue 1-1-1: Requirement applicability - Agreement (Monday online session) - For method #2, 3 and non-basic UE operation of method #1 - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is configured within active DL BWP - when CLI-RSSI measurement resource is configured within active DL BWP - FFS for basic UE operation for method #1 - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

In the latest RAN1 feature list agreements [3], there is FG60-1 related to basic UE operation for method 1, as copied below:

| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |
| --- | --- | --- | --- |

Clearly, whether a UE supports DL reception in two DL subbands in SBFD symbols is not concluded yet as indicated by the first FFS point.

In addition, there is another FG below, FG 60-8, that is related to this point. Component 2 says UE may support CLI-RSSI measurement resource configured across two DL subbands only, however, it is unclear whether the UE is required to measure CLI-RSSI across two DL subbands simultaneously.

| 60. NR_duplex_evo | 60-8 | L1 CLI-RSSI measurement and [aperiodic] reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG |
| --- | --- | --- | --- |

In summary, regarding basic UE operation for method #1, RAN1 discussion on UE capability is still ongoing. As such, it is proposed to keep it FFS.

At the same time, Apple’s preference is to consider UEs that cannot simultaneously measure the CLI-RSSI measurement resource configured over two DL subbands. For such UEs, it is proposed to double the measurement delay compared to the case where CLI-RSSI measurement resource is configured in one UL subband only, or in one DL subband only.

Proposal 1: Regarding basic UE operation for method #1, since the RAN1 discussion on UE capability is still ongoing without a conclusion, it is proposed to keep it FFS. Apple’s preference is to consider UEs that cannot simultaneously measure the CLI-RSSI measurement resource configured over two DL subbands. For such UEs, the measurement delay is doubled.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: Regarding basic UE operation for method #1, since the RAN1 discussion on UE capability is still ongoing without a conclusion, it is proposed to keep it FFS. Apple’s preference is to consider UEs that cannot simultaneously measure the CLI-RSSI measurement resource configured over two DL subbands. For such UEs, the measurement delay is doubled.

# 4	References

RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)," Huawei

RP-2508274, "WF on RRM requirements for NR_duplex_evo," Huawei, HiSilicon

R1-2504673, " Updated RAN1 UE features list for Rel-19 NR after RAN1 #121," Moderators (AT&T, NTT DOCOMO, INC.)