3GPP TSG-RAN WG4 Meeting #116		R4-2509444

Bengaluru, India, 25th – 29th August, 2025

Title:	[draft] Reply LS on Beam Correspondence Initial Access

Response to:	R5-253653

Release:	R18

Work Item:	NR_RF_FR2_req_Ph3-UEConTest

Source:	RAN4

To:	RAN5

Cc:

Contact person:	Steven Chen

steven.x.chen AT apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 would like to thank RAN5 for the LS. Regarding the questions in the LS, please see below RAN4’s responses.

Q1: RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Response: RAN4 core requirements are specified under the assumption that UE beamlock function is used in testing. The reason to use beamlock function is to address the issues raised during the core requirement discussion, e.g., “when the RAR is held to trigger PRACH retransmission, UE may attempt other beams since the RAR is not received which may lead to unstable test” This is important as in the core requirement, there is no tolerance allowed.

Therefore, RAN4 is of the view that test procedure with UE beamlock function is required to properly verify the RAN4 requirements, while it is uncertain whether test procedure without UE beamlock function can properly verify the RAN4 requirements.

Q2: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams?

Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Response: It is RAN4 understanding that in real-field operation, UE chooses its UL beams based on DL measurements of BS TX beams, which is the reason of specifying beam correspondence requirements in RAN4 for UEs requiring no UL beam sweeping. Therefore, using beamlock function in testing does not prevent UE from choosing UL beams in the real field. Instead, beamlock function is used to ensure testing accuracy, reliability, and repeatability during tests.

Q3: Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Response: As said above, RAN4 core requirements are specified under the assumption that UE beamlock function is used in testing. Therefore, verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode is not aligned to verifying RAN4 core requirements when the requirements were defined.

# 2	Actions

To RAN5

ACTION: 	RAN4 respectfully asks RAN5 to consider the above responses in its work.

# 3	Dates of next TSG RAN WG4 meetings

RAN4#116-bis	Oct 13 – 17, 2025		Prague, CZ

RAN4#117	Nov. 17 – 21, 2025	Dallas, US