# 3GPP TSG-RAN WG4 Meeting #116	                                                	     R4-2509457

# Bangalore, India, 25 – 29 August, 2025

Agenda Item:	7.25.4

Source: 	Apple

Title:	Discussion on RRM performance of NR mobility enhancements Phase 4

Document for:	Discussion

# Introduction

According to TU assignment for this WID, performance part starts from RAN4#115. In this contribution, we provide initial discussion on scope of performance. After discussion some proposals are provided.

# Event triggered L1 measurement reporting

As we discussed in core part, this objective doesn’t introduce any new measurement behavior. Only a new reporting mechanism is introduced. For measurement, RAN4 already has core requirements and corresponding test cases introduced in R18, including intra-frequency/inter-frequency, measurement without gap and within gap and so on. Therefore, in this objective RAN4 only needs to introduce some essential test cases. The following list is provided from our side:

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

- Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

- Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

Proposal 1: test case list for event triggered L1 measurement reporting:

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

- Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

- Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

# CSI-RS based L1-RSRP on LTM candidate cell

Performance part for CSI-RS based L1-RSRP on LTM candidate cell includes accuracy requirements. It was agreed in the last meeting to reuse legacy side condition of SINR=-3dB from serving cell L1-RSRP measurement. And it was also agreed to resue the same CSI-RS resource configuration and time domain configuration restriction, we think it’s natural the lagacy accuracy requirements for serving cell L1-RSRP are also applicable for neighbour cell.

Proposal 2: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell.

Proposal 3: introduce the following accuracy requirements for CSI-RS based L1-RSRP on LTM candidate cell.

CR for Accuracy requirements

| Accuracy requirements | Detail | Company |
| --- | --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |  |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Absolute Accuracy |  |

Performance part also includes test cases for core requirements and accuracy requirement. The following list is proposed based on SSB L1 RSRP test case list except scenarios which are not support from RAN4 requirement point of view, such as measurement with gap.

Proposal 4: introduce the following test cases for CSI-RS based L1-RSRP on LTM candidate cell

Test cases for core part

| Core requirements defined | Detail | Company |
| --- | --- | --- |
| Intra-f L1-RSRP measurement for LTM | A.6.6.26.2 - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |  |
|  | A.7.6.20.2 - Intra-f L1-RSRP measurement in FR2 Test case 1: - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated Test case 2: - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |  |

Test cases for performance part

| Test cases | Company |
| --- | --- |
| Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 |  |
| Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 |  |

# Conditional LTM

Given that UE needs to pass legacy LTM test cases, there is no need to verify all aspects of cell switch procedure. We only need to focus on the delta part, i.e. cell switch is triggered by UE. Therefore, we propose the following test list for CLTM and subsequent CLTM.

- Inter-frequency CLTM from FR1 to FR1

- Intra-frequency CLTM from FR2 to FR2

- Intra-frequency subsequent CLTM from FR1 to FR1

- Inter-frequency subsequent CLTM from FR2 to FR2

Proposal 5: introduce the following test cases for CLTM and subsequent CLTM.

- Inter-frequency CLTM from FR1 to FR1

- Intra-frequency CLTM from FR2 to FR2

- Intra-frequency subsequent CLTM from FR1 to FR1

- Inter-frequency subsequent CLTM from FR2 to FR2

One last point we would like to raise is that subsequent CLTM naturally include a complete CLTM procedure. Thus for UE which can pass subsequent CLTM, test case for CLTM can be skipped.

Proposal 6: for UE which can pass subsequent CLTM, test case for CLTM can be skipped.

# Conclusion

In this contribution, we provide discussion on performance requirements for NR mobility enhancements Phase 4. After discussion, the following conclusions are provided:

Proposal 1: test case list for event triggered L1 measurement reporting:

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

- Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

- Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

Proposal 2: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell.

Proposal 3: introduce the following accuracy requirements for CSI-RS based L1-RSRP on LTM candidate cell.

| Accuracy requirements | Detail | Company |
| --- | --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |  |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Absolute Accuracy |  |

Proposal 4: introduce the following test cases for CSI-RS based L1-RSRP on LTM candidate cell

Test cases for core part

| Core requirements defined | Detail | Company |
| --- | --- | --- |
| Intra-f L1-RSRP measurement for LTM | A.6.6.26.2 - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |  |
|  | A.7.6.20.2 - Intra-f L1-RSRP measurement in FR2 Test case 1: - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated Test case 2: - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |  |

Test cases for performance part

| Test cases | Company |
| --- | --- |
| Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 |  |
| Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 |  |

Proposal 5: introduce the following test cases for CLTM and subsequent CLTM.

- Inter-frequency CLTM from FR1 to FR1

- Intra-frequency CLTM from FR2 to FR2

- Intra-frequency subsequent CLTM from FR1 to FR1

- Inter-frequency subsequent CLTM from FR2 to FR2

Proposal 6: for UE which can pass subsequent CLTM, test case for CLTM can be skipped.

# Reference

- TS38.133