# 3GPP TSG-RAN WG4 Meeting #116	R4-2509476

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.8.6

Source:	Apple

Title:	On UE Demodulation and CSI Reporting Requirements for 7MHz

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The performance part for Rel-19 NR FR1 7MHz Channel Bandwidth WI was discussed in RAN4#115 and way forward [1] was agreed. In this contribution we present simulation results and views on requirements.

# 2. Discussion

In RAN4#115 the following agreements were made for PDSCH requirements:

Whether to define PDSCH single carrier requirements for 7MHz bandwidth Agreements: - Don’t define 7MHz for PDSCH requirements for single carrier Whether to define PDSCH CA requirments for 7MHz bandwidth Agreements: - Define normal PDSCH type A CA requirements for 7MHz - Not to consider HST-SFN and HST-DPS scenario. - Interested companies are welcome to bring results for 7MHz. - FFS how to define requirements: - Option 1: Define new requirements for 7MHz CC based on the simulations - Option 2: Reuse existing 5MHz/10MHz requirements SDR requirements Agreements: - Define SDR requirements for 7MHz bandwidth - For Table 5.5A-1 in TS 38.101-4, add the following in TS38.101-4 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 7MHz”. - For Table 5.5A-4 in TS 38.101-4, set the number of CORESET PRBs to ‘30’.

In [1] the following simulation assumptions were agreed:

Simulation assumptions for 7MHz CC for CA requirements

- Duplex mode: Only FDD

- Bandwidth/SCS: 7MHz/15kHz

- Number of RB for CORESET:30

- Number of RB: 35PRBs

- MCS: 13

- Rank: Rank2

- MCS: 16QAM,0.48

- Propagation conditions: TDLA30-10

- Antenna configuration: 2T2R, 2T4R ULA Low

- Other parameters: Reuse Table 5.2.1 in TS 38.101-4

Based on the agreed simulation parameters, we provide alignment and impairment results

Table 1: Simulation results for 7MHz CBW

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | SNR @ 70% Max TP (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Alignment | Impairment |
| 7 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 11.4 | 13.4 |
| 7 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 6.6 | 8.6 |

An open issue was whether to define new requirements or reuse existing requirements.We propose to collect simulation results to decide that.

- Decide whether existing requirements are reused or new requirements are defined based on simulation results collection.

# 3. Conclusion

In this paper, we provide simulation results for PDSCH CA requirements for 7MHz CBW. Our observations and proposals are captured below:

- Decide whether existing requirements are reused or new requirements are defined based on simulation results collection.

# Reference

- R4-2508757	, “Way Forward for [115][335] NR_7MHz_demod”, Huawei.