# 3GPP TSG-RAN WG4 Meeting #116	R4-2509484

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.20.4.1

Source:	Apple

Title:	On RRM Requirements for UE initiated beam management

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#115 the RRM requirements for UE initiated beam management was discussed and WF [1] was approved. In this contribution we present our views on RRM core requirements for UE initiated beam management.

# 2. Discussion

In [1] the following agreements were made for RRM core requirements with UE initiated beam management:

#### Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - RAN4 define different separate for measurement period for the UE is configured or not for “timeRestrictionForChannelMeasurement” - To check with RAN1 whether the parameter can be configured if RAN1 identify any issue #### Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams. - Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air. - UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period by assuming TReport =0. #### Issue 1-4: CSI-RS based L1-RSRP for inter-cell BM? Agreement: - Reuse the same requirements structure as that in Rel-17 clause 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements. #### Issue 1-5: Whether to specify new unified TCI switching delay requirements? Agreement: - The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management - Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion. #### Issue 1-7: Requirements for event-1 and event-7? Agreement: - For Event-1 and Even-7, reuse the same design as the requirements defined Event-2

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

<Way forward >

- Option 1: (Qualcomm, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

RAN1 has introduced evaluation of event based on a time window and event count to ensure a reliable event trigger condition and reporting, rather than reporting based on instantaneous changes in power. In a specified time window if at least eventInstanceCount-r19 number of event instances occur, then the UE sends a report. In terms of minimum requirements, it is impractical to introduce a requirement with eventInstanceCount-r19>1. As illustrated in the example below, for the evaluation window eventInstanceCount-r19 = 3. In case 1, the reporting delay is shorter than in case 2. What is the requirement for reporting delay defined as when eventInstanceCount-r19 = 3? There could be a case when condition is met only twice in the evaluation window, and no report is sent.

It is not feasible to define delay requirements based on evaluation window and can only be defined based on no time window configured or eventInstanceCount-r19 =1. Also, the purpose of defining requirements with eventInstanceCount-r19 >1 is unclear. What does it help verify at the UE?

- It is not feasible to define core requirement for reporting delay with eventInstanceCount-r19 >1.

- The purpose of defining requirements based on evaluation window/ counter is unclear.

Measurement reporting delay requirement for UE initiated beam report is defined for eventInstanceCount-r19 =1.

To extend the requirements to window-based evaluation, we can add a note that longer reporting delay is expected if eventInstanceCount-r19 > 1 is configured.

To extend the requirements for case when eventInstanceCount-r19 >1 is configured based on UE capability, add note longer delay is expected if eventInstanceCount-r19 >1 is configured.

#### Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

<Way forward >

- Option 1: (Nokia, Ericsson, MTK)

  - No need to capture this scenario in RAN4 requirements

- Option 2: (Xiaomi, Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

The assumption or applicability for event triggered L1 measurement needs to be defined. We propose the following conditions for the reporting delay requirements to be applicable:

- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Measurement reporting delay requirement applicability
- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE 
- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

#### Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

- Option 1: (Samsung, Apple, Huawei, Ericsson)

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2: (Nokia, MTK)

  - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

There is no necessity to define capabilities for Support of Event Triggering and Reporting Criteria for UEIBR.

For event trigger L1 reporting, do not define such capability in parallel per category like L3.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

Reporting Delay Requirements

- It is not feasible to define core requirement for reporting delay with eventInstanceCount-r19 >1

- The purpose of defining requirements based on evaluation window/ counter is unclear.

- Measurement reporting delay requirement for UE initiated beam report is defined for eventInstanceCount-r19 =1.

To extend the requirements for case when eventInstanceCount-r19 >1 is configured based on UE capability, add note longer delay is expected if eventInstanceCount-r19 >1 is configured.

Applicability

Measurement reporting delay requirement applicability
- The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE 
- The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

Capabilities

For event trigger L1 reporting, do not define such capability in parallel per category like L3.

# Reference

- R4-2508271, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.