3GPP TSG-RAN WG4 Meeting # 116	R4-2509504

# Bangalore, India, 25 Aug - 29 Aug, 2025

Agenda item:	7.14.2

Source:	Apple

Title:	On feature list for RRM phase 5 WI

Document for:	Discussion

# Introduction

In this contribution, we discuss the UE capability and feature groups for RRM phase 5 WI.

# Detailed design for capability of BSF enhancement

In the last meeting, the following WF was agreed:

## Sub-topic 1-1 RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF ### Issue 1-1-3: UE capability for Rx BSF optimization Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

As analyzed in our contribution on BSF, we see no strong need to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency). Therefore, the following capability is proposed:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-1 | Fast Rx beam sweeping for FR2-1 L3 measurement delay reduction | Supports beam sweeping factor reduction for L3 measurement for FR2-1. |  | Yes | N/A |  | Per band | TDD only | FR2-1 only |  | Candidate values: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |

# Detailed design for capability of CSSF enhancement

In previous meeting, the agreements for solution 1 and solution 3 under CSSF enhancement were:

Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, - the 3rd searcher resource sharing is same as the second searcher sharing. Not support solution 2 in Rel-19. Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Since RAN4 has agreed the capability design for solution 3, here we focus on solution 1. Based on the template shared by moderator of the feature list thread, the following detailed capability design for solution 1 is proposed.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | [49-x] | L3 serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap | Support of serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. Support of separate indications for FR1 and FR2-1. |  | Yes |  | UE does not support serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are [configured in a same band]. | Per-UE | No | Yes |  | UE indicating this capability shall meet the corresponding enhanced requirements in TS38.133 [Clause 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4] | Optional with capability signaling |

# Detailed design for capability of EMR enhancement

UE capability was discussed in the previous meeting. It is still open:

### Issue 2-1-1: UE capability for EMR-based fast SCell activation

FFS:

- Option 1: Introduce a UE capability for R19 SCell activation based on EMR

  - Prerequisite feature groups: 39-8 or 39-9 in R18 RAN4 feature list

  - Need for the gNB to know if the feature is supported: No

  - Granularity: Per-UE

  - FDD/TDD differentiation: No

  - FR1/FR2 differentiation: Yes

- Option 2: Not to introduce additional UE capability

Note that EMR enhancement reporting was introduced in R18. With enhanced EMR report, Scell activation latency can be reduced since the Scell is considered as ‘known’ after UE report. in other word, R18 UE can meet known cell activation delay requirement. However, the new requirements which are introduced in R19 are tightened compared with legacy known Scell activation delay (number of samples are reduced). Therefore, further enhancement is needed for UE supporting R18 EMR in order to support R19 EMR Scell activation delay requirements. So a new UE capability is necessary.

Observation 1: further enhancement is needed for UE supporting R18 EMR in order to support R19 EMR Scell activation delay requirements.

A new UE capability is necessary for R19 EMR enhancement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | Fast SCell activation based on early measurement report | Supports fast SCell activation specified in TS38.133 clause 8.3.2A | 39-8 or 39-9 in R18 RAN4 feature list | No | N/A | UE cannot meet fast SCell activation requirements as specified in TS38.133 clause 8.3.2A | Per-UE | No | No |  |  | Optional without capability signaling |

# Conclusion

In this contribution, we discuss the UE capability and feature groups for RRM phase 5 WI.

# References