3GPP RAN WG4 Meeting #116																	R4-2509536

Bangalore, India, 25th – 29th August 2025

Agenda item:	7.6.3.2.1

Source:	Apple

Title:	Power back-off results for PC2 on NTN L-band

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable higher power class PC2 for the satellite bands including L-band(s), e.g. n255. Thus, in this discussion paper we present our preliminary power back-off simulation results for the PC2 power class operating in band n255 frequency range.

# 2	Power back-off simulations for NTN L-band

## 2.1	Simulation assumptions and scenarios

Satellite L-band n255 was standardised during Rel-17, whereupon only the PC3 power class was assumed and for which in addition to the general requirements also the NS_02N was defined corresponding to the FCC requirements to protect the GNSS services. Here we present further results for the PC2 power class with the common assumptions as presented below:

-	Tx power: PC2 (+26dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: Requirements from NS_02N

For the sake of brevity, we ran simulations for the 5, 10, 15 and 20MHz channel bandwidths, whereupon the corresponding channel is always located at the lower edge of the n255 band:

-	5MHz: Fc=1629MHz

-	10MHz: Fc=1631.5MHz

-	15MHz: Fc=1634MHz

-	20MHz: Fc=1636.5MHz

## 2.2	Power back-off simulations

According to the preliminary simulation results, no additional power back-off is needed for the 5 and 10MHz channel bandwidths. Thus, the corresponding figures for these channel bandwidths will not be presented.

As for the 15 and 20MHz channels, some power back-off is needed as can be seen from Figure 2.2-1 and Figure 2.2-2 below.  Nevertheless, the 15MHz channel power back-off is quite marginal and is within the existing MPR margins. As for the 20MHz channel, it naturally experiences higher power back-off, which in some cases reaches existing MPR margins, especially for the full channel and single RB edge allocations.

Figure 2.2-1: Power back-off results for 15MHz at Fc=1634MHz.

Figure 2.2-2: Power back-off results for 20MHz at Fc=1636.5MHz.

# 3	Conclusions

In this discussion paper we have presented our power back-off simulation results for the NTN band n255, PC2 power class. Based on the presented results the following observations could be made:

Observation 1:	For n255 PC2 NS_02N, neither the 5MHz nor the 10MHz channel need any power back-off.

Observation 2:	For n255 PC2 NS_02N, the 15MHz channel needs power back-off, which is however within the existing MPR margins, i.e. there is no need to define A-MPR.

Observation 3:	For n255 PC2 NS_02N, the 20MHz channel needs power back-off, which is close to the existing MPR margins (which in turn may require defining A-MPR).

Proposal:	Account for the presented power back-off simulations while defining band n255 PC2 RF requirements.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"