3GPP RAN WG4 Meeting #116		R4-2509544

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.4.3

Source:	Apple

Title:	On the introduction of 7MHz CBW

WI/SI:	NR_FR1_7MHz_BW-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The WI [1] has the goal to introduce 7MHz channel bandwidth for n26 and n5. For the initial discussion this contribution focuses on selected objectives from the WID. Before any further analysis can be made it is important to specify the spectrum utilization. Another important objective is to determine the impact on network signaling label so that the A-MPR work can be started.

# 2  Discussion

##### 2.2 Introduction of 7MHz CBW to NS_12

During RAN4#115, several proposals for A-MPR regions and back-off values were consolidated. We propose to finalise the agreement on the tentative NS_12 regions and the A-MPR values for PC3 and PC2 as provided in WF [2].

Proposal 1: NS_12: Finalise the PC3 and PC2 regions and A-MPR by agreeing on the proposed values RAN4#115 WF.

##### 2.3 Introduction of 7MHz CBW to NS_13

During RAN4#115, several proposals for A-MPR regions and back-off values were consolidated. We propose to finalise the agreement on the tentative NS_13 regions and the A-MPR values for PC3 and PC2 as provided in WF [2].

Proposal 2: NS_13: Finalise the PC3 and PC2 regions and A-MPR by agreeing on the proposed values RAN4#115 WF.

##### 2.5 Introduction of 7MHz CBW to NS_15

The WF [2] from RAN4#115 captures the following agreement for PC3 NS_15:

The A-MPR was mostly agreeable but left for further checking. Two regions needed more thought as the proposed values were too divergent. The basic approach from our contribution [4] was to re-use the existing A-MPR and not to define additional values. If RAN4 wants to introduce new A-MPR we would like to propose the following compromise. If acceptable the A-MPR for the two debated regions might result into:

Table 1: NS_15 PC3 A-MPR based on proposals from last meeting

| Modulation/Waveform | Compromise proposal for [A1 or A7] | Compromise proposal for [A3 or A8] |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.0 | ≤ 1.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | ≤ 2.0 |
| DFT-s-OFDM 16 QAM | ≤ 8.0 | ≤ 2.5 |
| DFT-s-OFDM 64 QAM | ≤ 8.0 | ≤ 2.5 |
| DFT-s-OFDM 256 QAM | ≤ 8.0 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 9.0 | ≤ 3.0 |
| CP-OFDM 16 QAM | ≤ 9.0 | ≤ 3.0 |
| CP-OFDM 64 QAM | ≤ 9.0 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 9.0 | ≤ 6.5 |

Proposal 3: NS_15 PC3: Finalise the PC3 regions and A-MPR by agreeing on the compromise proposal from Table 1.

Proposal 4: NS_15 PC2: Finalise the PC2 regions and A-MPR by agreeing on the proposed values RAN4#115 WF.

# 4	Conclusions

This contribution provides initial insight and discussion points for the introduction of 7MHz channel. The following proposals are made:

Proposal 1: NS_12: Finalise the PC3 and PC2 regions and A-MPR by agreeing on the proposed values RAN4#115 WF.

Proposal 2: NS_13: Finalise the PC3 and PC2 regions and A-MPR by agreeing on the proposed values RAN4#115 WF.

Proposal 3: NS_15 PC3: Finalise the PC3 regions and A-MPR by agreeing on the compromise proposal from Table 1.

Table 1: NS_15 PC3 A-MPR based on proposals from last meeting

| Modulation/Waveform | Compromise proposal for [A1 or A7] | Compromise proposal for [A3 or A8] |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.0 | ≤ 1.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | ≤ 2.0 |
| DFT-s-OFDM 16 QAM | ≤ 8.0 | ≤ 2.5 |
| DFT-s-OFDM 64 QAM | ≤ 8.0 | ≤ 2.5 |
| DFT-s-OFDM 256 QAM | ≤ 8.0 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 9.0 | ≤ 3.0 |
| CP-OFDM 16 QAM | ≤ 9.0 | ≤ 3.0 |
| CP-OFDM 64 QAM | ≤ 9.0 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 9.0 | ≤ 6.5 |

Proposal 4: NS_15 PC2: Finalise the PC2 regions and A-MPR by agreeing on the proposed values RAN4#115 WF.

# 4	References

- RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA, 3GPP TSG RAN Meeting #106

- R4-2507954, “WF on 7 MHz channel BW”, Ericsson, 3GPP TSG-RAN4 Meeting #115

- R4-2504186, “Draft CR to 38.101-1: Introduction of requirements for PC2 operation in band n26”, Ericsson, 3GPP TSG-RAN WG4 Meeting#114bis

- R4-2505428, “On the introduction of 7MHz CBW”, Apple, 3GPP RAN WG4 Meeting #115