3GPP TSG-RAN WG4 Meeting #116                                       	                        R4-2509572

# Bangalore, India, August 25 – August 29, 2025

Agenda item:	7.1.2.3.2

Source:	Apple Inc.

Title:	Discussion on Release Independence for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

  - During the past few RAN4 meetings it has been agreed that RAN4 is unable to conclude on simulation assumptions to determine feasibility for handheld UEs. Because of this, it was also agreed that 6-layer performance requirements will be only defined for FWA, and 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. Afterwards, RAN4 sent a LS to RAN2 to discuss how to capture the fact that 6 MIMO layers is only supported by 6Rx FWA devices. For this discussion paper, we focus on our view on release independence of the feature.

# Discussion

During RAN4#114 companies debated largely about the feasibility of 6 MIMO layers for 6Rx handheld UEs. After many months of discussions, it was finally agreed that RAN4 was “unable to conclude on simulation assumptions to determine feasibility for handheld UEs”. Hence, 6-layer performance requirements are being defined for FWA and 4-layer performance requirements are being defined and will apply to FWA and handheld UE devices.

In addition, during RAN4#114bis it was agreed to send an LS to RAN2 to discuss capability signalling. In this regard, we leave this discussion to colleagues in the RAN2 WG.

  - The last topic that needs to be discussed as part of core requirements is release independence. In our view, 6Rx UE is a Release 19 feature and should be treated as such. The implementation of SU-MIMO is already quite involved in terms of UE architecture and implementation, especially in RF front end, channel estimation, MIMO detection, and in 2 CW processing at the LDPC decoder stage. Hence, we prefer to maintain the working assumption that the feature is specific to Release 19 and beyond.

  - Observation #1: The implementation of 6Rx UE is quite involved in terms of UE architecture and modem implementation in comparison to 4Rx.

  - In addition, during the past RAN4#115 meeting, release independence was also brought to the table in terms of SRS requirements in the RF room. In the end, that discussion did not advance since there was no consensus of a need to extend such requirements to earlier releases. Those SRS requirements go hand in hand to 6Rx UE related aspect in the DL direction.

Observation #2: 6Rx with 4 and 6 MIMO layer UE feature go hand in hand with SRS requirements, the latter not been deemed needed to be release independent during RAN4#115.

  - Because of this, we believe we have one more reason in addition to what we have argued in previous meetings. This solidifies our position that 6Rx with 4 or 6 MIMO layers does not need to be a release independent feature, and RAN4 should conclude the topic in this meeting.

Proposal #1: Do not define 6Rx UE with 4 and 6 MIMO layers as a release independent feature.

Proposal #2: RAN4 to decide in RAN4#116 whether 6Rx UE with 4 and 6 MIMO layers should be a release independent feature.

# Conclusions

Our observations and proposals are summarized below:

  - Observation #1: The implementation of 6Rx UE is quite involved in terms of UE architecture and modem implementation in comparison to 4Rx.

Observation #2: 6Rx with 4 and 6 MIMO layer UE feature go hand in hand with SRS requirements, the latter not been deemed needed to be release independent during RAN4#115.

Proposal #1: Do not define 6Rx UE with 4 and 6 MIMO layers as a release independent feature.

  - Proposal #2: RAN4 to decide in RAN4#116 whether 6Rx UE with 4 and 6 MIMO layers should be a release independent feature.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024