3GPP TSG-RAN4 Meeting #116	        R4-2509579 Bengaluru, India, 25th August – 29th August 2025

Source: 	Qualcomm Incorporated

Title: 	7 MHz Channel Bandwidth A-MPR Analysis for band n26

Agenda Item:	7.4.3

Document for:	Discussion

WI Code:	NR_FR1_7MHz_BW-Core

# Introduction

RAN4 agreed that the signalling labels NS_12, NS_13 NS_14, and NS_15 should be supported in band n26 for a 7 MHz channel bandwidth. The region definitions for a power class 3 UE were agreed, for a power class 2 were tentatively agreed and the A-MPR numbers were left for further study [1]. In this proposal, we present a A-MPR analysis based on measurements and simulations performed in band n26 for a power class 3 (PC3) and a power class 2 (PC2) UE.

# Bench Measurements and Simulation Study

For analyzing the implications on Tx performance, we measured and simulated [2,3] the impact on achievable output power. PC3 and PC2 transmitter was calibrated to just meet the ACLR with DFT-s-OFDM QPSK waveform with Start_RB = 0 and L_CRB = 100 and MPR = 1 dB. Other parameters are set up as follows :

- Channel Bandwidth = 7 MHz

- SCS = 15 kHz

- Maximum # of RBs = 35

- Band of Operation : n26 (Uplink : 814 – 849 MHz)

- Carrier Frequency : Varied based on observed NS label

- LO leakage = 28 dBc

- Image = 28 dBc

- CIM3 = 60 dBc

- Network Signalling Labels : NS_12, NS_13, NS_14 and NS_15

- Waveforms : CP-OFDM and DFT-s-OFDM

- Modulation : QPSK, 16 QAM, 64 QAM and 256 QAM

Results are presented below and are divided based on the power class.

Power Class 3

NS_12 :

Carrier frequency of 817.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth. New A-MPR values in Table 2 are highlighted.

Table 1: A-MPR regions for NS_12 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
| 7 MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A2 |
|  |  | ≤1.8 | >0.72 | A3 |
|  |  | >1.8 | >1.44 | A2 |
| 10MHz |  | ≤3.6 | >0 | A1 |

Table 2: A-MPR for NS_12 (Power Class 3)

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | ≤ 5.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 9.5 |  | ≤ 7 |

Proposal 1: Incorporate A-MPR impact due to NS_12 as proposed in Tables 1 and 2.

NS_13 :

Carrier frequency of 820.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth. New A-MPR values in Table 4 are highlighted.

Table 3: A-MPR regions for NS_13 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
| 7MHz | Fc = 820.5 | ≤1.8 | <1.44 | A2 |
|  |  | ≤1.8 | ≥1.44 | A2 |
|  |  | >1.8 | ≥1.8 | A5 |

Table 4: A-MPR for NS_13 (Power Class 3)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 4.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  | ≤ 5.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  | ≤ 8 |

Proposal 2: Incorporate A-MPR impact due to NS_13 as proposed in Tables 3 and 4.

NS_14 :

Carrier frequency of 827.5 MHz is used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth and no impact on A-MPR is observed.

Proposal 3: A-MPR is not impacted for signalling label NS_14.

NS_15 :

Carrier frequencies of 845.5 MHz and 841.5 MHz are used to evaluate A-MPR behavior over the entire allocation space of 7 MHz channel bandwidth. Updates compared to the agreement reached in [1] are in bold in Table 5 based on the measurements performed and new A-MPR values in Table 6 are highlighted.

Table 5: A-MPR regions for NS_15 (Power Class 3)

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A5 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.5 | >0 | A1 |
|  |  | ≥2.16, <4.5 | ≥1.8 | A2 |
|  |  | ≤1.44 | ≤0.36 >0 | A1 |
|  | 837 < Fc ≤ 841.5 | >0 | >2.7 | A2 |
|  |  | ≥5.94 | ≤2.7 | A7 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6: A-MPR for NS_15 (Power Class 3)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | ≤ 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | ≤ 2 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | ≤ 2.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | ≤ 3 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | ≤ 3.5 |

Proposal 4: Incorporate A-MPR impact due to NS_15 as proposed in Tables 5 and 6.

Power Class 2

NS_12 :

Carrier frequency of 817.5 MHz is used to evaluate A-MPR behavior for a PC2 UE over the entire allocation space of 7 MHz channel bandwidth. New A-MPR values in Table 8 are highlighted.

Table 7: A-MPR regions for NS_12 (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 815.5 ≤ Fc < 817.5 | ≤0.9 | >0 | A3 |
|  | 817.5 ≤ Fc < 818.5 | ≤0.18 | >0 | A2 |
| 5MHz |  | ≤1.8 | >0 | A1 |
|  |  | >1.8 | >1.08 | A2 |
| 7 MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A4 |
|  |  | >1.8 | >1.44 | A3 |
| 10MHz |  | ≤3.6 | >0 | A1 |
|  |  | >3.6, ≤5.04 |  | A2 |

Table 8: A-MPR for NS_12 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 | ≤ 7.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 | ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 4 | ≤ 6 | ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4.5 | ≤ 6.5 | ≤ 7.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 6.0 | ≤ 6.5 | ≤ 8.5 |
| CP-OFDM QPSK | ≤ 9 | 5.0 | ≤ 7.5 | ≤ 8.5 |
| CP-OFDM 16 QAM | ≤ 9 | 5.0 | ≤ 7.5 | ≤ 8.5 |
| CP-OFDM 64 QAM | ≤ 9 | 5.0 | ≤ 7.5 | ≤ 8.5 |
| CP-OFDM 256 QAM | ≤ 11.5 | 7.0 | ≤ 7.5 | ≤ 9.5 |

Proposal 5: Incorporate A-MPR impact due to NS_12 for PC2 UE as proposed in Tables 7 and 8.

NS_13 :

Carrier frequency of 820.5 MHz is used to evaluate A-MPR behavior for a PC2 UE over the entire allocation space of 7 MHz channel bandwidth.

Table 9: A-MPR regions for NS_13 (Power Class 2)

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 818.5 ≤ Fc < 820 | ≤0.54 | >0 | A1 |
|  |  | >0.54, ≤ 1.08 | ≥1.08 | A4 |
|  | 820 ≤ Fc < 821 | ≤0.18 | >0 | A4 |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
|  |  | >1.44 | ≥1.8 | A3 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |
|  |  |  | <3.24, ≥2.52 | A4 |
| 7MHz | Fc = 820.5 | ≤1.8 | <1.44 | A1 |
|  |  | ≤1.8 | ≥1.44 | A2 |
|  |  | >1.8 | ≥1.8 | A1 |

Table 10: A-MPR for NS_13 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 7 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 7.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7.5 | ≤ 8 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 10 | ≤ 10 | ≤ 7 |  |

Proposal 6: Incorporate A-MPR impact due to NS_13 for a PC2 UE as proposed in Tables 9 and 10.

NS_14 :

Carrier frequency of 827.5 MHz is used to evaluate A-MPR behavior for a PC2 UE over the entire allocation space of 7 MHz channel bandwidth and no impact on A-MPR is observed.

Proposal 7: A-MPR is not impacted for a PC2 UE for signalling label NS_14.

NS_15 :

Carrier frequencies of 845.5 MHz and 841.5 MHz are used to evaluate A-MPR behavior for a PC2 UE over the entire allocation space of 7 MHz channel bandwidth. Updates compared to the tentative agreement reached in [1] are in bold in Table 11.

Table 11: A-MPR regions for NS_15 (Power Class 2)

| Channel BW | Carrier Frequency, Fc MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 844.5 < Fc ≤ 847.5 | ≥1.44 | >0 | A2 |
|  |  | ≤ 0.18 | ≤ 0.36 | A6 |
|  | 843.0 < Fc ≤ 844.5 |  | ≥ 1.8 | A5 |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
|  |  | <2.53, ≥1.8 | ≥1.44 | A5 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
|  | 837.5 < Fc ≤ 840.5 |  | ≥ 3.0 | A5 |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | A4 |
|  |  | ≥2.52, <4.68 | ≥2.16 | A2 |
|  |  | ≥2.52, <4.68 | ≥1.44, <2.16 | A6 |
|  |  | <1.8 | ≤0.36 >0 | A1 |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | A2 |
|  |  | ≥5.94≥5.58 | <2.7 | A5 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  |  | <4.14, ≥1.8 | ≥2.52 | A5 |
|  |  | <2.52 | >0.36, ≤0.72 | A5 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
|  |  | <7.2, ≥5.22 | <4.32, ≥3.24 | A5 |
|  |  | <1.44 | >0.36, ≤0.72 | A5 |
|  | 829 < Fc ≤ 835 |  | ≥ 5.4 | A7 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  |  | <4.68, ≥1.8 | ≥2.52 | A5 |
|  |  | <3.96 | >0.36, ≤1.08 | A5 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  |  | <2.7 | >0.36, ≤0.72 | A5 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
|  |  | <1.08 | >0.36, ≤0.72 | A5 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  |  | < 8.46, ≥1.8 | ≥3.6 | A5 |
|  |  | <6.12 | >0.36, ≤1.08 | A5 |
|  |  | <12.24, ≥8.46 | < 5.4, ≥3.6 | A5 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |
|  |  | <4.68 | >0.36, ≤1.08 | A5 |

Table 12: A-MPR for NS_15 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3.5 | ≤ 4.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 4 | ≤ 5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 5 | ≤ 6 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 5 | ≤ 6 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 | ≤ 7 |

Proposal 8: Incorporate A-MPR impact due to NS_15 for a PC2 UE as proposed in Tables 11 and 12.

# Conclusions

In this contribution A-MPR analysis with network signalling labels NS_12, NS_13, NS_14 and NS_15 is presented in band n26 for 7 MHz channel bandwidth. Following proposals are made to update the A-MPR tables.

Power Class 3

Proposal 1: Incorporate A-MPR impact due to NS_12 as proposed in Tables 1 and 2.

Proposal 2: Incorporate A-MPR impact due to NS_13 as proposed in Tables 3 and 4.

Proposal 3: A-MPR is not impacted for signalling label NS_14.

Proposal 4: Incorporate A-MPR impact due to NS_15 as proposed in Tables 5 and 6.

Power Class 2

Proposal 5: Incorporate A-MPR impact due to NS_12 for a PC2 UE as proposed in Tables 7 and 8.

Proposal 6: Incorporate A-MPR impact due to NS_13 for a PC2 UE as proposed in Tables 9 and 10.

Proposal 7: A-MPR is not impacted for a PC2 UE for signalling label NS_14.

Proposal 8: Incorporate A-MPR impact due to NS_15 for a PC2 UE as proposed in Tables 11 and 12.

# References

[1]	R4-2420481, “WF NR_FR1_7MHz_BW”, Ericsson, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th –  23rd May, 2025.

[2]    R4-2507651, “Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 23rd May, 2025.

[3]    R4-2507653, “PC2 UE : Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 23rd May, 2025.