3GPP TSG-RAN WG4 Meeting #116	R4-2509612

Bengaluru, India, August 25 – 29, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	FR2-1 SSB based L3 measurement delay reduction with CSSF optimization

Agenda item:	7.14.3.2

Document for:	Discussion

Introduction

The discussion about RRM phase 5 enhancements for L3 measurement delay reduction started in the RAN4#110bis meeting. At RAN4#115 meeting, the draft Big CR was endorsed in [1] capturing two optimized CSSF solutions, but some details are still FFS in [2]. In this contribution we will continue discussing the leftover issues.

Solutions 1: measuring one serving CC per band

How to indicate SCC per band (RAN2 reply LS)

RAN2 have discussed the RRC configuration to support the CSSF optimization but didn’t conclude how to indicate a specific SCC per band for measurement. A reply LS was sent from RAN2 informing two alternatives and asking RAN4 to make decision [3].

RAN2 reply LS on CSSF optimization (R2-2504728): About the network indication of a specific SCC per-band for measurement, RAN2 discussed the following two alternatives: - Alternative 1: Introduce new RRC configuration to indicate which SCC to be measured. - When CSSF optimization feature is enabled, UE only performs the serving and intra-frequency neighbor measurement on the indicated SCC per band. - Alternative 2: No additional RRC signaling is needed. - When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement. - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.. Majority’s view in RAN2 is Alternative 2, but RAN2 did not reach a consensus. Therefore, RAN2 would like to ask RAN4 to decide which alternative to adopt for SCC indication and provide feedback during August meeting so that RAN2 can prepare the RRC CR for approval in August meeting.

In our understanding, this signaling was introduced in RAN4 to allow network to control which SCC the UE shall measure when this CSSF optimization feature is enabled. This is on the condition UE is configured to measure multiple SCCs on one band. In Alterative 2, in order to indicate specific SCC to measure, network is required to change the measurement configuration e.g. by configuring measurement reporting on single SCC of the band. However, the measurement configuration has been legacy behavior which depends on the network implementation. By changing existing measurement configuration, it is unclear to UE if the change is due to legacy network behaviour or because of the new CSSF optimization feature. In addition, network is likely to implement the feature as a new module, and the impact to legacy behavior shall be minimized to avoid over-complicated system design. Therefore, we prefer Alternative 1 which provides a clean approach for indicating a specific SCC to measure.

Observation #1: Network is likely to implement the CSSF optimization feature as a new module, and the impact to legacy behavior shall be minimized to avoid over-complicated system design.

- Prefer Alternative 1 which provides a clean approach for indicating a specific SCC to measure.

Revision on solution 1

In last meeting, there was a proposal to remove one condition where UE shall follow the network indicated SCC to be measured. It was argued the measurement shall be always configured with reporting hence the condition “if none of SCCs is configured with SSB based measurement reporting” is not a valid case. This was discussed in RAN4#115 meeting and didn’t reach any consensus.

Issue 1-1-1: revision on solution 1 [Wayforward]: Options of this meeting are captured in topic summary R4-2505561. - Option 1 (vivo, OPPO): For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Option 2 (Ericsson, ZTE, Nokia): Not to remove the phrase ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’. - (ZTE)NW indication of interested SCC is meaningful even only multiple SCCs without SSB based reporting are configured in the band. Under such case, the NW indication is used to indicate which serving cell to be measured and reported at least. - (Nokia): UE shall follow network indication to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band - Option 3 (Huawei): Discuss this revised Solution 1 update after the indication design is finalized.

According to existing measurement requirements in FR2, when neither PCC nor PSCC is in the same band, the UE will select one of SCCs on which UE is configured to report SSB based measurements if such SCC exists. Otherwise, which means there is no SCC on which UE is configured to report SSB based measurements, the selected SCC is up to UE implementation. This means UE is still required to select and measure one SCC on the band but it is not testable as no measurement reporting is configured.

Observation #2: In legacy requirements, when there is no SCC on which UE is configured to report SSB based measurement, UE is still required to select and measure one SCC on the band.

TS 38.133 clause 9.2.3.2:

9.2.3.2	Requirements for FR2

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	6 identified cells, and

-	24 SSBs with different SSB index and/or PCI,

where this single intra-frequency layer shall be:

-	PCC when UE is configured with SA NR operation mode with PCC in the band; or

-	PSCC when UE is configured with EN-DC with PSCC in the band; or

-	PSCC when UE is configured with NR-DC with PSCC in the band; or

-	One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

Similarly, when optimized measurement is enabled and network indicates which SCC to measure on the band, even if no measurement reporting is configured, UE is required to measure the indicated SCC. This should be independent from if UE is configured with SSB-based measurement reporting. It may not be testable if UE is not configured with SSB-based measurement reporting on the indicated SCC, but the rule for determining the selected SCC remains valid. Hence, the above case where UE follows network indication shall not be removed. UE shall follow network indication (if present) to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band.

Observation #3: It may not be testable if UE is not configured with SSB-based measurement reporting on the indicated SCC, but the rule for determining an SCC to measure on the band remains valid.

Some companies further argued that in RAN2 specification a UE shall perform measurements for each MeasId which is associated with one measurement reporting configuration. But RAN2 does not tell the UE shall not measure if measurement reporting is not configured. The UE measurement behavior and requirement shall follow RAN4 specification. Even when there is no SCCs on which UE is configured with measurement reporting, network may also indicate which SCC to measure and the UE is expected to measure the indicated SCC on the band accordingly.

- UE shall follow network indication (if present) to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band.

Solution 1 in FR1 scenarios

At RAN4#113 meeting, it was agreed to further discuss if the CSSF optimization solutions can be extended to FR1 after it is concluded in FR2. In last meeting, there was concern from companies about the feasibility of such extension.

RAN4#113 meeting: Issue 2-7: Other WID scope discussion Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable RAN4#114bis meeting: Issue 1-1-3: Expand solution 1 to FR1 case [Wayforward]: Further discuss in next meeting: If extend for FR1: Aspect 1: co-located or non-collocated - Option 1: only limit to serving cell measurement (E///) - Option 2: cover both serving cell and neighboring cell measurements - In clause 7.6.4 of TS 38.133, it clearly sys only co-located deployment is applied for intra-band contiguous CA. - Companies are encouraged to check the spec in this meeting. Aspect 2: Network configuration on enable/disable the feature - Option 1: the configuration is the same for different FRs - Option 2: the configuration can be different for different FRs (Nokia, ZTE, China Telecom, Apple, HW, E///) - Under the condition that the UE capability is per FR [or per band]. - Option 2 will be adopted if the solution 1 will be extended to FR1.

Regarding to Aspect 1, as specified in TS 38.133 clause 7.6.4, only co-located deployment is applied for FR1 intra-band contiguous CA. In the discussion of SCell activation, we have been assuming the same timing and channel status among contiguous carriers in one FR1 band. Taking SSB-less SCell activation as an example, the SSB-less SCell can be activated immediately (i.e. 3ms) if there is at least one contiguous active serving cell in the same band. This shows the very good synergy among the intra-band contiguous carriers in FR1. Hence, we believe the same principle of solution 1 can be applied at least to FR1 collocated intra-band contiguous CA. For other FR1 collocated scenarios, we may leave it out from this WI for now considering the timeline.

Observation #4: As specified in TS 38.133 clause 7.6.4, only co-located deployment is applied for FR1 intra-band contiguous CA. This shows the very good synergy among the intra-band contiguous carriers in FR1.

- Extend solution 1 to FR1 collocated intra-band contiguous CA.

In existing specifications, UE is already allowed to skip the neighbor cell measurement on some SCCs in one FR2 band as specified in TS 9.2.3.2. But this does not apply to the measurement on FR1 i.e. UE is required to measure both serving and neighbor cells in one FR1 carrier in the same way. When solution 1 is enabled for FR1, UE only needs to measure one serving CC per FR1 contiguous band as these serving cells are collocated. But it is open how and if the UE is allowed to skip the neighbor cells measurement. We are open to discussing this issue.

Observation #5: In existing specifications, UE is not allowed to skip the neighbor cell measurement on some SCCs in one FR1 band.

- RAN4 to discuss how to handle the neighbor cell measurement when network indicates to enable solution 1 for FR1 bands.

About Aspect 2 above, to allow flexible control on the optimized measurement, network may indicate to enable or disable this CSSF enhancement for FR1 and FR2 separately.

- Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

- Clarification on Interruption requirements

In existing specification TS 38.133 clause 8.2, interruptions on PCell or activated SCells are allowed due to measurements when an SCC is deactivated. However, when the CSSF enhancement solution 1 is enabled, UE is required to measure only one serving CC per FR2 band. We would like to clarify that interruptions on PCell or activated SCells due to measurements when an SCell is deactivated shall be allowed only if the UE is required to measure the SCC. In other words, the UE may still perform measurements on other non-selected SCCs up to UE implementation, but this shall not bring any interruptions on PCell or activated SCells. The clarification is provided in our companion CR [4].

TS 38.133 clause 8.2.1

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

- When UE is enabled to measure one serving CC per band, RAN4 to clarify that interruptions on PCell or activated SCells due to measurements when an SCell is deactivated shall be allowed only if the UE is required to measure the SCC.

Conclusion

The following observations and proposals were made in this paper:

Observation #1: Network is likely to implement the CSSF optimization feature as a new module, and the impact to legacy behavior shall be minimized to avoid over-complicated system design.

- Prefer Alternative 1 which provides a clean approach for indicating a specific SCC to measure.

Observation #2: In legacy requirements, when there is no SCC on which UE is configured to report SSB based measurement, UE is still required to select and measure one SCC on the band.

Observation #3: It may not be testable if UE is not configured with SSB-based measurement reporting on the indicated SCC, but the rule for determining an SCC to measure on the band remains valid.

- UE shall follow network indication (if present) to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band.

Observation #4: As specified in TS 38.133 clause 7.6.4, only co-located deployment is applied for FR1 intra-band contiguous CA. This shows the very good synergy among the intra-band contiguous carriers in FR1.

- Extend solution 1 to FR1 collocated intra-band contiguous CA.

Observation #5: In existing specifications, UE is not allowed to skip the neighbor cell measurement on some SCCs in one FR1 band.

- RAN4 to discuss how to handle the neighbor cell measurement when network indicates to enable solution 1 for FR1 bands.

- Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately.

- When UE is enabled to measure one serving CC per band, RAN4 to clarify that interruptions on PCell or activated SCells due to measurements when an SCell is deactivated shall be allowed only if the UE is required to measure the SCC.

- References

- R4-2508457, Draft big CR for NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2508264, Way Forward for NR_RRM_Ph5_Part1, Apple

- R2-2504728, Reply LS on CSSF optimization for NR RRM Phase 5, Apple

- R4-2509613, draftCR on CSSF optimization solutions, Nokia, Nokia Shanghai Bell