3GPP TSG-RAN WG4 Meeting #116	R4-2509616

Bengaluru, India, August 25 – 29, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for UE to UE CLI handling

Agenda item:	7.21.4.1

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, many agreements were made in the WF [1]. We will continue discussing the open issues on L1 UE-to-UE CLI measurement in this contribution.

2. L1-SRS-RSRP measurement

  - Measurement Restriction

On measurement restriction, the following agreements were reached. It is open how to handle the CLI measurement if the measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping.

- RAN4#114bis Agreements - Measurement requirements do not apply for L1-SRS-RSRP when the measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - When measurement resources for L1-SRS-RSRP and L1-CLI-RSSI are partially or fully overlapping, UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI, FFS the following options - Option 1: Prioritize L1-SRS-RSRP measurement - Option 2: Prioritize L1-CLI-RSSI measurement - Option 3: Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI - RAN4#115 Agreement - Take into account following RAN1 agreement in RAN1#121, and further discuss the measurement restriction between Method #1 and Method #2. Agreement (RAN1#121) A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol..

Based on the latest RAN1#121 agreement above, a UE is not expected to be configured with L1-CLI-RSSI measurement resources within an UL subband (Method#3) and L1-SRS-RSRP measurement resources on the same symbol (Method#2). However, the case of overlapping between L1-CLI-RSSI measurement resources within one or two DL subbands (Method#1) and L1-SRS-RSRP measurement resources (Method#2) is still open for discussion. Note that the configuration of this scenario is not ruled out by RAN1.

In our view, the L1-SRS-RSRP measurement should be prioritized over L1-CLI-RSSI measurement if they are partly or fully overlapping. The reason is that we expect L1-SRS-RSRP to provide more knowledge about the CLI conditions of the victim UE than the L1-CLI-RSSI measurement. Moreover, the configuration of a L1-SRS-RSRP requires the transmission of SRS from an aggressor UE, which might end up wasted if the L1-SRS-RSRP is not measured. We prefer not to support Option 3 since it creates ambiguity in the NW about what the content of the received measurement report. Therefore, we prefer to support Option 1.

When measurement resources for L1-SRS-RSRP (Method#2) and L1-CLI-RSSI (Method#1) are partly or fully overlapping in time, the UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI and shall prioritize L1-SRS-RSRP measurement.

3. L1-CLI-RSSI measurement

  - Measurement restrictions

#### Issue 1-3-3: Measurement restriction RAN4#114bis Agreements (adhoc session on Tuesday) - Collision between L1-CLI-RSSI measurements with different methods - Capture in RAN4 specs that UE is not expected to perform CLI measurement using Method #1 and Methods#3 in the same OFDM symbol. - Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1. RAN4#115 meeting: - Proposals for collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1 - Proposal 1a (CATT, ZTE) - Measurement requirements for L1-CLI-RSSI apply as default. - Proposal 1b (Samsung) - Measurement requirements for L1-CLI-RSSI apply, when SCS of L1-CLI-RSSI measurement and L1/L3 RRM measurements are same. - Proposal 1c (E////) - Measurement requirements for L1-CLI-RSSI apply, except when L1-CLI-RSSI measurement resources overlap MG - Proposal 1d (HW) - Measurement requirements for L1-CLI-RSSI apply, except when - L1-CLI-RSSI measurement resources overlap MG - L1-CLI-RSSI measurement resources are configured with a different SCS from the SSB used for L1/L3 measurement - Proposal 2 (Nokia) - Measurement requirements for L1-CLI-RSSI do not apply. - Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements in FR1 - Discuss the following 2 options on high level: - Option 1: Measurement requirements for L1-CLI-RSSI apply as default, and define exception cases where requirements do not apply. - Option 2: Measurement requirements for L1-CLI-RSSI do not apply.

There is a pending FFS on whether we should define measurement restrictions for Method#1 and Method#3 L1-CLI-RSSI measurements when colliding with L1/L3 RRM measurements for FR1. In our view, measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L3 RRM measurements party or fully overlap in time. But for L1-CLI-RSSI and L1 RRM measurement, we believe UE is able to measure both if the measurement resources have the same SCS. Therefore, the measurement requirements for L1-CLI-RSSI shall apply when CLI resources and resources for L1 RRM measurements and configured with the same SCS and partly or fully overlap in time.

Measurement requirements for L1-CLI-RSSI do not apply when CLI resources and resources for L3 RRM measurements partly or fully overlap in time for FR1.

Measurement requirements for L1-CLI-RSSI shall apply when CLI resources and resources for L1 RRM measurements are configured with same SCS and partly or fully overlap in time for FR1.

4. General

  - Requirement applicability

#### Issue 1-1-1: Requirement applicability - Agreement (Monday online session) - For method #2, 3 and non-basic UE operation of method #1 - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is configured within active DL BWP - when CLI-RSSI measurement resource is configured within active DL BWP - FFS for basic UE operation for method #1 - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

From RAN1 point of view, it is still open for decision about whether the basic capability for SBFD UEs would support the DL reception in one or two DL subbands [R1-2504673]. In our view, we can agree on an alternative text for the current requirement applicability and wait for RAN1 decision to confirm the assumptions.

Thus, if the basic SBFD UE feature supports DL reception over a single DL subband, the current agreement should include such condition:

  - For method #2, 3 and non-basic UE operation of method #1

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is configured within active DL BWP

      - when CLI-RSSI measurement resource is configured within active DL BWP

  - For basic UE operation of method #1:

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when CLI-RSSI measurement resource is configured within one DL subband within active DL BWP

Otherwise, if the basic SBFD UE feature supports simultaneous DL reception over two DL subbands, the current requirements applicability could be updated as follows:

  - For method #2, 3 and method #1

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is configured within active DL BWP

      - when CLI-RSSI measurement resource is configured within active DL BWP

RAN4 to discuss the potential changes on the requirements applicability depending on the basic SBFD UE operation while waiting for RAN1 decision.

  - L1 UE-to-UE CLI measurements in non-SBFD symbols

There have been discussions in RAN1 about whether a UE can be configured to measure L1 UE-to-UE CLI in non-SBFD symbols, as captured in the following text.

#### Proposal 2-5 Alt.1 The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting. - UE shall perform L1 UE-to-UE CLI measurement only in SBFD symbols Alt.2 The valid symbol type for CSI derivation configured in CSI-ReportConfig is not applicable for L1 UE-to-UE CLI measurement and reporting. - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig Note: L1 CLI measurement is supported in both SBFD symbols and non-SBFD symbols

RAN1 agreed in RAN1#121 to support the configuration of valid symbol type for L1 UE-to-UE CLI measurements. In particular, the RAN1 agreement defines the following:

Agreement (RAN1#121) For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

In other words, a UE configured with SBFD symbols shall perform periodic or semi-persistent L1-SRS-RSRP or L1-CLI-RSSI measurements in the configured valid symbol type as indicated in the CSI-ReportConfig IE. This is in line with Alt.2 captured in the last RAN1 meeting way-forward. Accordingly, the L1 CLI measurement requirements shall be defined assuming UE only measures the L1 CLI measurement resources based on the valid symbol type configured in CSI-ReportConfig.

- Observation 1: RAN1 agreed UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig.

RAN4 to assume the valid symbol type is configured in CSI-ReportConfig for L1 UE-to-UE CLI measurements.

RAN4 to define the L1 CLI measurement requirements assuming UE only measures the L1 CLI measurement resources based on the valid symbol type configured in CSI-ReportConfig.

5. Conclusion

The following Observations and Proposals were made:

- When measurement resources for L1-SRS-RSRP (Method#2) and L1-CLI-RSSI (Method#1) are partly or fully overlapping in time, the UE is not required to measure both L1-SRS-RSRP and L1-CLI-RSSI and shall prioritize L1-SRS-RSRP measurement.

Measurement requirements for L1-CLI-RSSI do not apply when CLI resources and resources for L3 RRM measurements partly or fully overlap in time for FR1.

Measurement requirements for L1-CLI-RSSI shall apply when CLI resources and resources for L1 RRM measurements are configured with same SCS and partly or fully overlap in time for FR1.

RAN4 to discuss the potential changes on the requirements applicability depending on the basic SBFD UE operation while waiting for RAN1 decision.

- Observation 1: RAN1 agreed UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig.

RAN4 to assume the valid symbol type is configured in CSI-ReportConfig for L1 UE-to-UE CLI measurements.

RAN4 to define the L1 CLI measurement requirements assuming UE only measures the L1 CLI measurement resources based on the valid symbol type configured in CSI-ReportConfig.

References

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei